3GPP TSG-RAN WG4 Meeting #101-bis-e                                                    R4-2200130

Electronic Meeting, January 17 - 25, 2022

Source: 	CATT

Title: 	Simulation results for UL timing adjustment demodulation requirements for FR2 HST

Agenda item:	6.9.5.3.1

Document for:	Approval

- Introduction

In RAN4#101-e, a way forward on FR2 HST demodulation was approved [1].

In this contribution, we provide our simulation results for UL timing adjustment demodulation following latest agreements.

- Simulation results

As per the WF [1], the simulation assumptions for UL timing adjustment requirement are shown in Table A-1.

The simulation results for UL timing adjustment requirement for FR2 HST are shown in Table 2-1.

Table 2-1, Simulation results for UL timing adjustment requirement for FR2 HST

| PUSCH |  |  |  |  |  | SNR@70% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCS and BW | MCS | Channel | DMRS mapping | Length of data | RS | Ideal | Impairment |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+0 | 6.4 | 8.9 |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+1 | 5.89 | 8.39 |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+1+1 | 6.05 | 8.55 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+0 | 6.36 | 8.86 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+1 | 5.98 | 8.48 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+1+1 | 5.89 | 8.39 |

- Conclusion

This contribution provides our simulation results for UL timing adjustment demodulation requirements for FR2 HST for alignment.

- References

- R4-2120703, WF on BS demodulation requirement for FR2 HST, Nokia, Nokia Shanghai Bell, Samsung, RAN4#101-e

- Annex

Table A-1, Simulation assumption for UL timing adjustment requirement for FR2 HST

| Parameters | Value |
| --- | --- |
| Channel Model | Stationary UE: AWGN, Moving UE: AWGN |
| UE speed | 350km/h |
| Waveform | CP-OFDM |
| CP length | Normal |
| A | AWGN |
| Δω | 1T2R |
| CBW | 50MHz and 200MHz |
| DM-RS type | Type 1 |
| Mapping type | B |
| Length of PUSCH data | 10 |
| RS configuration | 1DMRS+PT-RS(L=1,K=2), 2DMRS+PT-RS(L=1,K=2), 3DMRS+PT-RS(L=1,K=2) |
| Number of DM-RS CDM group(s) without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| DM-RS port | {0} |
| DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| PUSCH resource allocation | 50MHz: 0 to 15 RB for moving UE, 16 to 31 for stationary UE (if 100MHz 200MHz: 0 to 65RB for moving UE, 66 to 131 for stationary UE |
| SRS resource allocation | 50MHz CBW (32RBs)~C_SRS =9, B_SRS =0 200MHz CBW (132RBs)~ C_SRS=33, B_SRS=0 |