3GPP TSG-RAN WG4 Meeting # 101-bis-e 		                       R4-2200166

Electronic Meeting, January 17th - 25th, 2022

Title: 	ACLR/ACS proposal for satellite access node

Source: 	CATT

Agenda item:	6.13.2.3

Document for:	Discussion

- Introduction

2 WFs on satellite access node RF requirements were approved in [1, 2]. ACLR and ACS requirements are still open pending on co-existence simulations.

This paper will further discuss ACLR/ACS requirement for satellite access node.

- Discussion

The following outcomes were observed from companies’ simulations.

Table 2-1 ACIR requirement

| NTN station | Worst option (options with >1 contributions) | ACIR ranges | Note |
| --- | --- | --- | --- |
| GEO | *NTN type (GEO) *TN BS type (AAS) *Deployment scenario (Rural) | 8~16 | To be used for satellite access node ACLR derivation |
| LEO 1200 | *NTN type (LEO1200) *TN BS type (AAS) *Deployment scenario (Rural) | 18~26 |  |
| LEO 600 | *NTN type (LEO600) *TN BS type (AAS) *Deployment scenario (Rural) | 18~26 |  |
| GEO | *NTN type (GEO) *TN BS type (AAS) *Deployment scenario (Rural) | 8~22 | To be used for satellite access node ACLR derivation |
| LEO 1200 | *NTN type (LEO1200) *TN BS type (AAS) *Deployment scenario (Urban) | 18~30 |  |
| LEO 600 | *NTN type (LEO600) *TN BS type (AAS) *Deployment scenario (Urban) | 24~30 |  |

Usually the BS ACLR/ACS is determined ~10dB larger than the requirement ACIR for DL and UL co-existence scenarios. We propose to define satellite node ACLR requirement as 38dB and ACS requirement as 40dB.

Proposal 1: it is proposed to define satellite access node ACLR requirement as 38dB.

Proposal 2: it is proposed to define satellite access node ACS requirement as 40dB.

- Conclusion

This paper further discussed satellite access node ACLR/ACS requirement. The following proposals are concluded.

Proposal 1: it is proposed to define satellite access node ACLR requirement as 38dB.

Proposal 2: it is proposed to define satellite access node ACS requirement as 40dB.

- Reference

[1]	R4-2120774, Way Forward on NTN_solutions_Part1, Thales

[2] 	R4-2117378, Further discussion on NTN gNB classes, CATT

[3]	R4-2120675, NTN - WF on Rx RF requirements for satellite access node, Ericsson

[4]	R4-2120671, Simulation assumptions for NTN co-existence study, Samsung, CATT