3GPP TSG RAN WG4 Meeting #101-bis-e	R4-2200210

Online, 17th – 25th Jan. 2022

Agenda item:	5.11.1

Source:	SoftBank Corp.

Title:	TP for TR 38.717-03-02: CA_n41-n77-n257

Document for:	Approval

1	Introduction

NR-CA of 3 Band DL and 2Band UL, CA_n41-n77-n257, was approved in RAN#93 [1]. This TP is to capture the basic aspects for the NR-CA.

2	Text Proposal

[Unchanged Parts Skipped]

5.3.x	CA_n41-n77-n257

5.3.x.1	Operating bands for CA

Table 5.3.x.1-1: CA band combination of band n41+n77+n257

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

5.3.x.2	Channel bandwidths per operating band for CA

Table 5.3.x.2-1: Supported bandwidths per CA band combination of band n41+n77+n257

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 |  |  |
| CA_n41A-n77A-n257A | CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |  |
| CA_n41A-n77A-n257G | CA_n41A-n77A CA_n41A-n257A CA_n41A-n257G CA_n77A-n257A CA_n77A-n257G | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n77A-n257H | CA_n41A-n77A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n77A-n257I | CA_n41A-n77A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n41A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refer to Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

5.3.x.3	UE co-existence studies

Co-existence studies can be omitted because harmonic and intermodulation impact between FR1 bands have been already studied for CA_n41-n77, and harmonic and intermodulation impact between FR1 bands and FR2 band are negligible.

5.3.x.4	REFSENS requirements

There is no additional REFSENS requirement for this combination.

[Unchanged Parts Skipped]

3	Reference

[1] RP-212512	Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL, ZTE