3GPP TSG-RAN WG4 Meeting #101-e-bis	R4-2200241

# Electronic Meeting, 17 – 25 January, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Pre-MG activation/deactivation delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Core |  |  |  |  |  | Date: |  |  | 2022-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 agreed to introduce the additional transition time for Pre-MG Activation/Deactivation Delay (R4-2120301): Requirements for; Detail; New or impacted section. Activation/deactivation delay; Additional validation time after BWP switch to change pre-MG status; (new) 8.14. |
| Summary of change: |  | Introduce the additional transition time for Pre-MG Activation/Deactivation Delay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Activation/deactivation delay requirement for Pre-MG would still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 8.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

## 8.14	Pre-configured measurement gap activation/deactivation delay

### 8.14.1	Introduction

The requirements in this clause apply for a UE configured with PCell [or any activated SCell] in standalone NR.

UE shall complete the activation/deactivation of pre-configured measurement gap within the delay defined in this clause.

### 8.14.2	Pre-configured measurement gap activation/deactivation upon BWP switch

#### 8.14.2.1	Activation/deactivation upon BWP switch delay on a single CC

The requirements in this clause only apply to the case that the BWP switch is performed on a single CC with more than one BWP configurations configured.

When BWP switch occurs, which results in status change of pre-configured measurement gap according to [9.1.2A], UE shall be able to finish pre-configured activation or deactivation within [x] ms after BWP switch.

#### [8.14.2.2	Activation/deactivation upon BWP switch delay on multiple CC]

### [8.14.3	Pre-configured measurement gap activation/deactivation upon measurement object(s) update]

### [8.14.4	Pre-configured measurement gap activation/deactivation upon SCell(s) update]

### [8.14.4	Pre-configured measurement gap activation/deactivation upon LPP positioning request]

End of Change