3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200288

# Electronic Meeting, January 17-25, 2022

Agenda item:	6.10.2.1

Source:	Apple

Title:	Discussion on SRS antenna port switching

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM requirement for SRS antenna port switching has been discussed and the conclusions were captured in the WF[1]. However, there are couple of open issues from last meeting, and in this contribution, we continue discussing the RRM requirement for SRS antenna port switching.

# Scope of SRS antenna switching requirement

In last meeting, some agreements on scope of SRS antenna switching requirement were (open issues are highlighted in yellow),

Issue 1-1-1: whether scheduling restriction requirement would be defined in RRM for SRS antenna port switching Issue1-1-1a: Clarification on performance degradation due to SRS antenna port switching: Agreement in 2nd round: The performance degradation can be expected on 1 OFDM symbol before and after each SRS resource configured for antenna switching which is not overlapped with the guard period defined in TS 38.214 on the carrier where SRS antenna switching occurs. Issue1-1-1b: Whether to capture conclusion of issue 1-1-1a in RRM spec or in WF only: - Option 1 (Apple, QC, CMCC, vivo, Xiaomi, Intel, ZTE, CATT): in WF - Option 2 (HW, LGE, MTK, OPPO, Nokia, vivo): in RRM spec Agreement in 2nd round: TBA Issue 1-1-2: RAN4 requirement scope with different SRS resource configuration - Option 1 (CATT, Apple, Xiaomi, LGE, OPPO, HW, Intel, QC, MTK, ZTE,): - RAN4 to define a generic requirement regardless of SRS resource configuration. - Option 2 (Nokia): - RAN4 shall define the requirements for the following scenarios in Rel17 where - The SRS resources of a set are transmitted in the same slot with consecutive SRS transmission, or - The SRS resources of a set are transmitted in separate slots. - RAN4 do not define the requirements if the SRS resources of a set are transmitted in the same slot with non-consecutive SRS transmission, before the guard period in this scenario gets clarified in RAN1. - Option 3 (Ericsson): - define requirements under the assumption that SRS symbols are configured on consecutive symbols (with 1 symbol guard period between SRS symbols). - Option 4(vivo): - In R17 feRRM WI, the number of consecutive symbols for SRS transmission configured in a slot comprising UL symbols is no more than X, and X = 2 is preferred. Agreement in 2nd round: TBA

In TS38.101-1, it was defined that the transient period of 15 us applies before and after SRS transmission to all the transmit CCs in CA with the CC sounding SRS, and UE RF requirements do not apply during this transient period. In the last meeting, it was agreed that “The performance degradation can be expected on 1 OFDM symbol before and after each SRS resource configured for antenna switching which is not overlapped with the guard period defined in TS 38.214 on the carrier where SRS antenna switching occurs”. Since such degradation could be represented by RF clarification indirectly (‘no RF requirement applies’ means corresponding baseband UE behavior could not be guaranteed accordingly), we don’t see strong rationale to clarify such performance degradation in RRM spec either.

Proposal 1: No need to specify performance degradation in issue 1-1-1.

Since in WF[1] of last meeting RAN4 agreed to use the two scenarios for the minimum requirement design of SRS antenna port switching, the requirement could be specified/applied based on the SRS resource configurations in scenario 1 and scenario 2 of issue 1-4-2 in WF[1].

Proposal 2: RAN4 to define requirements based on SRS resource configurations in scenario 1 and 2 in agreed WF of RAN4 #101e meeting.

# Impact of SRS antenna port switching to other requirements

In last meeting, some agreements on impact of SRS antenna port switching to other requirements were (open issues are highlighted in yellow),

Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA - Option 1 (CATT, Nokia): UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with L1-RSRP/L1-SINR measurement, and the L1-RSRP/L1-SINR measurement will be interrupted when overlapping with aperiodic SRS transmission. - Option 2 (Apple, Nokia): UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, NR SRS antenna port switching shall be prioritized. - Option 3(QC): - Network should avoid scheduling conflict aperiodic SRS antenna switching and L1-RSRP measurement. When the collisions happen, it’s up to UE implementation for collision resolution. - Network should avoid scheduling conflict periodic SRS antenna switching and L1-RSRP measurement. If the network side solution is not feasible, the following requirement apply. UE can drop periodic SRS antenna switching when it conflicts with L1-RSRP measurement. L1-RSRP measurement requirement still applies. - Option 4 (Intel, HW, OPPO): Impact of SRS antenna port switching to RRM requirements in NR-SA will be: - The SRS antenna port switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26]. - The SRS antenna port switching is not colliding with any SSB/CSI-RS based L3 measurements and the measurements for RLM/BFD, L1-RSRP/L1-SINR. - Option 5 (MTK, Nokia): When SRS resource collides with L1-RSRP/L1-SINR RS, whether to transmit SRS resource is determined by the same rule between the SRS resource and the associated L1-RSRP/L1-SINR reporting type defined in section 6.2.1.3 of TS 38.214. - For NR-SA, UE is not required to perform SRS antenna port switching when periodic/semi-persistent SRS resource and the L1-RSRP/L1-SINR measurement RS for aperiodic report are scheduled in the same OFDM symbol. Otherwise, UE is required to perform SRS antenna port switching. - Option 6 (vivo): - For aperiodic SRS transmission, clarify in the spec by adding a note for L1-RSRP/L1-SINR measurement period requirements, - ‘Note: Longer measurement period is expected if semi-persistent/periodic L1-RSRP or L1-SINR report is scheduled in the same symbol with aperiodic SRS in the same carrier’ - UE is not required to perform SRS antenna port switching when SRS resource and the DL RS for NR L1-RSRP/L1-SINR measurements are scheduled in the same OFDM symbol in CA/DC. - Option 7 (Nokia): - Do not define the priority in RAN4 when SRS resource and L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol. - Option 8 (HW, Apple, MTK, Intel, vivo, OPPO): NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement. - Option 8a (Apple, QC): NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement. - No requirement applies for AP L1-RSRP/L1-SINR measurement colliding with AP SRS. - Option 8b (Nokia): NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement except: - No requirement applies when AP NR SRS resource and the P/SP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol Agreement in 2nd round: TBA Issue 1-2-2: Impact of SRS antenna port switching to RRM requirements in EN-DC or NE-DC Agreements in 1st round: Follow the conclusion from issue 1-2-1 Issue 1-2-3: Impact of SRS antenna port switching to RRM requirements in NR-DC Agreements in 1st round: Follow the conclusion from issue 1-2-1 Issue 1-2-4: Impact of SRS antenna port switching to other specific RRM requirements - Option 1 (CATT): It should not be needed adding clarification/note of the handover/reselection/SCell activation requirements are defined when there are not SRS antenna port switching happens during the process of handover/ reselection/ SCell activation. - Option 2 (Apple, ZTE): In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities. - Option 3 (HW, Nokia): No specification impact of impact of SRS antenna port switching to other specific RRM requirements except for NR measurement. - Option 4 (Apple, QC, vivo, LGE, Xiaomi, ZTE, OPPO, MTK): In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching occurs during those RRM activities. - Option 4a (Apple, QC, LGE, ZTE, Intel, vivo, Xiaomi, OPPO): In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching is configured during those RRM activities. Agreement in 2nd round: TBA Issue 1-2-5: Impact of SRS antenna port switching to CSF and other RS - Option 1 (CATT, HW, Nokia, Ericsson): RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. - Option 2 (Apple, Intel, Nokia, CATT): RAN4 to not define any solution and requirement for “SRS antenna port switching to avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report”. - Option 3 (QC, MTK, OPPO, Intel): Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed. - Option 3a (MTK, Intel, OPPO): Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed - Option 4 (Ericsson, Nokia): Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels is to be handled by RAN1. If anything is unclear, RAN4 shall send LS to RAN1 and ask for clarification. Agreement in 2nd round: TBA

In last meeting, there was no conclusion on prioritization when SRS resource and L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol. In table 5.2.1.4-1 in TS38.214, RAN1 specified the relation between CSI reporting and DL CSI-RS as following table,

| CSI-RS Configuration | Periodic CSI Reporting | Semi-Persistent CSI Reporting | Aperiodic CSI Reporting |
| --- | --- | --- | --- |
| Periodic CSI-RS | No dynamic triggering/activation | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Semi-Persistent CSI-RS | Not Supported | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Aperiodic CSI-RS | Not Supported | Not Supported | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |

And in TS38.214 section 6.2.1.3, the prioritization for CSI reporting and SRS carrier switching is summarized as below,

| PUCCH/PUSCH for CSI (L1-RSRP/L1-SINR) reporting | SRS transmission (including interruption) | Prioritization |
| --- | --- | --- |
| P/SP CSI | AP SRS | AP SRS |
| AP CSI | AP SRS | AP SRS |
| AP CSI | P/SP SRS | AP CSI |
| P/SP CSI | P/SP SRS | P/SP SRS |

Since this prioritization is defined between two colliding UE activities, we don’t need to consider CSI reporting type for the scenario when CSI measurement is colliding with SRS antenna port switching. Regarding the CSI reporting vs. SRS case, RAN1 had clear definition in TS38.214 and there is no need to revisit those conclusions.

We think the CSI reporting priority could be reused for CSI measurement priority, and SRS carrier switching prioritization rule could also be reused for SRS antenna port switching prioritization. Thus, UE is not required to perform SRS antenna port switching when P/SP SRS resource and the AP L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, SRS antenna port switching shall be prioritized, as summarized in the following table.

Table 1. Prioritization between L1-RSRP/L1-SINR measurement and SRS antenna port switching

| CSI-RS and SSB for L1-RSRP/L1-SINR | SRS transmission (including interruption) | Prioritization |
| --- | --- | --- |
| P/SP CSI-RS and SSB | AP SRS | AP SRS |
| AP CSI-RS | AP SRS | AP SRS |
| AP CSI-RS | P/SP SRS | AP CSI |
| P/SP CSI-RS and SSB | P/SP SRS | P/SP SRS |

However, in last meeting, option 8 was proposed by other companies as a compromise to converge the views from companies and simplify the UE requirements, and we can also accept option 8 as a compromise.

Proposal 3:

RAN4 to choose one from following options:

- Option 1:

- For NR SA, EN-DC and NE-DC, UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, NR SRS antenna port switching shall be prioritized.

- For NR-DC, UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol of same CG; otherwise, NR SRS antenna port switching shall be prioritized.

- Option 2:

- NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement.

Regarding the impact of SRS antenna port switching to other RRM requirement (e.g., handover, BWP switching, SCell activation/deactivation), we think the same methodology in LTE spec could be re-used. In LTE TS36.133, RAN4 has added some clarification in the impacted RRM requirements, e.g.,

So, we could clarify the relaxation/applicability in those RRM requirements whose procedure could be impacted by SRS antenna port switching, e.g., the delay requirement could be extended, or the requirement only applies when SRS antenna port switching is not colliding with the reference signal. How to coordinate between SRS antenna port switching and those UE activities could be left to UE implementation, and therefore RAN4 only needs to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching is configured during those activities. To make the requirement applicability clear, we prefer to capture such applicability clarification into the corresponding requirement section in TS38.133.

Proposal 4: In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching is configured during those RRM activities.

Regarding impact of SRS antenna port switching to CSF and other RS, we think network shall control to avoid such collision for serving cell, but if such collision happens as error case, no UE requirement shall be imposed. However, it’s not necessary to define such error cases in the RAN4 spec since it shall be covered in RAN1, and we are also fine to compromise to the option 1 in agreed WF: RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement.

Proposal 5: Regarding impact of SRS antenna port switching to CSF and other RS, RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement.

# Interruption requirement applicability

In last meeting, some agreements on interruption requirement applicability were (open issues are highlighted in yellow),

Issue 1-3-1: Interruption requirement applicability Agreements in 1st round: The victim cells due to SRS switching shall be defined for UL interruption and DL interruption respectively, e.g. an UL interruption is allowed on any of the serving cells indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells indicated by txSwitchImpactToRx. Issue 1-3-2: Whether and how to specify interruption requirement for sync case - Option 1 (CATT, LG, Nokia, Ericsson): Yes, define different requirements between sync and async cases. - Option 1a (CATT): The number of interrupted slots for sync cases will be subtracted 1 based on the requirement for async cases. - Option 1b(LG): The same interruption length for the asynchronous case could be defined if the slot after SRS antenna port switching is the uplink slot in the synchronous case (UL+UL slot configuration). - If the slot after SRS antenna port switching is the downlink slot (UL+DL slot configuration) in the synchronous case, the interruption length can be defined by Table 2. - Option 2 (Apple, QC, MTK, HW, Xiaomi, Intel, ZTE, OPPO): No need to further discuss a separate interruption requirement for sync cases. Agreement in 2nd round: TBA Issue 1-3-3: txSwitchImpactToRx for intra-band case Agreements in 1st round: No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case.

Regarding issue 1-3-3, we need to consider the TA between DL and UL as well as the time difference between CCs in CA(MTTD/MRTD). Regarding the different CA scenarios, in R16 carrier based SRS switching, we have conclusion in R4-2012155 that,

- Interruption requirements for CA case 1, case 2 and case 3

  - The same interruption requirements as for async case in the spec shall apply

Where,

  - Case 1: CA is co-location deployed

  - Case 2: Single TAG CA, or carriers in the same TAG for multiple TAG CA

  - Case 3: uplink time difference does not exceed a threshold X

    - X = [5] us

So here, we think the same principle shall be applied since sync and async may have the same interruption.

Proposal 6: No need to further discuss the minimum interruption requirements for sync cases.

# Interruption requirement design for SRS antenna port switching

In last meeting, some agreements on interruption requirement design were (open issues are highlighted in yellow),

Issue 1-4-1: The interruption requirement is defined based on slot level or symbol level - Option 1 (CATT, Apple, QC, CMCC, Xiaomi, LG, Intel, MTK, vivo, ZTE, HW, OPPO): based on slot level - Option 1a (LG): If SRS antenna port switching is configured in the flexible slot in the synchronous case, the interruption should apply to only uplink symbols in the interrupted slot. - Option 2 (Ericsson, Nokia): based on symbol level Agreement in 2nd round: TBA Issue 1-4-2: The components within interruption time of SRS antenna port switching in FR1 Agreements in GTW - The components of interruption time of SRS antenna port switching in FR1 are - Antenna switching time before and after SRS transmission occasion (2*15us) - SRS transmission time of X symbols - Option 1 (Apple, HW, LGE, Intel, CATT, MTK, Xiaomi, OPPO): X = 6 SRS symbols disregards actual number of configured SRS symbols - Option 2 (Ericsson, Nokia): X is the number of configured SRS symbols excluding guard symbols - FFS if different X can be considered depending on SRS resource configuration within a slot Newly added options in 2nd round: - Option 3 (Apple, CMCC, vivo(slot level interruption for scenario 1) ): - Requirements would be defined for two scenarios: - Scenario 1: when X=1 SRS symbol is configured in a slot for SRS antenna port switching, to define symbol-based interruption requirement - Scenario 2: otherwise, using X=6 SRS symbols in a slot as assumption to define slot-based interruption requirement - Option 3a: - Requirements would be defined for two scenarios: - Scenario 1: when X=1 SRS symbol is configured in a slot for SRS antenna port switching, use the number of configured SRS symbols as SRS transmission time - Scenario 2: otherwise, using X=6 SRS symbols in a slot as assumption of SRS transmission time - Option 4: - Define interruption requirements in symbols for sync case - Define interruption requirements in symbols/slots for async case. - If interruption requirement defined in symbols for async case, interruption length could be to the extent of slot length - Other options are not precluded. Agreement in 2nd round: - The components of interruption time of SRS antenna port switching in FR1 are - Antenna switching time before and after SRS transmission occasion (2*15us) - SRS transmission time of X symbols - Requirements would be defined for two scenarios: - Scenario 1: when X=1 SRS symbol is configured in a slot for SRS antenna port switching, the configured number of SRS symbols is used as SRS transmission time - Scenario 2: otherwise, using X=6 SRS symbols in a slot as assumption of SRS transmission time Issue 1-4-3: Interruption requirement proposals for async case Up to conclusions from issues 1-4-1/1-4-2. Issue 1-4-4: Interruption requirement proposals for sync case Up to conclusions from issues 1-4-1/1-4-2/1-3-2

The guard period in RAN1 TS38.214 is defined as below,

In our understanding, RAN1 has chosen to use the most conservative way to define the guard period and no signals would be expected to transmit in guard period. We propose to use the RAN1 guard period as baseline to determine the interruption requirement. However, similar as interruption requirements for SRS carrier-based switching, even though the GP is at symbol level, but the interruption requirement shall still be defined in unit of slot for NR and in unit of subframe for LTE. In last meeting, some companies thought the interrupting requirement may be defined in unit of symbols since transient time for SRS antenna port switching is 15us from RF spec, but we have different understanding: with the MTTD/MRTD and TA, we cannot precisely determine which DL symbols or UL symbols on other CCs are interrupted by SRS antenna port switching, and therefore using slot level interruption requirement would be a reasonable solution.

Proposal 7: Interruption requirement of SRS antenna port switching shall be defined based on slot level.

In last meeting, it was agreed that two scenarios would be considered for SRS antenna port switching interruption requirement. Based on antenna switching time before and after SRS transmission occasion plus SRS transmission time of X symbols in scenario 1 and 2, the interruption requirement could be defined as:

Proposal 8:

Scenario 1: The interruption requirement of SRS antenna port switching for scenario 1 is 2 slots of victim carrier’s SCS, as following,

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Unit of interruption requirement is slot for NR and subframe for LTE.

Scenario 2: The interruption requirement of SRS antenna port switching for scenario 2 is summarized as:

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

Unit of interruption requirement is slot for NR and subframe for LTE.

# Miscellaneous issues

In last meeting, some agreements on other miscellaneous issues were (open issues are highlighted in yellow),

Issue 1-5-1: Interruption requirement related with prioritization rule in RAN1 - Option 1 (CATT, HW, MTK, ZTE, OPPO, Nokia, Apple): RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. - Option 2 (vivo) : Send LS to RAN1 to check the prioritization rule for SRS antenna switching, i.e. whether the prioritization rule defined for SRS transmission and guard periods also applies for the transient periods, especially for CA/DC case and the corresponding UL band is indicated in txSwitchWithAnotherBand. Agreement in 2nd round: - RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement. - FFS: Interruption of SRS antenna switching is applicable only when the interruption time is not colliding with any other transmission with higher priority defined in clause 6.2.1 of TS 38.214. Issue 1-5-2: Two SRS colliding on same symbol - Option 1 (Apple, QC, HW, vivo, LGE, ZTE, OPPO, CATT, Nokia): This is up to RAN1 discussion, and no need to discuss this case in RAN4. - Option 2 (MTK, Ericsson): For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior are scheduled on the same OFDM symbol: - For UE not supporting R17 feMIMO, whether to transmit the SRS is up to UE implementation. - For UE supporting R17 feMIMO, follow the priority rule defined in RAN1 in R17, if any. Agreement in 2nd round: TBA

Regarding issue 1-5-1, we think RAN4 only need to define the interruption requirement based on the RAN4 RRM scenarios and don’t need to revisit any scenarios which have already been specified in RAN1 spec. In our understanding, option 1 is quite reasonable from RRM perspective. We propose to only discuss the prioritization between SRS and RRM activities but not discuss any other collision cases already defined in RAN1.

Proposal 9: regarding “Interruption requirement related with prioritization rule in RAN1”, RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement.

Regarding issue 1-5-2, we agree with other companies’ view in last meeting that this is an issue covered by RAN1.

Proposal 10:

For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol:

- This is up to RAN1 discussion, and no need to discuss this case in RAN4.

# Conclusion

In this contribution we discuss the RRM requirements for SRS antenna port switching.

Proposal 1: No need to specify performance degradation in issue 1-1-1.

Proposal 2: RAN4 to define requirements based on SRS resource configurations in scenario 1 and 2 in agreed WF of RAN4 #101e meeting.

Proposal 3:

RAN4 to choose one from following options:

- Option 1:

- For NR SA, EN-DC and NE-DC, UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol; otherwise, NR SRS antenna port switching shall be prioritized.

- For NR-DC, UE is not required to perform NR SRS antenna port switching when P/SP NR SRS resource and the AP CSI-RS for NR L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol of same CG; otherwise, NR SRS antenna port switching shall be prioritized.

- Option 2:

- NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement.

Proposal 4: In corresponding requirement section of TS38.133, RAN4 to clarify that other specific RRM requirements in which NR measurements are involved only applies when no SRS antenna port switching is configured during those RRM activities.

Proposal 5: Regarding impact of SRS antenna port switching to CSF and other RS, RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement.

Proposal 6: No need to further discuss the minimum interruption requirements for sync cases.

Proposal 7: Interruption requirement of SRS antenna port switching shall be defined based on slot level.

Proposal 8:

Scenario 1: The interruption requirement of SRS antenna port switching for scenario 1 is 2 slots of victim carrier’s SCS, as following,

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Unit of interruption requirement is slot for NR and subframe for LTE.

Scenario 2: The interruption requirement of SRS antenna port switching for scenario 2 is summarized as:

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

Unit of interruption requirement is slot for NR and subframe for LTE.

Proposal 9: regarding “Interruption requirement related with prioritization rule in RAN1”, RAN4 doesn’t define the rules to avoid collisions except what has been defined in RAN1 and NR measurement.

Proposal 10:

For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol:

- This is up to RAN1 discussion, and no need to discuss this case in RAN4.

# References

[1] R4-2120295, WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching, Apple, RAN4 #101-e meeting