3GPP TSG-RAN4 Meeting # 101-bis-e	R4-2200296

# Electronic Meeting, January 17-25, 2022

Agenda item:	6.20.3.3

Source:	Apple

Title:	Discussion on RRM relaxation requirement for RedCap

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM relaxation for RedCap has been discussed and the conclusions were captured in the WF[1].

RAN2 also achieved some agreements of RRM relaxation in last #116e meeting, duplicated as below.

Agreements via email - from offline 111: - UE is not allowed to relax its RRM measurements if both stationarity criterion and R17 not-at-cell-edge criterion are configured but UE meets only the R17 not-at-cell-edge criterion. - UE reports to network when it no longer meets relaxation criteria. - No additional signaling is introduced for network to tell UE whether and which criteria for RRM relaxation is considered satisfied when leaving RRC_CONNECTED state. - No need for UE to send UE Assistance Information to request network configuring it with relaxation criteria. - UE does not report its history/state of RRM relaxation when transitioning from RRC Idle/Inactive to RRC Connected. - Relaxation criteria for UEs in RRC Connected are configured by only dedicated signaling. Agreements online: - RAN2 assume that the existing RRM measurement framework can be used as baseline for enabling and disabling RRM relaxations for UEs in RRC Connected. Other methods can be considered too based on relaxation methods agreed by RAN4. - RAN2 understands that no prohibit timer is needed, if legacy measurement reporting framework is reused by UE to report its relaxation status - The granularity of RRM measurement relaxations (i.e. whether it should be specified per beam, per cell or per frequency) should be handled by RAN4

In this contribution, we further discuss the RRM relaxation requirement for RedCap.

# RRM measurement relaxation for Redcap

## 2.1 General aspects for RRM measurement relaxation for Redcap

Based on the following agreements from RAN2,

Rule 1: In Rel-17 RRM relaxation for RRC_IDLE/INACTIVE, Rel-17 stationary criterion is mandatory, and Rel-17 not-at-cell-edge criterion is optional configuration

Rule 2: A separate Rel-17 not-at-cell-edge threshold is introduced, and the new threshold is only associated with Rel-17 stationary criterion.

Rule 3: If configured with a not-at-cell-edge criterion, the R17 stationary criterion can only be configured together with the R17 not-at-cell-edge criterion, not with the R16 one.

To further clarify Rule 3, our understanding is: If RedCap UE is configured with ONLY ONE not-at-cell-edge criterion, the R17 stationary criterion can only be configured together with the R17 not-at-cell-edge criterion, not with the R16 one. However, it allows to configure both R16 and R17 non-at-cell-edge criterion (2 non-at-cell-edge criterion) together with R17 stationary criterion.

Based on RAN2 conclusions, our understanding on the R16/R17 criteria is as in the following table,

Proposal 1: RAN4 to consider the following scenarios/criterion for Rel-17 RedCap RRM relaxation in IDLE/inactive mode:

| No | Rel-16 relaxation criterion | Rel-17 relaxation criterion | Applicability |
| --- | --- | --- | --- |
| 1 | Rel-16 low mobility | Not configured | Legacy operation |
| 2 | Rel-16 not-at-cell-edge | Not configured | Legacy operation |
| 3 | Rel-16 low mobility & Rel-16 not-at-cell edge | Not configured | Legacy operation |
| 4 | Not configured | Rel-17 stationary | Allowed |
| 5 | Not configured | Rel-17 stationary & Rel-17 not-at-cell-edge | Allowed |
| 6 | Rel-16 low mobility | Rel-17 stationary | Allowed |
| 7 | Rel-16 not-at-cell-edge | Rel-17 stationary | Not allowed(Rule 3) |
| 8 | Rel-16 low mobility & Rel-16 not-at-cell-edge | Rel-17 stationary | Not allowed(Rule 3) |
| 9 | Rel-16 low-mobility | Rel-17 stationary & Rel-17 not-at-cell-edge | Allowed |
| 10 | Rel-16 not-at-cell-edge | Rel-17 stationary & Rel-17 not-at-cell-edge | Allowed |
| 11 | Rel-16 low mobility & Rel-16 not-at-cell-edge | Rel-17 stationary & Rel-17 not-at-cell-edge | Allowed |

Regarding the scenario when multiple criteria of R16/R17 are satisfied, RAN4 had some discussion in last meeting,

Issue 2-1-2: Relaxation when multiple criteria of Rel-16 and Rel-17 are satisfied - Option 1: For the case when UE is configured with both Rel-16 and Rel-17 relaxation criteria and UE has fulfilled both criteria, UE is allowed to meet the requirements that are most relaxed out of Rel-16 and Rel-17 requirements. (Ericsson) Nokia (providing combination with Rel16 ceriter is agreed) - Option 2: left to UE implementation (Xiaomi Huawei) - Option 3: When both rel-16 and rel-17 RRM relaxation criteria are met, the UE shall perform rel-17 RRM relaxation method; A note shall be added to the rel-17 RRM relaxation to mention that when rel-17 RRM relaxation criterion is fulfilled then the rel-16 RRM relaxation shall be disabled. (MTK) - Option 4: it’s not possible to configure R16 criteria for redcap UE (Apple CMCC Nokia)

In IDLE/Inactive modes, since network may have R16 PS UEs and R17 RedCap UEs, and the R16/R17 criteria is probably broadcasted via the SI, so R17 RedCap UE who can also support R16 PS may fulfill both R16 and R17 criterion for relaxation, and in this case, we assume RedCap UE shall follow the Rel-17 RRM relaxation method.

Proposal 2: In IDLE/Inactive mode, when both rel-16 and rel-17 RRM relaxation criteria are met, the UE shall perform Rel-17 RRM relaxation method.

However, R16 power saving is not applied in connected mode, and therefore only Rel-17 RRM relaxation criteria needs to be considered in connected mode.

Proposal 3: In RRC connected mode, only Rel-17 RRM relaxation criteria shall be considered for Rel-17 RRM relaxation method at RedCap UE.

Regarding the requirements when UE moves between different RRC states, our understanding is it could be discussed after RAN4 complete the requirement for both IDLE/Inactive and Connected mode, since so far there are still many unclear points at least for RRM relaxation in RRC connected mode.

Issue 2-1-4: Requirements when UE moves between different R17 states - Option 1: RAN4 needs to ensure that measurement requirements related to RRM relaxations are appropriately defined for transition between RRC states (Nokia): - In case the UE moves from RRC Idle / Inactive state to RRC Connected state, it returns to normal measurement mode, to obtain fresh neighbour cell measurements, even if it has been staying in relaxed measurement mode. Thereafter, the evaluation of RRM relaxation criteria for RRC Connected state is started. - In case the UE moves from RRC Connected state to RRC Idle / Inactive state, it can stay within the same measurement mode, e.g. if it was in relaxed measurement mode in RRC Connected state, it will stay there in RRC Idle / Inactive state until the next evaluation of RRM relaxation criteria takes place. - Option 2: FFS (Apple MTK Xiaomi Huawei Ericsson)

Proposal 4: Requirements for transition between RRC states could be discussed after RAN4 complete the RRM relaxation requirements for both IDLE/Inactive and Connected modes.

## 2.2 RRM measurement relaxation for Redcap at Idle/Inactive state

Regarding IDLE/Inactive mode RRM relaxation, RAN2 agreed that network can configure R17 stationarity criterion/criteria together with a R17 not-at-cell-edge criterion, to trigger RRM relaxations in RRC Idle/Inactive for R17 UEs supporting the feature. We think the RRM requirement delay requirement relaxation in RAN4 could be similar as R16 power saving WI.

In R16 power saving WI, when one of the criterion is fulfilled, UE would have a scaling factor K1/K2 to relax the RRM requirement, and if both of the criterion is fulfilled, RAN4 has a relative long fix value for RRM requirement interval, duplicated as below.

In last RAN2 meeting, it was agreed that,

  - UE is not allowed to relax its RRM measurements if both stationarity criterion and R17 not-at-cell-edge criterion are configured but UE meets only the R17 not-at-cell-edge criterion.

Regarding the scaling factor value, we think FR1 extension when single criteria is met could be as much as FR2 measurement delay with beam sweeping, which means extended by a beam sweeping factor, like 8 as conservative approach. Thus, regarding relaxation when single Rel-17 criteria is satisfied, we propose that,

Proposal 5:

If single criteria (stationarity or not-at-cell-edge) is configured and fulfilled, or if both stationarity criterion and R17 not-at-cell-edge criterion are configured but UE meets only the R17 stationarity criterion:

use scaling factor K1_RedCap=8 to relax the RRM requirement.

Like R16 inter-frequency higher priority measurement relaxation, for inter-frequency measurement relaxation requirement of R17 RedCap, if only Rel-17 stationarity criterion is met and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, then the UE shall search for inter-frequency layers of higher priority at least every K2_RedCap*Thigher_priority_search. The K2_RedCap shall be designed to be equivalent to the extension when both R17 criterions are met. For the case when both R17 criterions are met, we use the similar logic as for proposal 1, that is, use 8 times extension compared to the legacy measurement delay. Thus, the measurement relaxation for this case could be 8 hours and therefore the K2_RedCap could be 8*60=480.

Proposal 6:

For inter-frequency measurement relaxation requirement of R17 RedCap, if only Rel-17 stationarity criterion is met and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, then the UE shall search for inter-frequency layers of higher priority at least every K2_RedCap*Thigher_priority_search, where K2_RedCap=480.

As discussed above, when both Rel-17 stationary and Rel-17 not-at-cell-edge criteria are satisfied, we could use a fixed long measurement period for requirement relaxation, and this fixed long measurement period could be 8 hours.

Proposal 7:

When both Rel-17 stationary and Rel-17 not-at-cell-edge criteria are satisfied, use a fixed long measurement period of 8 hours for requirement relaxation.

## 2.3 RRM measurement relaxation for Redcap at CONNECTED state

Regarding RRC connected mode RRM relaxation, RAN2 still has many open issues (reporting configuration, or relaxation mechanism based on network reconfiguration or not) to discuss and RAN4 shall wait more conclusions from RAN2 before going to the requirement design.

RAN2 also agreed that,

  - UE reports to network when it no longer meets relaxation criteria.

That means, when the criterion is met, UE shall report it to the network; and when the criterion turns to be “not met”, UE shall also report it to the network since network needs to know when UE shall quit the RRM relaxation mode.

Proposal 8: For RedCap RRM relaxation in RRC connected mode, RAN4 to defer requirement design until sufficient conclusions are made in RAN2.

Proposal 9: For RedCap RRM relaxation in RRC connected mode, based on measurement, when UE turns from ‘fulfil criteria’ to ‘not fulfil criteria’, this is also needed to report to network.

# Conclusion

In this contribution, we further discuss the RRM relaxation requirement for RedCap.

Proposal 1: RAN4 to consider the following scenarios/criterion for Rel-17 RedCap RRM relaxation in IDLE/inactive mode:

| No | Rel-16 relaxation criterion | Rel-17 relaxation criterion | Applicability |
| --- | --- | --- | --- |
| 1 | Rel-16 low mobility | Not configured | Legacy operation |
| 2 | Rel-16 not-at-cell-edge | Not configured | Legacy operation |
| 3 | Rel-16 low mobility & Rel-16 not-at-cell edge | Not configured | Legacy operation |
| 4 | Not configured | Rel-17 stationary | Allowed |
| 5 | Not configured | Rel-17 stationary & Rel-17 not-at-cell-edge | Allowed |
| 6 | Rel-16 low mobility | Rel-17 stationary | Allowed |
| 7 | Rel-16 not-at-cell-edge | Rel-17 stationary | Not allowed(Rule 3) |
| 8 | Rel-16 low mobility & Rel-16 not-at-cell-edge | Rel-17 stationary | Not allowed(Rule 3) |
| 9 | Rel-16 low-mobility | Rel-17 stationary & Rel-17 not-at-cell-edge | Allowed |
| 10 | Rel-16 not-at-cell-edge | Rel-17 stationary & Rel-17 not-at-cell-edge | Allowed |
| 11 | Rel-16 low mobility & Rel-16 not-at-cell-edge | Rel-17 stationary & Rel-17 not-at-cell-edge | Allowed |

Proposal 2: In IDLE/Inactive mode, when both rel-16 and rel-17 RRM relaxation criteria are met, the UE shall perform Rel-17 RRM relaxation method.

Proposal 3: In RRC connected mode, only Rel-17 RRM relaxation criteria shall be considered for Rel-17 RRM relaxation method at RedCap UE.

Proposal 4: Requirements for transition between RRC states could be discussed after RAN4 complete the RRM relaxation requirements for both IDLE/Inactive and Connected modes.

Proposal 5:

If single criteria (stationarity or not-at-cell-edge) is configured and fulfilled, or if both stationarity criterion and R17 not-at-cell-edge criterion are configured but UE meets only the R17 stationarity criterion:

use scaling factor K1_RedCap=8 to relax the RRM requirement.

Proposal 6:

For inter-frequency measurement relaxation requirement of R17 RedCap, if only Rel-17 stationarity criterion is met and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, then the UE shall search for inter-frequency layers of higher priority at least every K2_RedCap*Thigher_priority_search, where K2_RedCap=480.

Proposal 7:

When both Rel-17 stationary and Rel-17 not-at-cell-edge criteria are satisfied, use a fixed long measurement period of 8 hours for requirement relaxation

Proposal 8: For RedCap RRM relaxation in RRC connected mode, RAN4 to defer requirement design until sufficient conclusions are made in RAN2.

Proposal 9: For RedCap RRM relaxation in RRC connected mode, based on measurement, when UE turns from ‘fulfil criteria’ to ‘not fulfil criteria’, this is also needed to report to network.

# References

[1] R4-2120325, WF on eDRX and RRM measurement relaxations requirements for Redcap UE, vivo, RAN4 #101e