3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200331

# Electronic Meeting, January 17-25, 2022

Source: 	MediaTek Inc.

Title: 	Discussion on UE TX requirements for NTN

Agenda Item:	6.13.4.1

Document for:	Approval

Introduction

Regarding NR NTN Rel-17 WF [3] from RAN4#101e, there are still many RF requirements unfinished. In the contribution, the evaluation of UE TX requirements are provided.

Discussion

In the contribution, several evaluated RF requirements (e.g., TX 64QAM, ACLR, Spurious emission requirements) about NTN UE TX are considered.

TX 64QAM

In [1], it is observed and reasonable that LEO1200 CL loss is 6dB higher than LEO600 CL. On top of that, the distance between NTN UE and GEO satellite for estimating GEO path loss needs further discussion.

Observation 1: Regarding GEO 18000~36000 km, it seems there is additional 23.5~29.5dB path loss compared to LEO 1200km.

In RAN4#101e meeting, the WF [3] is agreed to have further discussion about UL 64QAM.

Transmit modulation quality Proposals Option 1: this requirement can be reused for NTN UE for QPSK and 16QAM. 64QAM and 256QAM should be excluded. Option 2: this requirement can be reused for NTN UE for QPSK, 16QAM and 64QAM. 256QAM should be excluded. Option 3: other, please specify. WF: Option 2 as starting point and further check whether UL 64QAM needs to be removed later.

NR EVM requirement is shown below for reference.

Regarding high free space path loss between UE and satellites for NTN system, ability to take 64QAM is low.

Proposal 1: Regarding high free space path loss between UE and satellites for NTN system, ability to take 64QAM is low. NTN UE UL 64QAM can be optional capability.

ACLR

Observation 2: In NTN Tdocs [4] and [5], there are discussion about ACIR which is relevant to ACLR. From implementation perspective, it is challenging to have higher NTN UE ACLR than NR 30dB ACLR.

Proposal 2: Based on above observation, regarding TX ACLR of UE band n255 and n256, the ACLR is [30dB].

Spurious emission

- General spurious emissions

From table 6.5.3.1-2 in TS 38.101-1

From ERC Recommendation 74-01

Recommends 3:

By checking mobile UE requirements from TS 38.101-1 and ERC Recommendation 74-01, the general spurious requirement is the same. NR UE’s general spurious requirements from TS 38.101-1 are reused.

Proposal 3: By checking mobile UE requirements from TS 38.101-1 and ERC Recommendation 74-01, the general spurious requirement is the same. NR UE’s general spurious requirements from TS 38.101-1 are reused for NTN.

- Spurious emissions for UE co-existence

In TS 38.101-1, Table 6.5.3.2-1 provides requirements for spurious emissions for UE co-existence.

Due to co-band between n24 and n255 and same frequency usage, n255 reuses n24 spurious emission requirements.

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n24, n99, n255 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |

Proposal 4: Bands n24 and n255 are co-band and same frequency range. To assume n255 reuses n24 spurious emission requirements for UE co-existence.

When single wide-band filter is used for bands n256(Band BW=30MHz) and n65(90MHz), we think that n65 spurious emission for UE co-existence can be at least baseline for n256.

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n65 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 43 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for channel bandwidths up to 20 MHz within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when the carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when the carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

Proposal 5: To consider using single filter for n256(Band BW=30MHz) and n65(90MHz), the n65 spurious emission requirements for UE co-existence is baseline for n256.

Conclusion

In the contribution, the evaluated NTN UE TX requirements are provided.

QAM

Observation 1: Regarding GEO 18000~36000 km, it seems there is additional 23.5~29.5dB path loss compared to LEO 1200km.

Proposal 1: Regarding high free space path loss between UE and satellites for NTN system, ability to take 64QAM is low. NTN UE UL 64QAM can be optional capability.

ACLR

Observation 2: In NTN Tdocs [4] and [5], there are discussion about ACIR which is relevant to ACLR. From implementation perspective, it is challenging to have higher NTN UE ACLR than NR 30dB ACLR.

Proposal 2: Based on above observation, regarding TX ACLR of UE band n255 and n256, the ACLR is [30dB].

Spurious emission

- General spurious emissions

Proposal 3: By checking mobile UE requirements from TS 38.101-1 and ERC Recommendation 74-01, the general spurious requirement is the same. NR UE’s general spurious requirements from TS 38.101-1 are reused for NTN.

- Spurious emission for UE co-existence

Proposal 4: Bands n24 and n255 are co-band and same frequency range. To assume n255 reuses n24 spurious emission requirements for UE co-existence.

Proposal 5: To consider using single filter for n256(Band BW=30MHz) and n65(90MHz), the n65 spurious emission requirements for UE co-existence is baseline for n256.

- References

[1] R4-2117378, “Further discussion on NTN gNB classes”, CATT

[2] R4-2119300, “MEO Scenarios and Characteristics”, Hughes/EchoStar, Thales

[3] R4-2120676, “WF on NTN UE RF requirement”, Huawei, HiSilicon

[4] R4-2201262, “NTN coexistence scenarios and simulations”, MediaTek Inc.

[5] R4-2117384, “Co-existence simulation results for NTN <--> TN scenarios”, CATT