3GPP TSG-RAN WG4 Meeting #101Bis-e	R4-2200342

Jan 2022

Title:	Two sets of SRS sets and GP needed in between

Source:	Qualcomm Incorporated

Agenda item:	6.19.2.3

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Responsible WG:	RAN1

Document for:	Approval

# 1.	Introduction

RAN1 has agreed changes to the SRS configurations under feMIMO WI that will have impact on RF aspects of a NR  UE. This paper discusses possible amendments and changes needed in RAN4.

# 2. 	Discussion

## 2.1	Guard period issues

38.214 – clause 6.2.1 UE sounding procedure

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying N S ∈ {1, 2, 4} adjacent OFDM symbols within the last 6 symbols of the slot, or at any symbol location within the slot if resourceMapping-r16 is provided subject to UE capability, where all antenna ports of the SRS resources are mapped to each symbol of the resource

The Rel-15 only enables SRS symbols to be transmitted in the last 6 symbols of the slot as described in TS 38.214 and described in TS38.211 section 6.4.1.4.1 and in TS 38.331 SRS-Resource – resource mapping – start position description.

Rel-17 enables SRS’s to be transmitted in any symbol of the slot [1] depending on UE capability in 38.306 srs-StartAnyOFDMSymbol. This capability was there in Rel-16 added by NR-U WI but corresponding RAN1 changes of guard period were not made.

Rel-15 only allows one SRS-ResourceSet for antenna switching with type aperiodic. Exception is for the case of 1T4R when the sequence of four SRS resources can not be transmitted in one slot due to gaps and restriction of 6 symbols per slot, then two SRS-ResourceSets can be configured for UE to enable sounding of four antennas. This can be found from 38.214 6.2.1.2.

Rel-17 enables extending the number of SRS resources sets for 1T2R, 2T4R and 1T4R based on UE capability. The extended configuration allows of two sets of SRS resources for 1T2R/2T4R and four SRS sets for 1T4R where one set contains only one resource and the SRS set usage is set for antenna switching [1] and corresponding capabilities will be defined.

The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set.

Table 6.2.1.2-1: The minimum guard period between two SRS resources of an SRS resource set for antenna switching

|  |  | Y [symbol] |
| --- | --- | --- |
| 0 | 15 | 1 |
| 1 | 30 | 1 |
| 2 | 60 | 1 |
| 3 | 120 | 2 |

RAN1 specification defines a guard period for SRS resources for ant switching as follows (from Rel-15 38.214 6.2.1.2):

To point out to the reader the obvious, the specification reads that

- The guard period is applicable in between the SRS resources in the same set.

- UE does not transmit any other signal in the guard period

The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot.

In Rel-17, RAN1 has agreed that when SRS are in different sets but in consecutive slots, there is also a guard period. However, whether the UE will transmit other symbols in the guard is left open. Here is the change in [1]:

The guard is the minimum between SRS but the language saying “guard period of Y symbols, in which the UE does not transmit any other signal” is left out.

## 2.2	UE requirements in the gap

These changes discussed in previous section enable SRS transmission scenarios as in Figure 1 (example shown for 30KHz SCS with 1 symbol guard period)

The figure 1 a) case is covered by the RAN1 CR [1] but the b) where there maybe other signals in the guard is left open.

The guard period may be larger than the minimum guard period one defined in the 38.214 Table 6.2.1.2-1. In Rel-17, with more than one SRS resource set configured and SRS placed freely on any symbol of the slot, the SRS may be scheduled with larger gaps than the minimum.

Figure 1: Possible SRS transmissions with rel-17 flexibility for SRS symbols in the slot and two sets for antenna switching.

Observation 1: If UE is configured with more than one SRS resource-set with usage set for antenna switching, with current agreements UE maybe required to transmit other signals in the guard period.

Proposal 1: Antenna switching time i.e. time between end of transmission in antenna port n and begging of transmission in antenna port n+1 is 15 usec.

The agreement in Rel-15 for the finite switching time between antenna ports [2] literary read as follows:

This implies that for any reason UE needs to switch the transmission from one antenna port to another, UE needs time to perform the switching when it is not capable of transmitting anything.

Proposal 1: In case UE is required to transmit any signals in to two different antenna ports in different symbols following each other, the switching time of 15 usec is allowed for the UE and UE does not transmit anything for the duration of the switching time.

With proposed agreement in proposal 1, it still leaves the open the case if there is longer time than the minimum guard in between transmissions between the two different antenna ports. Figure 2 depicts the case.

Figure 2. Other transmissions in between SRSs with longer than minimum guard period

Observation 2: It is not clear if UE is expected to be capable of other signal transmissions in the guard when SRS’s for antenna switching are from different sets.

This maybe feasible based on earlier agreements but it should be at least detailed somewhere that UE needs the guard period to switch the TX to the other port. RAN1 specification does not address this case currently and RAN4 should discuss if this kind of case should be added to the section 6.3 or does Figure 6.3.3.7-2 cover this case already.

Proposal 2: RAN4 shall discuss and clarify atleast the following issues:

- Is UE expected to transmit other signals between SRSs belonging to different set where usage is set to antenna switching

- Should there be a guard period defined in ran1 specs or a switching time defined in ran4 specs in between SRSs and other signals in the case 1 scenario

Proposal 3: For the issue 1, we propose that UE is not required to transmit other signals.

# Conclusion

We discuss new features from feMIMO WI and made the following observations and proposals

Observation 1: If UE is configured with more than one SRS resource-set with usage set for antenna switching, with current agreements UE maybe required to transmit other signals in the guard period.

Proposal 1: In case UE is required to transmit any signals in to two different antenna ports in different symbols following each other, the switching time of 15 usec is allowed for the UE and UE does not transmit anything for the duration of the switching time.

Observation 2: It is not clear if UE is expected to be capable of other signal transmissions in the guard when SRS’s for antenna switching are from different sets.

Proposal 2: RAN4 shall discuss and clarify atleast the following issues:

- Is UE expected to transmit other signals between SRSs belonging to different set where usage is set to antenna switching

- Should there be a guard period defined in ran1 specis or a switching time defined in ran4 specs in between SRSs and other signals in the case 1 scenario

Proposal 3: For the issue 1, we propose that UE is not required to transmit other signals.

# References

[1]	R1-2112949, “Introduction of further enhancements on MIMO for NR”, Nokia, 3GPP TSG-RAN WG1 Meeting #107-e, Electronic Meeting, November 11th – 19th, 2021

[2]	R4-1709833, “SRS Hopping”, Qualcomm Inc, 3GPP TSG-RAN WG4 Meeting NR ad-hoc #3, Nagoya, Japan, 18 - 21 Sep, 2017