3GPP TSG-WG RAN4 Meeting #101-bis-e	R4-2200350

# Online, 17 – 25 January, 2022

Source: 				Nokia, Nokia Shanghai Bell

Title: 					TP to TR 36.717-03-01: CA_2-7-38

Agenda Item: 		8.2.2 [LTE_CA_R17_3BDL_1BUL]

Document for:		Approval

# Introduction

This TP introduces the 2 band E-UTRA CA configurations CA_2A-7A-38A and CA_2A-7C-38A for 3DL/1UL.

TP to TR 36.717-03-01

## <Start of Changes>

## 5.x	CA_2-7-38

### 5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-7A-38A17 | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-7C-38A17 | - | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 17: UL carrier shall be supported in Band 2 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 5.x.2	∆TIB and ∆RIB values

Table 5.x.2-1: IB,c

| CA_2-7-38 | 2 | 0.5 |
| --- | --- | --- |
|  | 7 | 0.5 |
|  | 38 | 0.5 |

Table 5.x.2-2: R IB,c

| CA_2-7-38 | 2 | 0 |
| --- | --- | --- |
|  | 7 | 0 |
|  | 38 | 0 |

### 5.x.3 	REFSENS requirements

There is no MSD requirement needed for CA_2A-7A-38A and CA_2A-7C-38A.

## <End of Changes>