3GPP TSG RAN WG4 Meeting #101-bis-e	    R4-2200401

Electronic Meeting, January 17 – January 25, 2022

Source:	vivo

Title:	Simulation results for cell detection, PBCH detection, SSB measurement and L1 RSRP measurement

Agenda item:		6.20.3.1.5

Document for:	Information

- Introduction

In this contribution, we provide simulation results of cell detection, PBCH detection, SSB measurement and L1 RSRP measurement based on simulation assumptions in [1], [2], [3] and [4].

- Simlulation results

  - Simulation results of cell detection

Simulation results of cell detection delay for 1Rx and 2Rx are provided in Table 1.

Table 1: PSS/SSS detection delay

| SCS | Propagation Condition | 90 percentiles of the PSS/SSS acquisition time |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | -8dB |  | -6dB |  | -2dB |  |
|  |  | 1Rx | 2Rx | 1Rx | 2Rx | 1Rx | 2Rx |
| 15kHz | AWGN | 4 | 2 | 1 | 1 | 1 | 1 |
|  | TDL-A | 4 | 3 | 3 | 2 | 2 | 1 |
|  | TDL-B | 4 | 3 | 3 | 2 | 2 | 1 |
|  | TDL-C | 4 | 3 | 3 | 2 | 2 | 1 |
| 30kHz | AWGN | 4 | 2 | 1 | 1 | 1 | 1 |
|  | TDL-A | 4 | 3 | 3 | 2 | 2 | 1 |
|  | TDL-B | 4 | 3 | 3 | 2 | 2 | 1 |
|  | TDL-C | 4 | 3 | 3 | 2 | 2 | 1 |
| 120kHz | AWGN | 4 | 2 | 1 | 1 | 1 | 1 |
|  | TDL-A | 4 | 3 | 3 | 2 | 2 | 1 |
| 240kHz | AWGN | 4 | 2 | 1 | 1 | 1 | 1 |
|  | TDL-A | 4 | 3 | 3 | 2 | 2 | 1 |

Observation 1：One sample need to be increased when SINR=-8dB and 1Rx is used compared with the existing requirements for PSS/SSS detection.

Observation 2：One sample need to be increased when SINR=-6dB and 1Rx is used compared with the existing requirements for PSS/SSS detection.

Observation 3：One sample need to be increased when SINR=-2dB and 1Rx is used compared with the existing requirements for PSS/SSS detection.

  - Simulation results of PBCH detection

Simulation results of PBCH detection in different channel models and PBCH SNR for 1Rx and 2Rx are provided in Table 2 and Table 3 for FR1 and FR2 respectively.

Table 2: PBCH-DMRS time index reading for FR1

| Case |  | Number of acquisition attempts |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15kHz |  | 30kHz |  |
| Propagation Condition | SNR (dB) | 1Rx | 2Rx | 1Rx | 2Rx |
| AWGN | -8 | 1 | 1 | 1 | 1 |
|  | -7 | 1 | 1 | 1 | 1 |
|  | -6 | 1 | 1 | 1 | 1 |
|  | -5 | 1 | 1 | 1 | 1 |
|  | -4 | 1 | 1 | 1 | 1 |
|  | -3 | 1 | 1 | 1 | 1 |
| TDL-A | -8 | 3 | 2 | 3 | 2 |
|  | -7 | 3 | 2 | 3 | 2 |
|  | -6 | 2 | 2 | 2 | 2 |
|  | -5 | 2 | 2 | 2 | 1 |
|  | -4 | 2 | 1 | 2 | 1 |
|  | -3 | 2 | 1 | 2 | 1 |
| TDL-B | -8 | 3 | 2 | 3 | 2 |
|  | -7 | 3 | 2 | 2 | 2 |
|  | -6 | 2 | 1 | 2 | 1 |
|  | -5 | 2 | 1 | 2 | 1 |
|  | -4 | 2 | 1 | 2 | 1 |
|  | -3 | 2 | 1 | 2 | 1 |
| TDL-C | -8 | 3 | 2 | 3 | 2 |
|  | -7 | 3 | 2 | 2 | 2 |
|  | -6 | 2 | 2 | 2 | 2 |
|  | -5 | 2 | 1 | 2 | 1 |
|  | -4 | 2 | 1 | 2 | 1 |
|  | -3 | 2 | 1 | 2 | 1 |

Table 3: PBCH reading time for FR2

| Case |  | Number of acquisition attempts |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 120 kHz |  | 240 kHz |  |
| Propagation Condition | SNR (dB) | 1Rx | 2Rx | 1Rx | 2Rx |
| AWGN | -8 | 7 | 1 | 6 | 1 |
|  | -7 | 2 | 1 | 3 | 1 |
|  | -6 | 2 | 1 | 1 | 1 |
|  | -5 | 1 | 1 | 1 | 1 |
|  | -4 | 1 | 1 | 1 | 1 |
|  | -3 | 1 | 1 | 1 | 1 |
| TDL-A | -8 | 9 | 5 | 11 | 5 |
|  | -7 | 8 | 4 | 8 | 4 |
|  | -6 | 6 | 3 | 6 | 3 |
|  | -5 | 5 | 2 | 6 | 2 |
|  | -4 | 5 | 2 | 5 | 2 |
|  | -3 | 4 | 2 | 3 | 2 |

Observation 4: At least 2 samples without considering the margin are required for PBCH detection for FR1 and 6 samples for FR2 when 1Rx is used compared with 2 samples for FR1 and 3 samples for FR2 assumed for the legacy UE.

  - Simulation results of SSB measurement

Simulation results for SSB measurements in different channel models and samples are provided in Table 4 and Table 5 for FR1 and FR2 respectively.

Table 4：SSB measurement results for FR1

| Channel | Samples | Delta RSRP [dB] |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS =15kHz |  |  |  |  |  | SCS =30kHz |  |  |  |  |  |
|  |  | 1Rx |  |  | 2Rx |  |  | 1Rx |  |  | 2Rx |  |  |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 3 | -0.53 | 0.28 | 1.01 | -0.12 | 0.53 | 1.11 | -0.56 | 0.28 | 1.00 | -0.11 | 0.52 | 1.12 |
|  | 5 | -0.34 | 0.29 | 0.87 | -0.03 | 0.47 | 0.95 | -0.37 | 0.29 | 0.86 | -0.03 | 0.47 | 0.94 |
| TDL-A | 3 | -2.09 | -0.15 | 0.89 | -1.09 | -0.09 | 0.70 | -2.21 | -0.15 | 1.02 | -1.17 | -0.08 | 0.79 |
|  | 5 | -1.35 | -0.16 | 0.63 | -0.86 | -0.10 | 0.50 | -1.44 | -0.15 | 0.72 | -0.89 | -0.09 | 0.59 |
| TDL-B | 3 | -2.37 | -0.16 | 1.10 | -1.24 | -0.08 | 0.91 | -2.47 | -0.14 | 1.20 | -1.31 | -0.04 | 1.02 |
|  | 5 | -1.59 | -0.15 | 0.79 | -0.97 | -0.08 | 0.69 | -1.62 | -0.15 | 0.87 | -1.04 | -0.06 | 0.77 |
| TDL-C | 3 | -2.38 | -0.14 | 1.25 | -1.40 | -0.04 | 1.08 | -2.43 | -0.14 | -2.43 | -1.43 | -0.01 | 1.16 |
|  | 5 | -1.64 | -0.14 | 0.90 | -1.08 | -0.06 | 0.83 | -1.72 | -0.13 | -1.72 | -1.11 | -0.03 | 0.90 |

Table 5：SSB measurement results for FR2

| Channel | Samples | Delta RSRP [dB] |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS =120kHz |  |  |  |  |  | SCS =240kHz |  |  |  |  |  |
|  |  | 1Rx |  |  | 2Rx |  |  | 1Rx |  |  | 2Rx |  |  |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 3 | -0.58 | 0.28 | 0.99 | -0.10 | 0.52 | 1.13 | -0.56 | 0.29 | 0.99 | -0.12 | 0.52 | 1.13 |
|  | 5 | -0.37 | 0.28 | 0.85 | -0.02 | 0.47 | 0.96 | -0.35 | 0.29 | 0.85 | -0.02 | 0.47 | 0.95 |
| TDL-A | 3 | -2.53 | -0.16 | 1.27 | -1.42 | -0.03 | 1.13 | -2.56 | -0.14 | 1.21 | -1.37 | -0.01 | 1.06 |
|  | 5 | -1.74 | -0.16 | 0.94 | -1.12 | -0.05 | 0.87 | -1.70 | -0.14 | 0.89 | -1.06 | -0.05 | 0.80 |

Observation 5: The inter-frequency and intra-frequency RSRP accuracy for FR1 and FR2 needs to be relaxed about 1.5dB when 1Rx is used compared with the legacy UE when 3 samples are assumed.

  - Simulation results of L1-RSRP measurement

    - SSB based on L1-RSRP measurement

Simulation results for SSB based on L1-RSRP in different channel models and samples are provided in Table 6 and Table 7 for FR1 and FR2 respectively.

Table 7: SSB based on L1-RSRP for FR1

| Channel | Samples | Delta RSRP [dB] |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS =15kHz |  |  |  |  |  | SCS =30kHz |  |  |  |  |  |
|  |  | 1Rx |  |  | 2Rx |  |  | 1Rx |  |  | 2Rx |  |  |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -1.86 | -0.01 | 1.34 | -0.78 | 0.46 | 1.58 | -1.89 | -0.02 | 1.34 | -0.80 | 0.46 | 1.60 |
|  | 3 | -1.00 | 0.00 | 0.84 | -0.45 | 0.28 | 0.96 | -1.00 | -0.01 | 0.83 | -0.47 | 0.27 | 0.97 |
| TDL-A | 1 | -4.23 | 0.04 | 3.30 | -1.75 | 0.05 | 1.71 | -4.19 | -0.02 | 2.56 | -1.78 | 0.05 | 1.64 |
|  | 3 | -1.40 | 0.00 | 1.04 | -0.87 | 0.03 | 0.82 | -1.40 | -0.04 | 1.03 | -0.88 | 0.03 | 0.80 |
| TDL-B | 1 | -4.15 | -0.01 | 2.38 | -1.66 | 0.05 | 1.59 | -3.71 | -0.03 | 2.09 | -1.61 | 0.06 | 1.50 |
|  | 3 | -1.31 | 0.00 | 1.02 | -0.88 | 0.03 | 0.79 | -1.29 | -0.02 | 0.93 | -0.83 | 0.03 | 0.80 |
| TDL-C | 1 | -3.28 | -0.02 | 2.00 | -1.54 | 0.07 | 1.47 | -2.90 | -0.03 | 1.80 | -1.50 | 0.05 | 1.45 |
|  | 3 | -1.21 | -0.02 | 0.93 | -0.80 | 0.05 | 0.84 | -1.12 | -0.01 | 0.91 | -0.76 | 0.03 | 0.81 |

Table 8: SSB based on L1-RSRP for FR2

| Channel | Samples | Delta RSRP [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS =120kHz |  |  |  |  |  |
|  |  | 1Rx |  |  | 2Rx |  |  |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -1.83 | -0.02 | 1.37 | -0.81 | 0.47 | 1.60 |
|  | 3 | -0.95 | -0.01 | 0.83 | -0.44 | 0.28 | 0.97 |
| TDL-A | 1 | -3.27 | -0.02 | 1.91 | -1.56 | 0.03 | 1.43 |
|  | 3 | -1.25 | -0.01 | 0.94 | -0.80 | 0.03 | 0.80 |

Observation 6: The SSB based on L1-RSRP measurement accuracy for FR1 needs to be relaxed about 2.5dB when 1Rx is used compared with the legacy UE for one sample.

Observation 7: The SSB based on L1-RSRP measurement accuracy for FR2 needs to be relaxed about 2dB when 1Rx is used compared with the legacy UE for one sample.

    - CSI-RS based on L1-RSRP measurement

Simulation results for CSI-RS based on L1-RSRP in different channel models and samples are provided in Table 8 nd Table 9 or FR1 and FR2 respectively.

Table 8: CSI-RS based on L1-RSRP for FR1

| Channel | Samples | Delta RSRP [dB] |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS =15kHz |  |  |  |  |  | SCS =30kHz |  |  |  |  |  |
|  |  | 1Rx |  |  | 2Rx |  |  | 1Rx |  |  | 2Rx |  |  |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -1.77 | -0.04 | 1.28 | -0.75 | 0.44 | 1.51 | -1.78 | -0.04 | 1.29 | -0.75 | 0.44 | 1.51 |
|  | 3 | -0.97 | -0.02 | 0.77 | -0.40 | 0.27 | 0.92 | -0.93 | -0.02 | 0.77 | -0.41 | 0.26 | 0.92 |
| TDL-A | 1 | -3.64 | 0.00 | 2.22 | -1.55 | 0.04 | 1.44 | -3.01 | -0.01 | 1.84 | -1.45 | 0.06 | 1.39 |
|  | 3 | -1.21 | -0.03 | 0.94 | -0.79 | 0.02 | 0.78 | -1.13 | -0.01 | 0.90 | -0.75 | 0.02 | 0.78 |
| TDL-B | 1 | -2.56 | -0.02 | 1.63 | -1.37 | 0.05 | 1.33 | -2.40 | -0.03 | 1.58 | -1.30 | 0.06 | 1.30 |
|  | 3 | -0.99 | -0.01 | 0.84 | -0.78 | 0.03 | 0.76 | -1.07 | -0.01 | 0.84 | -0.75 | 0.03 | 0.77 |
| TDL-C | 1 | -2.29 | -0.02 | 1.56 | -1.30 | 0.06 | 1.32 | -2.24 | -0.03 | 1.58 | -1.31 | 0.07 | 1.34 |
|  | 3 | -0.98 | -0.01 | 0.87 | -0.76 | 0.03 | 0.74 | -0.97 | -0.01 | 0.85 | -0.72 | 0.07 | 0.77 |

Table 9: CSI-RS based on L1-RSRP for FR2

| Channel | Samples | Delta RSRP [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS =120kHz |  |  |  |  |  |
|  |  | 1Rx |  |  | 2Rx |  |  |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -1.75 | -0.02 | 1.28 | -0.74 | 0.45 | 1.51 |
|  | 3 | -0.92 | -0.01 | 0.82 | -0.41 | 0.26 | 0.90 |
| TDL-A | 1 | -2.59 | -0.02 | 1.65 | -1.37 | 0.07 | 1.37 |
|  | 3 | -1.09 | -0.01 | 0.84 | -0.75 | 0.05 | 0.76 |

Observation 8: The CSI-RS based on L1-RSRP measurement accuracy for FR1 needs to be relaxed about 2.5dB when 1Rx is used compared with the legacy UE for one sample.

Observation 9: The CSI-RS based on L1-RSRP measurement accuracy for FR2 needs to be relaxed about 2dB when 1Rx is used compared with the legacy UE for one sample.

- Summary

Observation 1：One sample need to be increased when SINR=-8dB and 1Rx is used compared with the existing requirements for PSS/SSS detection.

Observation 2：One sample need to be increased when SINR=-6dB and 1Rx is used compared with the existing requirements for PSS/SSS detection.

Observation 3：One sample need to be increased when SINR=-2dB and 1Rx is used compared with the existing requirements for PSS/SSS detection.

Observation 4: At least 2 samples without considering the margin are required for PBCH detection for FR1 and 6 samples for FR2 when 1Rx is used compared with 2 samples for FR1 and 3 samples for FR2 assumed for the legacy UE.

Observation 5: The inter-frequecny and intra-frequency RSRP accuracy for FR1 and FR2 needs to be relaxed about 1.5dB when 1Rx is used compared with the legacy UE when 3 samples are assumed.

Observation 6: The SSB based on L1-RSRP measurement accuracy for FR1 needs to be relaxed about 2.5dB when 1Rx is used compared with the legacy UE for one sample.

Observation 7: The SSB based on L1-RSRP measurement accuracy for FR2 needs to be relaxed about 2dB when 1Rx is used compared with the legacy UE for one sample.

Observation 8: The CSI-RS based on L1-RSRP measurement accuracy for FR1 needs to be relaxed about 2.5dB when 1Rx is used compared with the legacy UE for one sample.

Observation 9: The CSI-RS based on L1-RSRP measurement accuracy for FR2 needs to be relaxed about 2dB when 1Rx is used compared with the legacy UE for one sample.

- References

- R4-2115359, “Simulation assumptions for RedCap cell detection performance”, vivo

- R4-2115363, “Simulation assumptions for RedCap PBCH detection”, vivo

- R4-2115360, “Simulation assumptions for RedCap SSB based RRM measurement performance”, Huawei, HiSilicon

- R4-2115362, “Simulation assumptions for RedCap L1 RSRP measurement performance”, Ericsson