3GPP RAN WG4 Meeting #101bis-e		R4-2200445

Online, January 17th – 25th, 2022

Agenda item:	5.36.2.2

Source:	Apple Inc.

Title:	HPUE A-MPR Proposal for NS_48

Release:	Rel-17

Document for:	Approval

# 1	Introduction

During last RAN4 meeting, RAN4#101-e, a new way forward on Tx requirements was agreed [1]. The way forward asks interested companies to provide PC2 A-MPR proposals for certain network signalling. This contribution provides proposals for NS_48.

# 2  Discussion

###### 2.1 Simulation assumptions

The following assumptions for the simulations were used:

- Power Class 2

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

- Filter rejection at B3 Rx: 50dB

- Filter rejection at B34 Rx: 11dB

The simulations were done for NS_48. The spurious co-existence was according to table 1.

Table1: Co-existence for n1

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n1, n84 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |

###### 2.2 Simulation results

Simulations have been conducted for all relevant channel bandwidth. The PC3 regions were used as basis and adopted to suit the needs of PC2. Some of the A-MPR RB regions need to be extended in size to meet the higher power backoff demand for PC2. Selected results for DFT-s-OFDM and CP-OFDM with QPSK can be found in section 3. Table 2 contains the PC2 RB regions. Differences to PC3 are marked in red. Table 3 provides the A-MPR values for the individual regions.

Table 2: A-MPR regions for NS_48 (Power Class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A4 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≥0 | ≥max (0, 12*SCS*RBend - 1.08) < 9.72 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0 | ≥12.96 | A4 |
|  |  | ≥21.6 | <1.08 | A5 |
|  |  | ≥0 | ≥max (0, 12*SCS*RBend - 3.6) < 12.96 | A3 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <2.88 | ≥0 | A2 |
|  |  | ≥2.88, <17.1 | ≥max (0, 12*SCS*RBend - 4.68) | A3 |
|  |  | ≥17.1, <27.36 | ≥12.96 | A4 |
|  |  | ≥27.36, <34.56 | ≥12.96 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A3 |
|  |  | ≥34.56 | ≥0 | A1 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥0, <5.22 | >0 | A2 |
|  |  | ≥4.86, <19 | ≥max (0, 12*SCS*RBend - 5.4) | A4 |
|  |  | ≥19, <37.44 | ≥14.04 | A2 |
|  |  | ≥30.96, <37.44 | <1.08 | A5 |
|  |  | ≥37.44 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <7.2 | >0 | A2 |
|  |  | ≥6.12, <20.7 | ≥max (0, 12*SCS*RBend – 5.4) | A4 |
|  |  | ≥20.7, <41.04 | ≥15.12 | A2 |
|  |  | ≥33.84, <41.04 | <1.08 | A5 |
|  |  | ≥41.04 | >0 | A1 |

Table 3: A-MPR for NS_48 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤11 | ≤7.0 | ≤4.0 | ≤5.0 | ≤6.0 |
|  | QPSK | ≤11 | ≤7.0 | ≤4.0 | ≤5.0 | ≤6.0 |
|  | 16 QAM | ≤11 | ≤7.0 | ≤4.0 | ≤5.0 | ≤6.0 |
|  | 64 QAM | ≤11 | ≤7.0 | ≤4.0 | ≤5.0 | ≤6.0 |
|  | 256 QAM | ≤11 | ≤7.0 | ≤4.0 | ≤5.0 | ≤6.0 |
| CP-OFDM | QPSK | ≤12 | ≤8.0 | ≤5.5 | ≤6.5 | ≤6.0 |
|  | 16 QAM | ≤12 | ≤8.0 | ≤5.5 | ≤6.5 | ≤6.0 |
|  | 64 QAM | ≤12 | ≤8.0 | ≤5.5 | ≤6.5 | ≤6.0 |
|  | 256 QAM | ≤12 | ≤8.0 | ≤5.5 | ≤6.5 | ≤6.0 |

It has been found that there exist A-MPR need for channel bandwidth of 20MHz and below especially if the channel is close to lower band edge. Therefore, we propose the define A-MPR regions as captured in Table 3. Defining A-MPR allows to use RB allocations above 54 RB. When A-MPR is put into spec Table 2 and Table 4 can be merged of course.

Table 4: A-MPR regions for NS_48 (Power Class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 1925≤ FC ≤ 1975 | ≥0 | ≥8.1 | A3 |
|  |  | <1.8 | ≥0 | A3 |
| 15 MHz | 1927.5≤ FC ≤ 1972.5 | ≥0 | ≥9 | A3 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 2.88) <9 | A3 |
| 20 MHz | 1930≤ FC ≤ 1970 | ≥0 | ≥9.72 | A4 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 3.6) <9.72 | A3 |

Proposal: Use regions found in Table 2 and Table 4 with the A-MPR allowance found in Table 3 for NS_48

# 3  Simulations

###### 3.2 CBW above 20MHz

| Lower Edge | Upper Edge |
| --- | --- |

###### 3.2 CBW below 25MHz Lower Edge

# 4	Conclusions

Following the way forward on Tx requirements [1] this contribution provides A-MPR simulation results and proposals for NS_48 with PC2. The following proposal is made:

Proposal: Use regions found in Table 2 and Table 4 with the A-MPR allowance found in Table 3 for NS_48

# 4	References

- R4-2119946, “WF on NR FDD PC2 HPUE“, China Unicom, 3GPP TSG-RAN WG4 Meeting #101-e