3GPP TSG-RAN WG4 Meeting # 101b-e	R4-2200495

# Electronic Meeting, January 17-25, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	MPR for PC2 intra-band contiguous ULCA&MIMO 26+23 case

Agenda Item:	6.3.2.4	Intra-band UL contiguous CA for UL MIMO (n41C and n78C)	[NR_RF_FR1_enh-Core]

Document for:	Approval

# Introduction

In the RAN4#101 e-meeting, the MPR for PC2 contiguous UL CA with UL MIMO support using TxD was discussed in [1] and resulted in WF [2] covering both UL MIMO and TxD aspects. In this contribution, we provide additional measured data for 2Tx PC2 contiguous UL CA for 26+23dBm PA architecture, compare them to PC2 1Tx and 23+23 2Tx results and make MPR proposals.

# Discussion

## MPR and Architectures aspects to be confirmed

Here is the summary of agreements in [2] which are partially derived from measurements in [1].

Agreement: for 23+23dBm delta MPR

|  | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| contiguous inner | +0.5 | +1 |
| contiguous outer | +0.5 | +1 |
| non-contiguous inner | +0.5 | +1 |
| non-contiguous outer 1 | +1 | +1 |
| non-contiguous outer 2 | +1 | +1 |

Agreed WF for 26+26dBm MPR: In release 17 the 1Tx MPR can be reused without additional MPR for both 1Tx and 2Tx operations, i.e., rank-1 and rank-2 operations.

Agreed WF for 23+26dBm MPR: Depending on the capability signaling for 23+26dBm PC2 intra-band UL CA+UL-MIMO, either 23+23dBm MPR or 1Tx PC2 MPR will be applied

- FFS the capability signaling

  - Option 1: reusing TxD

  - Option 2: new signaling

WF UE capability options: FFS after the conclusion of 23+26 MPR and considering the following cases:

- Case1: 23+26 follows 1Tx PC2 MPR

- Case 2: 23+26 follows 23+23 MPR

As can be seen, the only open items are:

- The MPR requirement for the 26+23dBm case, and although in [1], we had measurements for this case we could not present them due to lack of time for analysis and knowing this was the lowest priority case

- Sorting the signaling aspects to be able to map the MPR.

## Measurement results for 2Tx PC2 contiguous UL CA based on 26+23 dBm architecture

Since the measurements follow exactly the same methodology as in [1], please refer to it for measurement conditions and setup.

The results in Tables 1 and 2 are based on the worst results from the three 2Tx measurements and compares the 26+23dBm case with the 23+23dBm case and the 1Tx PC2 (26dBm) case.

Table 1 summarizes the ACLR related Pmax results for contiguous intra-band ULCA with contiguous allocations for:

- PC2 1Tx

- PC2 2Tx with two 23dBm PAs for WC 2Tx measurement

- PC2 2Tx with one 26dBm PA and one 23dBm PAs for WC 2Tx measurement

Table 2 provides the same results and same analysis for contiguous intra-band ULCA with non-contiguous allocations.

Table 1: Maximum power for ACLR requirements for PC2 1Tx and 2Tx cases for contiguous allocations

| Waveform\ Tx mode& power class |  |  |  |  |  | PC2 1Tx 26 | PC2 2Tx 23+23 | PC2 2Tx 26+23 | 26+23 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WF | CC1+CC2 BW | type | CC1 alloc. | CC2 alloc. | RB Tot. |  |  |  | vs PC2 | vs 23+23 |
| CP-OFDM | 5+5 | Contiguous Inner | 12RB13 | 12RB0 | 24 | 25.9 | 25.4 | 25.6 | -0.3 | 0.2 |
|  | 20+5 |  | 53RB53 | 1RB0 | 54 | 25.4 | 25.1 | 25.2 | -0.2 | 0.1 |
|  | 20+20 |  | 53RB53 | 53RB0 | 106 | 25.5 | 24.5 | 24.9 | -0.6 | 0.4 |
|  | 50+50 |  | 135RB135 | 135RB0 | 270 | 25.4 | 24.3 | 25.2 | -0.2 | 0.9 |
| DFT-s- OFDM | 5+5 |  | 12RB13 | 12 | 24 | -1 | 25.1 | 25.6 | -0.7 | 0.5 |
|  | 20+20 |  | 50RB56 | 50RB0 | 100 | -0.8 | 24.7 | 24.8 | -1.4 | 0.1 |
|  | 50+50 |  | 135RB135 | 135RB0 | 270 | 25.5 | 24.2 | 24.4 | -1.1 | 0.2 |
| CP-OFDM | 5+5 | contiguous Outer | 25RB0 | 1RB0 | 26 | 22.9 | 22.1 | 22.3 | -0.6 | 0.2 |
|  | 5+5 |  | 25RB0 | 12RB0 | 37 | 22.2 | 21.5 | 21.8 | -0.4 | 0.3 |
|  | 5+5 |  | 25RB0 | 25RB0 | 50 | 22.4 | 21.9 | 22.1 | -0.3 | 0.2 |
|  | 20+5 |  | 53RB53 | 25RB0 | 78 | 22.5 | 21.5 | 21.7 | -0.8 | 0.2 |
|  | 20+5 |  | 106RB0 | 1RB0 | 107 | 22.4 | 21.7 | 21.9 | -0.5 | 0.2 |
|  | 20+5 |  | 106RB0 | 25RB0 | 131 | 22.4 | 21.6 | 21.9 | -0.5 | 0.3 |
|  | 50+5 |  | 135RB135 | 1RB0 | 136 | 23.6 | 22.6 | 23.1 | -0.5 | 0.5 |
|  | 50+20 |  | 135RB135 | 53RB0 | 188 | 24 | 22.8 | 23.5 | -0.5 | 0.7 |
|  | 20+20 |  | 106RB0 | 106RB0 | 212 | 22.5 | 21.6 | 22.1 | -0.4 | 0.5 |
|  | 50+5 |  | 270RB0 | 1RB0 | 271 | 22.8 | 21.7 | 22.5 | -0.3 | 0.8 |
|  | 50+50 |  | 270RB0 | 270RB0 | 540 | 22.2 | 21.2 | 22.1 | -0.1 | 0.9 |
| DFT-s-OFDM | 5+5 |  | 25RB0 | 1RB0 | 26 | 24.8 | 23.3 | 24.2 | -0.6 | 0.9 |
|  | 5+5 |  | 25RB0 | 12RB0 | 37 | 23.8 | 21.9 | 23.1 | -0.7 | 1.2 |
|  | 5+5 |  | 25RB0 | 25RB0 | 50 | 23.5 | 22.1 | 22.9 | -0.6 | 0.8 |
|  | 20+5 |  | 50RB56 | 1RB0 | 51 | -3.3 | 25.5 | 25.7 | -1.3 | 0.2 |
|  | 20+5 |  | 50RB56 | 25RB0 | 75 | 23.7 | 22 | 22.5 | -1.2 | 0.5 |
|  | 50+5 |  | 135RB135 | 1RB0 | 136 | 25.2 | 23.1 | 23.7 | -1.5 | 0.6 |
|  | 50+20 |  | 135RB135 | 50RB0 | 185 | 24.3 | 23.2 | 23.4 | -0.9 | 0.2 |
|  | 20+20 |  | 100RB6 | 100RB0 | 200 | 23.6 | 21.8 | 21.9 | -1.7 | 0.1 |
|  | 50+5 |  | 270RB0 | 1RB0 | 271 | 24.7 | 22.6 | 23 | -1.7 | 0.4 |

Table 2: Maximum power for ACLR requirements for PC2 1Tx and 2Tx cases for non-contiguous allocations

| Waveform\ Tx mode& power class |  |  |  |  |  | PC2 1Tx 26 | PC2 2Tx 23+23 | PC2 2Tx 26+23 | 26+23 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WF | CC1+CC2 BW | type | CC1 alloc. | CC2 alloc. | RB Tot. |  |  |  | vs PC2 | vs 23+23 |
| CP-OFDM | 10+10 | Non-contiguous | 1RB | 1RB | 2 | 22.4 | 21.3 | 21.6 | -0.8 | 0.3 |
|  | 10+10 |  | 1RB | 2RB | 3 | 22.1 | 21.4 | 21.8 | -0.3 | 0.4 |
|  | 10+10 |  | 2RB | 2RB | 4 | 22.6 | 21.4 | 21.9 | -0.7 | 0.5 |
|  | 10+10 |  | 1RB | 4RB | 5 | 22.4 | 20.6 | 21.1 | -1.3 | 0.5 |
|  | 10+10 |  | 4RB | 4RB | 8 | 22.5 | 22 | 22.3 | -0.2 | 0.3 |
|  | 10+10 |  | 1RB | 12RB | 13 | 23.5 | 22.4 | 22.8 | -0.7 | 0.4 |
|  | 10+10 |  | 12RB | 12RB | 24 | 22.5 | 21.3 | 21.8 | -0.7 | 0.5 |
|  | 10+10 |  | 25RB | 25RB | 50 | 22.3 | 21.4 | 21.8 | -0.5 | 0.4 |
|  | 20+40 |  | 1RB | 53RB | 54 | 25.2 | 24.6 | 24.9 | -0.3 | 0.3 |
|  | 20+40 |  | 2RB | 53RB | 55 | 24.3 | 23.6 | 23.7 | -0.6 | 0.1 |
|  | 20+40 |  | 25RB | 53RB | 78 | 22.2 | 21.7 | 21.8 | -0.4 | 0.1 |
|  | 40+40 |  | 53RB | 53RB | 106 | 22.7 | 21.6 | 22 | -0.7 | 0.4 |
|  | 50+50 |  | 1RB | 135RB | 136 | -1.5 | 25.9 | 26.2 | -0.3 | 0.3 |
|  | 50+50 |  | 53RB | 135RB | 188 | 22 | 20.7 | 21.2 | -0.8 | 0.5 |
|  | 40+40 |  | 106RB | 106RB | 212 | 22 | 21.2 | 21.5 | -0.5 | 0.3 |
|  | 50+50 |  | 135RB | 135RB | 270 | 21.6 | 20.6 | 21.7 | 0.1 | 1.1 |
|  | 50+50 |  | 1RB | 270RB | 271 | 21.9 | 21.5 | 22 | 0.1 | 0.5 |
| DFT-s-OFDM | 10+10 |  | 1RB | 1RB | 2 | 23.2 | 21.7 | 22.7 | -0.5 | 1.0 |
|  | 10+10 |  | 1RB | 2RB | 3 | 23.5 | 22 | 22.7 | -0.8 | 0.7 |
|  | 10+10 |  | 2RB | 2RB | 4 | 23.5 | 21.5 | 23.2 | -0.3 | 1.7 |
|  | 10+10 |  | 1RB | 4RB | 5 | 23.9 | 22.3 | 23.3 | -0.6 | 1.0 |
|  | 10+10 |  | 4RB | 4RB | 8 | 23.7 | 22.1 | 23.3 | -0.4 | 1.2 |
|  | 10+10 |  | 1RB | 12RB | 13 | 24.5 | 23.1 | 24 | -0.5 | 0.9 |
|  | 10+10 |  | 12RB | 12RB | 24 | 23.2 | 21.7 | 22.7 | -0.5 | 1.0 |
|  | 10+10 |  | 25RB | 25RB | 50 | 23.5 | 22 | 22.7 | -0.8 | 0.7 |
|  | 20+40 |  | 1RB | 50RB | 51 | 25.8 | 24.7 | 24.9 | -0.9 | 0.2 |
|  | 20+40 |  | 25RB | 50RB | 75 | 23.4 | 22 | 22.4 | -1.0 | 0.4 |
|  | 40+40 |  | 50RB | 50RB | 100 | 23.6 | 21.3 | 21.8 | -1.8 | 0.5 |
|  | 50+50 |  | 1RB | 135RB | 136 | -3.6 | 25.9 | 26.3 | -0.9 | 0.4 |
|  | 50+50 |  | 50RB | 135RB | 185 | 23.4 | 21.1 | 22 | -1.4 | 0.9 |
|  | 50+50 |  | 135RB | 135RB | 270 | 22.4 | 21.2 | 21.4 | -1.0 | 0.2 |
|  | 50+50 |  | 1RB | 270RB | 271 | 24 | 22.7 | 22.9 | -1.1 | 0.2 |

Notes on the results: The comparison analysis values highlighted in yellow are an extrapolation for the cases where there are still margin to ACLR at the 26dBm maximum power. In those cases, the delta value is calculated by assuming a 3dB/dB increase of ACLR vs. Pout. This is verified on the ACLR slope in our measurements.

Observations:

- When comparing 26+23dBm 2Tx measurements versus 1Tx PC2 measurements in the same setup and with the same PA, it can be seen that the same power cannot be achieved (negative margin), in general the difference is higher:

  - For DFT-s-OFDM versus CP-OFDM

  - For contiguous allocations versus non-contiguous allocation

- When comparing 26+23dBm 2Tx measurements versus 2Tx 23+23dBm measurements in the same setup and with the same PA, as expected the 26+23dBm architecture has margins in all cases. Accounting for the measurement error, it can be seen that the 26+23dBm architecture can achieve on average ~0.5dB better power than 23+23dBm case. there is some variation though in the results and in some cases, assuming that the measurement error is in the worst direction, there are case where only 0.3dB difference is achieved.

Given the results and the small improvement provided by the 26+23dBm we propose the following:

Proposal for 26+23dBm delta MPR: same delta MPR is adopted than for 23+23dBm case

| Allocation type | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| contiguous inner | +0.5 | +1 |
| contiguous outer | +0.5 | +1 |
| non-contiguous inner | +0.5 | +1 |
| non-contiguous outer 1 | +1 | +1 |
| non-contiguous outer 2 | +1 | +1 |

## Signalling Aspects

Before we discuss the signaling aspects for ULCA+MIMO it is worth noting that a similar discussion is on-going for 1CC 2Tx cases and last meeting GTW agreements are copied below from [3].

Agreement, Declaration of TxD for UE’s with at least one full power PA:

- Leave TxD as implementation aspect and assume that UE that does not declare TxD meets 1Tx requirements and has at least one full power PA

  - Only UE supporting 23+23 for PC2 and UE supporting 26+26 for PC1.5 are allowed to report TxD

    - FFS whether 1Tx PC2 MPR requirement or 23+23 TxD MPR requirement needs be applied to 23+26 UE

  - If PC2 UE does not report TxD, then 1Tx PC2 MPR requirement will be applied at least in one Tx operation mode

Agreement PC2 26+23 dBm MPR: encourage companies to provide more evaluation and measurement data in future.

It should be noted that there is an agreement that TxD can only be signaled for 2Tx PC2 and PC1.5 that correspond to the 2x1/2 architecture described in [4] and it seems logical that the same applies to the 2CC case (i.e. that it only applies to the 23+23dBm case for PC2 ULCA).

The PC2 2Tx 26+26dBm case for ULCA+MIMO is different from the 26+26dBm case for 1CC 2Tx PC1.5 case:

- First the output power is bounded by the PC2 MOP range and thus the overall UE thermal and power management is dimensioned for 26dBm instead of 29dBm

- If for single CC operation the 26+26dBm architecture can reach MOP for any allocation while in the CA case, as equal PSD is used between CC and not equal power, 29dBm can only be achieved for equal allocated RB bandwidth in each CC

- Compared to 1CC 2Tx PC1.5 case, the PC2 2Tx 26+26dBm case for ULCA+MIMO does not need to implement TxD to achieve 1Tx ULCA operation.

Observation:

- In the context of PC2 ULCA+MIMO, the 26+26dBm architecture cannot be associated with PC1.5 and is not mandated to implement or signal TxD

- Logically only the 23+23dBm architecture can signal TxD for PC2 ULCA (with or without MIMO).

Based on this observation, there is a need to distinguish the 26+23dBm and 26+26dBm cases for ULCA+MIMO that both do not signal TxD. To build on the approach adopted for PC1.5 MPR for smartphone and FWA, it is proposed to use modifiedMPR-Behaviour which given the fact that this architecture has large margins to the 1Tx/1PA PC2 ULCA MPR, will also allow updates in the future.

Proposal on PC2 ULCA+MIMO MPR mapping to TxD and modifiedMPR-Behaviour

| Architecture | 2Tx Delta MPR | 1Tx MPR | TxD | modifiedMPR-Behaviour |
| --- | --- | --- | --- | --- |
| 23+23dBm | Yes as in section 1.1 in WF R4-2119954 | Same as 2TX | yes | no |
| 26+23dBm | Yes (same as 23+23dBm) | 1Tx PC2 MPR in section 6.2A.2.1 | no | no |
| 26+26dBm | no | 1Tx PC2 MPR in section 6.2A.2.1 | no | yes |

## Applicability of the 23+23dBm case to ULCA without MIMO

As proposed in [4], it should be noted that PC2 delta MPR agreed to in R4-2119954 for PC2 contiguous ULCA+MIMO 23+23dBm case can apply for PC2 contiguous ULCA without MIMO (TxD is signaled). This mode is needed for this architecture when MIMO is no longer configured for PC2 ULCA+MIMO while the other architectures can use the available 26dBm PA to meet the 1TX PC2 MPR as shown in the Table above.

Proposal for contiguous UL CA without MIMO for PC2: A case for bandwidth class B and C is added in section 6.2A.2.1 of 38.101-1for UE signaling TxD with delta MPR vs Table 6.2A.2.1-1a and Table 6.2A.2.1-3, as in section 1.1 in WF R4-2119954.

# Conclusions

In this contribution, we provided measurement update for 2Tx PC2 intra-band contiguous UL CA 26+23dBm case on top of last meeting measurements for 2x26dBm and 2x23dBm architectures, it allowed the following proposals.

Proposal for 26+23dBm delta MPR: same delta MPR is adopted than for 23+23dBm case

| Allocation type | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| contiguous inner | +0.5 | +1 |
| contiguous outer | +0.5 | +1 |
| non-contiguous inner | +0.5 | +1 |
| non-contiguous outer 1 | +1 | +1 |
| non-contiguous outer 2 | +1 | +1 |

Proposal on PC2 ULCA+MIMO MPR mapping to TxD and modifiedMPR-Behaviour

| Architecture | 2Tx Delta MPR | 1Tx MPR | TxD | modifiedMPR-Behaviour |
| --- | --- | --- | --- | --- |
| 23+23dBm | Yes as in section 1.1 in WF R4-2119954 | Same as 2TX | yes | no |
| 26+23dBm | Yes (same as 23+23dBm) | 1Tx PC2 MPR in section 6.2A.2.1 | no | no |
| 26+26dBm | no | 1Tx PC2 MPR in section 6.2A.2.1 | no | yes |

Proposal for contiguous UL CA without MIMO for PC2: A case for bandwidth class B and C is added in section 6.2A.2.1 of 38.101-1for UE signaling TxD with delta MPR vs Table 6.2A.2.1-1a and Table 6.2A.2.1-3, as in section 1.1 in WF R4-2119954.

# References

- R4-2118115 MPR for PC2 intra-band contiguous UL CA with UL MIMO support, Skyworks Solutions Inc., RAN4#101e

- R4-2119954 WF on PC2 intra-band contiguous UL CA with UL MIMO, OPPO, RAN4#101e

- R4-2119970 WF on PC2 TxD implementations with 26+23 and 26+26 PA’s, Qualcomm Incorporated, RAN4#101e

- R4-2200499 Requirement and signaling aspect of features requiring two transmit paths, Skyworks Solutions Inc., RAN4#101b-e