3GPP TSG-RAN WG4 Meeting # 101b-e	R4-2200496

# Electronic Meeting, January 17-25, 2022

Source: 	Skyworks Solutions, Inc., Apple

Title: 	RedCap UL Architecture and power class

Agenda Item:	6.20.2.1.1 Tx requirements (power class) [NR_redcap-Core]

Document for:	Approval

# Introduction

In previous R4#101 e-meetings, a way forward [1] captured a number of agreements including those achieved in the dedicated GTW session. Notably, a number of power class agreements were achieved and, in this contribution, we further discuss the potential for PC2 implementations and preferred approach.

# Discussion

In R4#101e, the way forward [1] captured following GTW agreements on FR1 power class and given guideline in [2] , Issue 1-1 is no longer relevant:

Issue 1-1: PC5 support

Agreement:

PC5 will be supported in Rel-17, if n46 and n96 is agreed to be supported in Rel-17.

Issue 1-2: PC2 support

Agreement: Support PC2 RedCap UE  based on operator request.

- Use 1 Tx as baseline

- Further discussion whether the antenna isolation assumptions for existing 2Tx requirements is valid for RedCap.

Issue 1-3: UE architecture for FR1 RedCap UE

Agreement: to define the Tx RF requirements for RedCap

- Use 1 Tx as baseline

- 2Tx is kept open for discussion

In this contribution we will tackle aspects pertaining to issues 1-2 and 1-3 on PC2 support and 2Tx architecture.

## Two Tx architecture aspects

One of the main aspects of a RedCap UE architecture is that only one receive antenna is the default and two receive antennas is optional, thus in any case it is unlikely that one RedCap UE would use 2Tx when only one antenna is implemented.

If it is already agreed that 1Tx is the baseline and 2Tx is open for discussion, it is obviously only worth to discuss it in the scope of a UE that already supports 2 antennas in receive.

One aspect of supporting 2Tx is related to the antenna design and whether it may be feasible, for some form factors and frequencies, to equip a RedCap device with two antennas. Even if two antennas are implemented, there are still questions as to which antenna efficiency is achieved, but also how much coupling may be expected between the two antennas. Additionally wearables are almost constantly close to the body and thus may see further degraded antenna coupling and performance:

- Even if two antennas are available, there may be limited diversity gain and radiated power may be less than that of a single good antenna.

- Also all 2TX MPR in RAN4, which is higher than for 1Tx, have assumed at least 10dB isolation between antennas which may be difficult to obtain in a redcap UE, and thus, the current 2Tx MPR values derived for smartphone devices may not be valid.

- 2Tx PC3 MPR is currently re-using 1Tx MPR which is based on the assumption that either:

  - At least one PC3 PA is available; or

  - There is significantly higher antenna isolation than 10dB; or

  - Higher power than 20dBm is available from the two PAs.

For all the above reasons and given the time left for Release 17, we propose to restrict RedCap to 1Tx and postpone 2Tx to Release 18 if use-cases justify a higher UL throughput and UL MIMO. Even so, 1Tx operation using TxD may be further challenging for a small form factor.

Proposal 1:

- For RedCap UEs in Release 17:

  - Only 1Tx PC3 architecture (23dBm PA) is specified.

  - If PC2 support is considered, only 1Tx architecture (26dBm PA) is specified.

- Postpone 2Tx studies to Release 18

## PC2 applicability to HD-FDD

Another specific aspect of RedCap is that HD-FDD mode is available in FDD bands, which means that TDD-like operation is expected and thus a PC2 PA could be used, provided that:

- HD-FDD would ensure less than 50% UL duty cycle

- Post PA losses are similar to TDD as heavy TX filtering is not required in HD-FDD.

- Note that this is independent to the R17 PC2 FDD work started in RAN4.

This approach can be beneficial to RedCap UEs as the antenna performance may not enable good radiated power to maintain a good link and some DL or UL use cases could benefit from the additional 3dB of power and guaranty a good cell range.

Proposal 2: in FDD bands, when HD-FDD is used, to mitigate the UL duty cycle loss and poor antenna performance, support of 1Tx PC2 is further studied.

# Conclusions

In this contribution, we examine the architecture and power class options for FR1 RedCap UEs and make the following proposals.

Proposal 1:

- For RedCap UEs in Release 17:

  - Only 1Tx PC3 architecture (23dBm PA) is specified.

  - If PC2 support is considered, only 1Tx architecture (26dBm PA) is specified.

- Postpone 2Tx studies to Release 18

Proposal 2: in FDD bands, when HD-FDD is used, to mitigate the UL duty cycle loss and poor antenna performance, support of 1Tx PC2 is further studied.

# References

- R4-2120067 WF on the RedCap RF, Ericsson, RAN4#101e

- RP-213616 Moderator’s proposals from [94e-39-R17-RedCap-WI] (for discussion in GTW), Intel, RP#94e