3GPP TSG-RAN WG4 Meeting # 101bis-e	R4-2200555

# Electronic Meeting, January 17-25, 2022

Source:	LG Electronics

Title:	RF requirements for CA_n257A_n259A based on IBM

Agenda item:	6.4.2.2.2

Document for:	Discussion

Introduction

In this paper, we provide our view on RF requirements for inter-band UL CA_n257A_n259A based on IBM.

Discussion

In the last RAN4 meeting, one WF[1] was made for inter-band UL CA with IBM.

Based on the WF, we would like to discuss MOP, PA-PA interaction, minimum peak EIRP, and spherical coverage for CA_n257A_n259A.

R4-2119960 [1]

Way forward on FR2 Inter-band UL CA - MOP and MPR framework - RAN4 agree to down-select the relaxation framework from the following two options, where X stands for relaxation for one band and Y for the other band - Option 1 - CA MOP = single carrier MOP – X&Y - CA MPR = max { MPRPA-PA, MPRwaveform&modulation&BW&etc } - Option 2 - CA MOP = single carrier MOP - CA MPR = max { X&Y, MPRPA-PA, MPRwaveform&modulation&BW&etc } - MBR handling - Option 1 - MBR is part of X&Y - Option 2 - MBR is part of single carrier MOP - Total power handling - UE power consumption should be addressed and further study how to address it - PA-PA interaction - how to incorporate PA-PA interaction - Agreement - It is agreed that PA-PA interaction is included in CA MPR (i.e. Option 1). - PA-PA interaction aspect - Agreement - RAN4 should assume the worst-case PA-PA interaction, which arises from the UE using the same or collocated antenna arrays to form the UL beams (i.e. Option 2). - how to define MPRPA-PA - Agreement - [MPRPA-PA per band combination. MPRPA-PA applies equally to both bands in the ULCA band combination when PAs are activated simultaneously]. - Other solution is not precluded - Relaxation, X & Y for CA_n257A_n259A - X and Y for min peak EIRP & Sphericaql coverage. - Options - Option 1: X=Y=3.5 dB - Option 2: X=3.7 dB and Y=3.5 dB - Option 3: ΔTIB,P,n shall be ΔRIB,P,n -1 dB - Option 4: X and Y = 0 but UL CA relaxation included in CA MPR - Option 5: n257=22.4-X dBm, n259=18.7-Y dBm, where X = Y = 2.5 dB. - Option 6: X=Y=9 dB - Agreement - Relaxation factors analysis for X and Y are encouraged

For CA MOP

Figure 2.1 shows 2 options for CA MOP in [1]. We think there is no difference of CA Tx power after MPR between option 1 and option 2 because MPRs(MPRPA-PA & MPRwaveform&modulation&BW&etc) in option 2 can be always considered as being X&Y larger than MPR of option 1 with assumption that MPRs is derived based on CA_MOP and X&Y are derived based on single carrier MOP.

In addition, regarding that MOP of single carrier in FR2-1 is defined with assumption dual polarization, CA_MOP in option 1 can be interpreted as being based on single polarization and CA_MOP in option 2 can be interpreted as being based on dual polarization. In the aspect of minimum requirements, option 1 is more preferred.

Figure 2.1:  Options for CA MOP

Proposal 1: Define CA MOP and CA MPR with Option 1.

  - Option 1

    - CA MOP = single carrier MOP – X&Y

    - CA MPR = max { MPRPA-PA, MPRwaveform&modulation&BW&etc }

For MBR, it needs to be handled according to the band combination. Our preference is to handle it as part of X&Y.

Proposal 2: Handle MBR as part of X&Y (option 1).

  - Option 1

    - MBR is part of X&Y

Relaxation, X & Y for CA_n257A_n259A

For inter-band DL CA_n257A_n259A based on IBM, relaxation of EIS and EIS spherical coverage for power class 3 were specified as RIB,P = 4dB, RIB.S=4dB.  For the delta_RIB, MBR was not considered. Therefore, we do not need to consider delta_RIB for X&Y.

Taking MBR of UE supporting n257 and n259 with single RF into account,  MBR of 0.7dB can be part of X for n257 and MBR of 0.5dB can be part of Y for n259. On top of it, as mentioned in  CA_MOP, in the aspect of implementation, it is possible to activate single pole between V and H for n257 and/or for n259.  It can be relaxed up to 3dB additionally compared to single carrier MOP.

Proposal 3: Specify relaxation of 3.7dB(X) in n257 and 3.5dB(Y) in n259 for mimim peak EIRP and spherical coverage(option2).

  - X and Y for min peak EIRP & Sphericaql coverage.

    - Option 2: X=3.7 dB and Y=3.5 dB

Conclusion

In this contribution, we provided our views on the RF requirement for inter-band UL CA_n257A_n259A based on IBM. Proposals are as follows.

For CA MOP

Proposal 1: Define CA MOP and CA MPR with Option 1.

  - Option 1

    - CA MOP = single carrier MOP – X&Y

    - CA MPR = max { MPRPA-PA, MPRwaveform&modulation&BW&etc }

Proposal 2: Handle MBR as part of X&Y (option 1).

  - Option 1

    - MBR is part of X&Y

Relaxation, X & Y for CA_n257A_n259A

Proposal 3: Specify relaxation of 3.7dB(X) in n257 and 3.5dB(Y) in n259 for mimim peak EIRP and spherical coverage(option2).

  - X and Y for min peak EIRP & Sphericaql coverage.

    - Option 2: X=3.7 dB and Y=3.5 dB

- Reference

[1] R4-2119960, “WF on UL CA based on IBM”, Samsung