3GPP TSG-RAN WG4 Meeting # 101bis-e	R4-2200556

# Electronic Meeting, January 17-25, 2022

Source:	LG Electronics

Title:	MPR for NR V2X intra-band con-current operation with Uu

Agenda item:	6.15.3.1

Document for:	Approval

Introduction

For MPR in TR 38.785[2], we provide more MPR simulation results considering a combination of each modulation order(MO). Based on the simulation results, we would like to propose the MPR for NR V2X intra-band con-current operation.

Discussion

In [2], the MPRs for PC3 and PC2 are captured as follows.

Table 5.2.4.1.2-3: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.0] |
|  | 16QAM | ≤ [1.0] | ≤ [1.0] |
|  | 64QAM | ≤ [1.0] | ≤ [1.0] |
|  | 256QAM | ≤ [1.5] | ≤ [1.5] |

Table 5.2.4.1.2-4: Non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 16QAM | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 64QAM | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 256QAM | ≤ [1.5] | ≤ [2.0] | ≤ [3.0] |

Table 5.2.4.1.2-5: Contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [1.5] | ≤ [3.5] |
|  | 16QAM | ≤ [2.0] | ≤ [3.5] |
|  | 64QAM | ≤ [3.0] | ≤ [3.5] |
|  | 256QAM | ≤ [5.0] | ≤ [5.0] |

Table 5.2.4.1.2-6: Non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [4.0] | ≤ [6.0] |
|  | 16QAM | ≤ [2.5] | ≤ [4.0] | ≤ [6.0] |
|  | 64QAM | ≤ [3.5] | ≤ [4.5] | ≤ [6.0] |
|  | 256QAM | ≤ [4.5] | ≤ [5.0] | ≤ [6.0] |

For the MPRs, we did more simulations with different combinations of SL MO and UL MO with the following simulation assumptions.

Considered simulation assumptions.

- Channel BW configurations for class B - 10MHz (NR SL, 30kHz SCS) + 10MHz (NR Uu, 30kHz SCS) - Waveform - CP-OFDM(NR SL) + CP-OFDM(NR Uu) - Maximum total output power - 26dBm : 23dBm 2 PAs - Modulation order - {SL, UL} = {QPSK, QPSK/16QAM/64QAM/256QAM} {16QAM, QPSK/16QAM/64QAM/256QAM} {64QAM, QPSK/16QAM/64QAM/256QAM} {256QAM, QPSK/16QAM/64QAM/256QAM} - RB allocation - Refer table 2-1 & 2-2

The following tables show simulation results of MPR with a maximum total output power of 26dBm assuming 23dBm 2 PAs.

- Table 2.1: MRR for contiguous RB allocation

- Table 2.2: MPR for non-contiguous RB allocation

Table 2.1 : MPR for  contiguous RB allocation in 10MHz(SL) + 10MHz(UL) @ Max.Total.Power = 26dBm

| SL Config. | UL Config. |  |  | MPR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CBW(10MHz): SCS(30kHz) | CBW(10MHz): SCS(30kHz) |  |  |  |  |  |  |
| (L_CRB,StartRBidx) |  |  |  | UL Modulation Order(MO) |  |  |  |
| SL_RB | UL_RB | Inner/Outer | SL MO | QPSK | 16QAM | 64QAM | 256QAM |
| (10,14) | (1,0) | Inner | QPSK | 0.13 | 0.43 | 1.47 | 3.14 |
| (10,14) | (13,0) | Inner | QPSK | 0.28 | 0.69 | 1.90 | 3.89 |
| (10,14) | (1,0) | Inner | 16QAM | 0.84 | 0.81 | 1.48 | 3.18 |
| (10,14) | (13,0) | Inner | 16QAM | 0.63 | 0.68 | 1.91 | 3.91 |
| (10,14) | (1,0) | Inner | 64QAM | 2.08 | 2.15 | 2.08 | 3.04 |
| (10,14) | (13,0) | Inner | 64QAM | 1.86 | 1.88 | 1.96 | 3.90 |
| (10,14) | (1,0) | Inner | 256QAM | 4.06 | 4.07 | 4.11 | 4.02 |
| (10,14) | (13,0) | Inner | 256QAM | 3.72 | 3.64 | 3.67 | 3.87 |
| (10,14) | (14,0) | Outer | QPSK | 0.40 | 0.77 | 2.00 | 3.91 |
| (10,14) | (24,0) | Outer | QPSK | 2.29 | 2.27 | 2.28 | 4.54 |
| (10,14) | (14,0) | Outer | 16QAM | 0.63 | 0.75 | 1.99 | 3.98 |
| (10,14) | (24,0) | Outer | 16QAM | 2.30 | 2.27 | 2.27 | 4.42 |
| (10,14) | (14,0) | Outer | 64QAM | 1.82 | 1.86 | 2.00 | 3.95 |
| (10,14) | (24,0) | Outer | 64QAM | 2.29 | 2.29 | 2.27 | 4.43 |
| (10,14) | (14,0) | Outer | 256QAM | 3.73 | 3.64 | 3.69 | 3.99 |
| (10,14) | (24,0) | Outer | 256QAM | 3.65 | 3.64 | 3.68 | 4.31 |

Table 2.2 : MPR for  non-contiguous RB allocation in 10MHz(SL) + 10MHz(UL) @ Max.Total.Power = 26dBm

| SL Config. | UL Config. |  |  | MPR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CBW(10MHz): SCS(30kHz) | CBW(10MHz): SCS(30kHz) |  |  |  |  |  |  |
| (L_CRB,StartRBidx) |  |  |  | UL Modulation Order(MO) |  |  |  |
| SL_RB | UL_RB | Inner/Outer1 /Outer2 | SL MO | QPSK | 16QAM | 64QAM | 256QAM |
| (10,14) | (1,2) | Inner | QPSK | 0.14 | 0.34 | 1.53 | 3.20 |
| (10,14) | (1,2) | Inner | 16QAM | 0.68 | 0.66 | 1.40 | 3.10 |
| (10,14) | (1,2) | Inner | 64QAM | 1.90 | 1.94 | 1.92 | 3.07 |
| (10,14) | (1,2) | Inner | 256QAM | 3.85 | 3.86 | 3.86 | 3.86 |
| (10,14) | (14,3) | Outer1 | QPSK | 1.10 | 1.13 | 1.93 | 3.91 |
| (10,14) | (14,10) | Outer1 | QPSK | 2.46 | 2.47 | 2.44 | 3.70 |
| (10,14) | (14,3) | Outer1 | 16QAM | 1.10 | 1.10 | 1.92 | 3.92 |
| (10,14) | (14,10) | Outer1 | 16QAM | 2.46 | 2.49 | 2.46 | 3.66 |
| (10,14) | (14,3) | Outer1 | 64QAM | 1.81 | 1.81 | 1.93 | 3.97 |
| (10,14) | (14,10) | Outer1 | 64QAM | 2.46 | 2.44 | 2.47 | 3.73 |
| (10,14) | (14,3) | Outer1 | 256QAM | 3.67 | 3.65 | 3.55 | 3.89 |
| (10,14) | (14,10) | Outer1 | 256QAM | 3.64 | 3.58 | 3.64 | 3.68 |
| (10,9) | (1,12) | Outer2 | QPSK | 2.29 | 2.29 | 2.29 | 3.71 |
| (10,9) | (1,12) | Outer2 | 16QAM | 2.29 | 2.32 | 2.32 | 4.30 |
| (10,9) | (1,12) | Outer2 | 64QAM | 2.29 | 2.30 | 2.30 | 3.86 |
| (10,9) | (1,12) | Outer2 | 256QAM | 3.81 | 3.85 | 3.92 | 4.08 |

From Table 2.1 and Table 2.2, we can see different MPRs under the same SL MO according to the combination of UL MO.

Table 2.3 and Table 2.4 summarize maximum MPR among the relative differences to MPR of the combination of {QPSK, QPSK} for all MO combinations in each Inner/Outer for Table 2.3 and in each Inner/Outer1/Outer2 for Table 2.4.

Table 2.3 : Relative maximum difference of other MO combinations from a maximum MPR of {QPSK, QPSK} for contiguous RB allocation in 10MHz(SL) + 10MHz(UL) @ Max.Total.Power = 26dBm

|  |  | Relative MPR from {QPSK, QPSK} |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | UL Modulation Order(MO) |  |  |  |
| Inner/Outer | SL MO | QPSK | 16QAM | 64QAM | 256QAM |
| Inner | QPSK | 0 | 0.41 | 1.62 | 3.61 |
|  | 16QAM | 0.56 | 0.53 | 1.63 | 3.63 |
|  | 64QAM | 1.8 | 1.87 | 1.8 | 3.62 |
|  | 256QAM | 3.78 | 3.79 | 3.83 | 3.74 |
| Outer | QPSK | 0 | -0.02 | -0.01 | 2.25 |
|  | 16QAM | 0.01 | -0.02 | -0.02 | 2.13 |
|  | 64QAM | 0 | 0 | -0.02 | 2.14 |
|  | 256QAM | 1.44 | 1.35 | 1.4 | 2.02 |

Table 2.4 : Relative maximum difference of other MO combinations from a maximum MPR of {QPSK, QPSK}  for  non-contiguous RB allocation in 10MHz(SL) + 10MHz(UL) @ Max.Total.Power = 26dBm

|  |  | MPR |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | UL Modulation Order(MO) |  |  |  |
| Inner/Outer1 /Outer2 | SL MO | QPSK | 16QAM | 64QAM | 256QAM |
| Inner | QPSK | 0 | 0.2 | 1.39 | 3.06 |
|  | 16QAM | 0.54 | 0.52 | 1.26 | 2.96 |
|  | 64QAM | 1.76 | 1.8 | 1.78 | 2.93 |
|  | 256QAM | 3.71 | 3.72 | 3.72 | 3.72 |
| Outer1 | QPSK | 0 | 0.01 | -0.02 | 1.45 |
|  | 16QAM | 0 | 0.03 | 0 | 1.46 |
|  | 64QAM | 0 | -0.02 | 0.01 | 1.51 |
|  | 256QAM | 1.21 | 1.19 | 1.18 | 1.43 |
| Outer2 | QPSK | 0 | 0 | 0 | 1.42 |
|  | 16QAM | 0 | 0.03 | 0.03 | 2.01 |
|  | 64QAM | 0 | 0.01 | 0.01 | 1.57 |
|  | 256QAM | 1.52 | 1.56 | 1.63 | 1.79 |

Based on the difference, there can be 3 options for how to define MPR for intra-band con-current operation in a licensed band.

  - Option1: Define MPR for all combinations of SL MO and UL MO

  - Option 2: Define MPR based on the combination of SL MO and highest UL MO

  - Option 3: Define MPR based on the higher MO between SL MO and UL MO

For Option 1,

- PC2 MPR can be updated as

  - Table 2.5 for contiguous RB allocation taking the difference of Table 2.3 on top of Table 5.2.4.1.2-5 into account

  - Table 2.6 for non-contiguous RB allocation taking the difference of Table 2.4 on top of Table 5.2.4.1.2-6 into account.

- PC3 MPR can be updated as

  - Table 2.7 for contiguous RB allocation taking the difference of Table 2.3 on top of Table 5.2.4.1.2-3 into account

  - Table 2.8 for non-contiguous RB allocation taking the difference of Table 2.4 on top of Table 5.2.4.1.2-4 into account.

For Option 2,

- PC2 MPR can be updated as

  - Table 2.9 for contiguous RB allocation taking the difference of Table 2.3 on top of Table 5.2.4.1.2-5 into account

  - Table 2.10 for non-contiguous RB allocation taking the difference of Table 2.4 on top of Table 5.2.4.1.2-6 into account.

- PC3 MPR can be updated as

  - Table 2.11 for contiguous RB allocation taking the difference of Table 2.3 on top of Table 5.2.4.1.2-3 into account

  - Table 2.12 for non-contiguous RB allocation taking the difference of Table 2.4 on top of Table 5.2.4.1.2-4 into account.

For Option 3,

- PC2 MPR can be updated as

  - Table 2.13 for contiguous RB allocation taking the difference of Table 2.3 on top of Table 5.2.4.1.2-5 into account

  - Table 2.14 for non-contiguous RB allocation taking the difference of Table 2.4 on top of Table 5.2.4.1.2-6 into account.

- PC3 MPR can be updated as

  - Table 2.15 for contiguous RB allocation taking the difference of Table 2.3 on top of Table 5.2.4.1.2-3 into account

  - Table 2.16 for non-contiguous RB allocation taking the difference of Table 2.4 on top of Table 5.2.4.1.2-4 into account.

Table 2.5: Contiguous RB allocation for Power Class 2 (Option 1)

|  |  | MPR for bandwidth class B(dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner |  |  |  | outer |  |  |  |
| Modulation |  | UL MO |  |  |  | UL MO |  |  |  |
| SL MO |  | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| CP-OFDM | QPSK | ≤ [1.5] | ≤ [2.0] | ≤ [3.0] | ≤ [5.0] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [5.5] |
|  | 16QAM | ≤ [2.0] | ≤ [2.0] | ≤ [3.0] | ≤ [5.0] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [5.5] |
|  | 64QAM | ≤ [3.0] | ≤ [3.0] | ≤ [3.0] | ≤ [5.0] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [5.5] |
|  | 256QAM | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.5] |

Table 2.6: Non-Contiguous RB allocation for Power Class 2 (Option 1)

|  |  | MPR for bandwidth class B(dB) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner |  |  |  | Outer1 |  |  |  | Outer2 |  |  |  |
| Modulation |  | UL MO |  |  |  | UL MO |  |  |  | UL MO |  |  |  |
| SL MO |  | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [2.0] | ≤ [3.0] | ≤ [5.0] | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] | ≤ [5.5] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.5] |
|  | 16QAM | ≤ [2.5] | ≤ [2.5] | ≤ [3.0] | ≤ [5.0] | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] | ≤ [5.5] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.5] |
|  | 64QAM | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [5.0] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] | ≤ [5.5] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.5] |
|  | 256QAM | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [6.5] | ≤ [6.5] | ≤ [6.5] | ≤ [6.5] |

Table 2.7: Contiguous RB allocation for Power Class 3 (Option 1)

|  |  | MPR for bandwidth class B(dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner |  |  |  | outer |  |  |  |
| Modulation |  | UL MO |  |  |  | UL MO |  |  |  |
| SL MO |  | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.5] |
|  | 16QAM | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.5] |
|  | 64QAM | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.5] |
|  | 256QAM | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.5] |

Table 2.8: Non-Contiguous RB allocation for Power Class 3 (Option 1)

|  |  | MPR for bandwidth class B(dB) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner |  |  |  | Outer1 |  |  |  | Outer2 |  |  |  |
| Modulation |  | UL MO |  |  |  | UL MO |  |  |  | UL MO |  |  |  |
| SL MO |  | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.5] | ≤ [1.5] | ≤ [1.5] | ≤ [2.5] | ≤ [3.0] | ≤ [3.0] | ≤ [3.0] | ≤ [3.5] |
|  | 16QAM | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.5] | ≤ [1.5] | ≤ [1.5] | ≤ [2.5] | ≤ [3.0] | ≤ [3.0] | ≤ [3.0] | ≤ [3.5] |
|  | 64QAM | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.5] | ≤ [1.5] | ≤ [1.5] | ≤ [2.5] | ≤ [3.0] | ≤ [3.0] | ≤ [3.0] | ≤ [3.5] |
|  | 256QAM | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] |

Table 2.9: Contiguous RB allocation for Power Class 2 (Option 2)

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [5.0] | ≤ [5.5] |
|  | 16QAM | ≤ [5.0] | ≤ [5.5] |
|  | 64QAM | ≤ [5.0] | ≤ [5.5] |
|  | 256QAM | ≤ [5.0] | ≤ [5.5] |

Table 2.10: Non-Contiguous RB allocation for Power Class 2 (Option 2)

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [5.0] | ≤ [5.5] | ≤ [6.5] |
|  | 16QAM | ≤ [5.0] | ≤ [5.5] | ≤ [6.5] |
|  | 64QAM | ≤ [5.0] | ≤ [5.5] | ≤ [6.5] |
|  | 256QAM | ≤ [5.5] | ≤ [5.5] | ≤ [6.5] |

Table 2.11: Contiguous RB allocation for Power Class 3 (Option 2)

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [2.5] |
|  | 16QAM | ≤ [2.0] | ≤ [2.5] |
|  | 64QAM | ≤ [2.0] | ≤ [2.5] |
|  | 256QAM | ≤ [2.0] | ≤ [2.5] |

Table 2.12: Non-Contiguous RB allocation for Power Class 3 (Option 2)

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [2.5] | ≤ [3.5] |
|  | 16QAM | ≤ [2.0] | ≤ [2.5] | ≤ [3.5] |
|  | 64QAM | ≤ [2.0] | ≤ [2.5] | ≤ [3.5] |
|  | 256QAM | ≤ [2.5] | ≤ [2.5] | ≤ [3.5] |

Table 2.13: Contiguous RB allocation for Power Class 2 (Option 3)

| Higher Modulation of {SL MO, UL MO} |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [1.5] | ≤ [3.5] |
|  | 16QAM | ≤ [2.0] | ≤ [3.5] |
|  | 64QAM | ≤ [3.0] | ≤ [3.5] |
|  | 256QAM | ≤ [5.0] | ≤ [5.5] |

Table 2.14: Non-Contiguous RB allocation for Power Class 2 (Option 3)

| Higher Modulation of {SL MO, UL MO} |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [4.0] | ≤ [6.0] |
|  | 16QAM | ≤ [2.5] | ≤ [4.0] | ≤ [6.0] |
|  | 64QAM | ≤ [3.5] | ≤ [4.5] | ≤ [6.0] |
|  | 256QAM | ≤ [5.5] | ≤ [5.5] | ≤ [6.5] |

Table 2.15: Contiguous RB allocation for Power Class 3 (Option 3)

| Higher Modulation of {SL MO, UL MO} |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.0] |
|  | 16QAM | ≤ [1.0] | ≤ [1.0] |
|  | 64QAM | ≤ [1.0] | ≤ [1.0] |
|  | 256QAM | ≤ [2.0] | ≤ [2.5] |

Table 2.16: Non-Contiguous RB allocation for Power Class 3 (Option 3)

| Higher Modulation of {SL MO, UL MO} |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 16QAM | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 64QAM | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 256QAM | ≤ [2.5] | ≤ [2.5] | ≤ [3.5] |

Based on the analysis, we propose as follows,

Proposal 1: Define MPR for intra-band con-current operation in a licensed band with one option among the following 3 options.

- Option1: Define MPR with Table 2.5 and Table 2.6 for Power Class 2, Table 2.7 and Table 2.8 for Power Class 3 considering all combinations of SL MO and UL MO.

- Option 2: Define MPR with Table 2.9 and Table 2.10 for Power Class 2, Table 2.11 and Table 2.12 for Power Class 3 considering the combination of SL MO and highest UL MO(256QAM).

- Option 3: Define MPR with Table 2.13 and Table 2.14 for Power Class 2, Table 2.15 and Table 2.16 for Power Class 3 considering the higher MO  between SL MO and UL MO.

Proposal 2: Update TP for MPR with one option in Proposal 1.

For simplicity, option 3 is preferred.

Conclusion

Based on the MPR simulation results, we propose as follows.

Proposal 1: Define MPR for intra-band con-current operation in a licensed band with one option among the following 3 options.

- Option1: Define MPR with Table 2.5 and Table 2.6 for Power Class 2, Table 2.7 and Table 2.8 for Power Class 3 considering all combinations of SL MO and UL MO.

- Option 2: Define MPR with Table 2.9 and Table 2.10 for Power Class 2, Table 2.11 and Table 2.12 for Power Class 3 considering the combination of SL MO and highest UL MO(256QAM).

- Option 3: Define MPR with Table 2.13 and Table 2.14 for Power Class 2, Table 2.15 and Table 2.16 for Power Class 3 considering the higher MO  between SL MO and UL MO.

Proposal 2: Update TP for MPR with one option in Proposal 1.

- Reference

[1] R4-2114983, “WF on MPR for intra-band V2X con-current operation”, LG Electronics

[2] TR 38.785 V0.3.0

R4-2107870, “WF on MPR for intra-band V2X con-current operation”, LG Electronics