3GPP TSG-RAN WG4 Meeting #101bis-e	R4-2200578

Electronic Meeting, January 17th – 25th 2022

Title: 	Discussion on ACIR requirements for 52.6-71 GHz

Source: 	Korea Testing Laboratory

Agenda item:	6.16.5

Document for:	Discussion

- Introduction

In [1], companies presented the simulation results in order to decide ACIR requirements under the coexistence environment in NR beyond 52.6 GHz. Since simulation results have differences in [1] and [2], we had investigated the reason while varying the simulation parameters to compare.

- Discussion

Although ACIR requirements for 70 GHz were studied in [2], ACIR requirements for NR beyond 52.6 GHz have been studied again according to system environment changes for example low transmission power, radiation pattern of antenna elements, and the number of transmitting antennas.

In [3], the simulation parameters such as transmission power, radiation patterns of BS and UE etc. are discussed and as a result, the simulation parameters in [2] were partially changed and these agreed simulation parameters were used in [1]. In this contribution, we analysis effect of transmission power, antenna radiation patterns, and the number of transmitting antennas to study the ACIR gap between [1] and [2].

  - DL ACIR

According to the simulation results in [1], the bandwidth is little relevant when decides ACIR requirements. Thus, 100MHz bandwidth was considered only for this contribution.

The DL ACIR results with two 3dB beam width conditions are summarized in Table 1. The other simulation parameters are referred to [3].

Table 1 : 5% throughput loss DL ACIR results with antenna radiation patterns

| 3dB beam width 5% TP loss | 90° [2] | 130° [3] |
| --- | --- | --- |
| 60GHz AVE | 19.0 | 21.2 |
| 60GHz EDGE | 21.4 | 22.2 |
| 70GHz AVE | 20.5 | 19.6 |
| 70GHz EDGE | 23.7 | 23.9 |

The DL ACIR results with two transmission power conditions are summarized in Table 2. The other simulation parameters are referred to [3].

Table 2 : 5% throughput loss DL ACIR results with transmission power

| Transmission power | 17 dBm[3] | 23 dBm[2] |
| --- | --- | --- |
| 60GHz AVE | 21.2 | 19.2 |
| 60GHz EDGE | 22.2 | 22.2 |
| 70GHz AVE | 20.5 | 19.7 |
| 70GHz EDGE | 23.7 | 21.3 |

The DL ACIR results with the number of transmitting antennas are summarized in Table 3. The other simulation parameters are referred to [3].

Table 3 : 5% throughput loss DL ACIR results with the number of transmitting antennas

| The number of transmitting antennas | 32[3] | 128[2] |
| --- | --- | --- |
| 60GHz AVE | 21.2 | 17.0 |
| 60GHz EDGE | 22.2 | 18.6 |
| 70GHz AVE | 20.5 | 16.7 |
| 70GHz EDGE | 23.7 | 19.9 |

- Conclusion

Based on the simulation results, the following observations are obtained.

Observation 1: Antenna elements radiation pattern and transmission power have less influence than the number of transmitting antennas in determining ACIR requirement.

Observation 2: ACIR requirements become stringent as the number of transmitting antennas decreases.

Proposal 1: It is the implementing domain that determines the number of transmitting antennas. However, it is necessary to establish a minimum number of transmitting antennas to determine the DL ACIR requirements.

Reference

[1] R4-2120001, “WF on DL/UL ACIR and BS/UE ACLR/ACS for FR2-2”, CATT, vivo, Nokia, Qualcomm, KTL, ZTE

[2] TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”

[3] R4-2114993, “WF on co-existence simulation for NR_ext_to_71GHz”, Qualcomm