3GPP TSG-RAN WG4 Meeting #101bis-e	R4-2200590

Electronic Meeting, 17st - 25th Jan, 2022

Source:	ZTE Corporation

Title:	Discussion on RRM aspect of UL gap for Tx power management

Agenda Item:	6.4.6.3

Document for:	Approval

- Introduction

In 101 e-meeting, one WF [1] was approved, the agreements about RRM aspect are summarized as follows.

- Tx power management:RRM aspect Agreements: - In R17, the following 4 gap configurations are introduced - The same requirements should be specified for all gap configurations - UGL indicates the number of consecutive static UL slots configured as UL gap per UGRP UGL [ms]; UGRP [ms]; UGL/UGRP. ULGP #0; 1.0; 20; 5%. ULGP #1; 1.0; 40; 2.5%. ULGP #2; 0.5; 160; ~0.31%. ULGP #3; 0.125; 5; 2.5%. Agreements: - UL gaps are configured by the network using RRC configuration. - UL gaps are deconfigured by the network using RRC configuration. - Related to activation and deactivation of UL gaps: - The UL gaps can be activated when configured (using RRC signalling). - FFS: The UL gaps can additionally and optionally be activated and deactivated using MAC command after UL gap is configured by RRC Signaling - The UL gaps are deactivated when deconfigured (using RRC signalling). Agreements: - With the agreement that UE can explicitly indicate to NW on “need for UL gap” and “no need for UL gap”, RAN4 will leave the detailed signaling design to RAN2 Agreements: - Once the UL gap is configured, and activated, the UE will perform BPS sensing only in the static UL slot, i.e. no DL slot or special slot will be used as UL gap. For example, in case of ULGP#1, DDDSU configuration with 120KHz SCS, the following figure depicts the available slots for UL gap. - UE is not expected to be scheduled with UL transmission during the gap. Details FFS, e.g., RACH Agreements: - No interruption across FR. FR2 UL gap does not cause FR1 interruption. Agreements: - UL gap slot is static UL slot. UL gap slot applies same TA as other UL slots. Agreements: - No conflict b/w TDD config and UL gap. UL gap is always configured in static UL slot Agreements: - FFS the impact on measurement gap configuration and requirement due to UL gap.

According to the conclusions achieved during 101 e-meeting, for the Tx power management use case, we provide further analysis about the remaining RF aspect in another document. In this document, based on the progress during 101 e-meeting, some further RRM aspect discussion are provided, focus on the following issues.

- Activation/deactivation of UL gaps

- UE behavior during UL gaps

- The impact on measurement gap configuration and requirement due to UL gaps

- The range of impact on DL/UL transmission due to UL gaps

- Discussion

2.1 Activation/deactivation of UL gaps

For the issue of activation/deactivation of UL gaps, it has been agreed that the UL gaps can be activated when configured and deactivated when deconfigured by NW RRC signalling during 101 meeting. For another activation/deactivation mechanism, i.e. be activated and deactivated using MAC command after configured by RRC Signaling, it is still FFS.

From our perspective, we suggest the two mechanisms can not coexist, down-selection between the two mechanisms is required. Either only supporting activation/deactivation by RRC signalling when configuring or only supporting activation/deactivation by MAC CE after configuring. If the two mechanisms are both supported, the UE is confused when receiving an UL gap configuration signaling, whether the configured UL gap is activated by this signaling directly or should wait for the activation MAC CE? If the two methods are supported, additional activation/deactivation signaling should be added except for the configuration signaling itself.

Considering the mechanism of activation/deactivation by RRC signalling when configuring has been approved, then there is no need to support the MAC CE activation/deactivation mechanism. So, we suggest to remove the MAC CE activation/deactivation mechanism.

Proposal 1: The RRC signaling activation/deactivation and MAC CE activation/deactivation should not coexist. Since the former has been approved, we suggest remove the latter.

2.2 UE behavior during UL gaps

It has been agreed that the configuration of UL gaps should not conflict with the TDD frame structure during 101 meeting. So the UL gaps can only be configured in ‘UL’ slot. Further more, the UE is not expected to be scheduled with UL transmission during the UL gaps. But whether UE can transmit PRACH during UL gaps, it is still FFS.

In our view, on one hand, in order to guarantee UE can do BPS during UL gaps, UE should not transmit any UL transmission including dynamic scheduled by NW or semi-static configured by NW during UL gaps. The rule is applicable to all UL transmission, of course including PRACH. On the other hand, from the perspective of NW behavior, for type 1 UL gap, NW can assign those resources within UL gap to other UEs for UL transmission, and for the use case of BPS, only type 1 UL gap can be applied as specified during 98 meeting. In order to avoid the interference between the PRACH transmission and other UE’s scheduled UL transmission during the UL gaps. Consequently, UE is not expected to transmit PRACH during UL gaps.

Proposal 2: In order to guarantee the BPS operation and avoid the interference with other UEs, UE is not expected to transmit PRACH during UL gaps. So, all types of UL transmission is not expected during the UL gaps.

2.3 The impact on measurement gap configuration and requirement due to UL gaps

This issue is still FFS during 101 meeting. So the discussion should continue to address this issue. Firstly, the configuration of UL gap and measurement gap should be independent. UL gap can only be configured in the ‘UL’ slot, however the measurement gap can be configured in each type of slot, of course possible in ‘UL’ slot. No matter with UL gap configuration or without UL gap configuration, the configuration of measurement gap should not be impacted.

If the collision between UL gap and measurement gap occurs, it is up to UE implementation whether executing both BPS and RRM measurement, or choose one of them. We believe which rely on UE capability and the BPS realization. So no need to specify. But it should be noted that the legacy RRM requirements related with measurement gap such as measurement period should not be influenced or scaled. The introduce of FR2 UL gap should be highly attractive and cannot affect the existing functions or requirements.

Proposal 3: The configuration of UL gaps should not affect the configuration of measurement gap and the corresponding requirements.

2.4 The range of impact on DL/UL transmission due to UL gaps

As mentioned in previous section, during UL gaps, all UL transmissions are not expected by the UE. Such limitation should effect only one serving cell, multiple serving cells in the same band, multiple serving cells in the same FR or each serving cell the UE operating, which should be determined. Only addressing this question, the UL gap can be applicable not only in single cell case, but also in CA or DC case.

Since UL gap is used for BPS detection in FR2, and UE would emit radar signal for the sake of proximity detection, a general understanding is that only the UL scheduling of serving cells of FR2 are restricted. The serving cells of FR1 would not be effected by UL gaps. So if not introduce any additional UE capability, the default range of impact on UL scheduling constraint is all serving cells of FR2.

However since the BPS detection really rely on UE implementation, if other company concern the general understanding beyond UE capability in the field, we are also open for the introduction of additional UE capability which can indicate the exact UE capability for the range of impacted serving cells.

Proposal 4: A general understanding is that all the serving cells in FR2 are impacted by UL gaps. If it is verified that any additional UE capability is needed, we are fine for the introduction of new UE capability.

- Conclusion

In this contribution, we have the following proposals for RRM impact of UL gap Tx power management use case:

Proposal 1: The RRC signaling activation/deactivation and MAC CE activation/deactivation should not coexist. Since the former has been approved, we suggest remove the latter.

Proposal 2: In order to guarantee the BPS operation and avoid the interference with other UEs, UE is not expected to transmit PRACH during UL gaps. So, all types of UL transmission is not expected during the UL gaps.

Proposal 3: The configuration of UL gaps should not affect the configuration of measurement gap and the corresponding requirements.

Proposal 4: A general understanding is that all the serving cells in FR2 are impacted by UL gaps. If it is verified that any additional UE capability is needed, we are fine for the introduction of new UE capability.

- Reference

R4-2119962, “WF on FR2 enhancement part 2: UL gaps”, Apple.