3GPP TSG-RAN WG4 Meeting #101-bis-e                                                     R4-2200624

Electronic Meeting, January 17-25, 2022

Source:	ZTE Corporation

Title:	TP for TR 37.717-31-11: DC_1A-3A-38A_n78A

Agenda Item:	5.16.1

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC_1A-3A-38A_n78A for TR37.717-31-11[1].

- Reference

- TR 37.717-31-11 Rel-17 DC combinations LTE 3DL and one NR band v0.7.0

- Text Proposal

----- Start of TP -----

## 5.1.x	DC_1-3-38_n78

### 5.1.x.1	Configurations for EN-DC

Table 5.1.x.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-38A_n78A | DC_1A_n78A DC_3A_n78A |

### 5.1.x.2	∆TIB and ∆RIB values

For DC_1-3-38_n78, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.x.2-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-38_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 38 | 0.5 |
|  | n78 | 0.8 |

Table 5.1.x.2-2: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-38_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 38 | 0.4 |
|  | n78 | 0.5 |

### 5.1.x.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

----- End of TP -----