3GPP TSG-RAN4 WG4 Meeting #101-bis-e	R4-2200654

# Electronic meeting, January 17 – January 25, 2022

Source: 	vivo

Title: 	Further discussion on RRM impacts for extending NR operation to 71GHz

Agenda Item:	6.16.7.1

Document for:	Discussion

Introduction

In RAN4 #101e, there were discussions on RRM impacts due to introducing of extending current NR operation to 71GHz. The agreements/conclusions are captured in [1].

Rx beam sweeping scaling factor - Wait for more agreements from RF session before deciding the Rx beam scaling factor for FR2-2. Scheduling restrictions - Scheduling restrictions due to beam switching time to continue waiting for the conclusion about beam switching time from RF session - Assumptions on deriveSSB-IndexFromCell - Do not change cell phase synchronization accuracy - For 120kHz SCS - deriveSSB-IndexFromCell is always enabled - For 480kHz and 960kHz SCS - Option 1: deriveSSB-IndexFromCell is always enabled - Option 1A: revisit UE assumptions on the maximum timing difference between the reference cell and target cell from 2 SSB symbols to a larger value - Option 1B: introduce [1-3] symbols scheduling restriction before and/or after SSB transmission - Option 2: deriveSSB-IndexFromCell may be either enabled or not enabled and up to network configuration - FFS whether to have different approaches for licensed/unlicensed bands Measurement procedures - Measurement capability - As a starting point, reuse FR2-1 measurement capability of number of cells and beams for FR2-2. - FFS - Cell detection, PBCH detection for SSB index acquisition, SSB measurements - Channel model to be considered for FR2-2 - Option 1: Channel model from RAN1 - Option 2: FR2-1 channel models - Further discuss whether additional samples are needed

In this contribution, we further discuss on RRM impacts for extending NR operation to 71GHz.

Discussion

Rx beam sweeping scaling factor

Whether the new scaling factor is needed for FR2-2 depends on the conclusion of RF group. A WF [2] from RF session related to antenna module physical dimension was duplicated as below:

Agreement:

- Commercial FR2-1 antenna module physical dimension can be treated as the feasible FR2-2 antenna module dimension.

  - Commercial FR2-1 antenna module is equipped with 1x4 or 2x2 antenna elements

- Address the concerns related to the following identified factors into the next meeting

In addition, we notice there is still no clear conclusion on antenna element numbers for 52.6GHz-71GHz from RF group. Therefore, it is recommended to continue waiting for the RF outcome before deciding the Rx beam scaling factor for FR2-2.

Proposal 1: Rx beam scaling factor for FR2-2 need to continue waiting for the RF outcome.

Scheduling restrictions

The scheduling restriction was also discussed in the last meeting. All companies agreed that scheduling restrictions due to beam switching time to continue waiting for the conclusion about beam switching time from RF session. A WF [3] from RF session related to beam switching time was duplicated as below:

- Proposals: - Option 1: 200 nsec for all SCS (Huawei, vivo, Apple, MediaTek, Qualcomm, OPPO) - Option 2: 50 nsec for all SCS (Nokia, Ericsson) - Option 3: 200 ns is too long, this requires more discussion (Ericsson) - Recommended WF by Moderator in the first round: - Option 1 - Tentative agreement based on majority view: - Option 1 (200 nsec for all SCS) NOTE: no agreement was reached on this topic

It is observed from the above table that there is still no clear conclusion on UE beam switching time. Therefore, it is feasible to continue waiting for the RF conclusion on beam switching time for scheduling restrictions.

Proposal 2: Scheduling restrictions need to continue waiting for the conclusion about beam switching time from RF session.

Another key question is regarding assumptions on deriveSSB-IndexFromCell. In the existing requirements, the frame boundary alignment across cells on the same frequency is within a tolerance not worse than min (2 SSB symbols, 1 PDSCH symbol). For 960kHz, the value is indeed less than the cell phase synchronization accuracy of 3us. We review the past discussion and notice the deriveSSB-IndexFromCell tolerance is related to MRTD. An agreement in the RAN4 #87 was duplicated as below:

Agreements use [MRTD] the tolerance of useServingCellTimingForSync and revise scheduling availability requirements if necessary Outcome of discussion will be captured in 38.133, and LS informing RAN2 that this has been done will be sent Frame boundary is aligned and Same SFN number is used in all cells on the same carrier is assumed if useServingCellTimingForSync=TURE for the carrier

We copied the agreement related to SSB pattern for 480kHz and 960kHz in the RAN1 #106-e as below:

Agreement: - For 480kHz and 960kHz sub-carrier spacing, first symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

As shown in Figure 1, it is observed that it is possible to distinguish the SSB index if the timing difference between reference cell and target cell can be guaranteed less than 3 symbols. That is, if the timing difference across cells on the same frequency is less than 3.375us (3*1.125) or 6.75us (3*2.25us) for 960kHz and 480kHz respectively, SSB index detection process may be skipped. As mentioned in our other paper, considering the cell converge is smaller for FR2-2 compared with FR2-1, the propagation delay difference of FR2-2 should be 3.33us when the coverage is 1km. And add to the cell phase synchronization accuracy of 3us, the timing difference between cells for FR2-2 can be guaranteed within 6.33us.

Figure 1

Observation 1: It is possible to distinguish the SSB index if the timing difference between reference cell and target cell can be guaranteed less than 3 symbols.

Observation 2: When the coverage is 1km for FR2-2, the timing difference between cells for FR2-2 can be guaranteed within 6.33us.

Therefore, for 480kHz, it is feasible to skip SSB index detection for FR2-2. That is, deriveSSB-IndexFromCell should be always enabled for 480kHz.  And the existing deriveSSB-IndexFromCell tolerance requirement (e.g., min (2SSB symbols, 1 PDSCH symbol) should be revised to 3 SSB symbols for 480kHz.

For 960kHz, due to the shorter slot length, the timing difference of 6.33us would up to 5 symbols. It may be not feasible to skip SSB index detection for 960kHz. In our understanding, this may be up to network configuration. According to the information of network configuration, deriveSSB-IndexFromCell may be either enabled or not enabled. In addition, we understand if deriveSSB-IndexFromCell is not enabled, the requirement of SSB index detection for intra-frequency measurement for FR2-2 need to be specified.

Proposal 3: DeriveSSB-IndexFromCell should be always enabled for 480kHz of FR2-2. And the deriveSSB-IndexFromCell tolerance requirement should be revised to 3 SSB symbols.

Proposal 4: DeriveSSB-IndexFromCell may be either enabled or not enabled which is up to network configuration for 960kHz of FR2-2. And the deriveSSB-IndexFromCell tolerance requirement should be revised to 3 SSB symbols.

Proposal 5: If deriveSSB-IndexFromCell is not enabled, the requirement of SSB index detection for intra-frequency measurement for FR2-2 need to be specified.

RRM requirements impact due to higher SCS

In the last meeting, we provided the preliminary simulation results for cell detection, PBCH detection and SSB measurement. Most companies also agreed to further clarify the issue. In this paper, we give further analysis on the impact on intra-frequency measurement and inter-frequency measurement due to higher SCS of 480kHz and 960kHz. Compared with FR2-1, symbol length would become shorter for new SCS. If reusing the channel model for legacy FR2, the delay spread may occupy much percentage of CP. And this may cause imperfection on the channel estimation and affect demodulation performance. We copy the delay profiles on TDL-A 30 and TDL-C 60 for FR2-1 from [38.104].

Table B.2.1.2-1: Delay profiles for NR channel models Model; Number of channel taps; Delay spread (r.m.s.); Maximum excess tap delay (span); Delay resolution. TDLA30; 12; 30 ns; 290 ns; 5 ns. TDLC60; 12; 60 ns; 520 ns; 5 ns. Table B.2.1.2-2 TDLA30 (DS = 30 ns) Tap #; Delay [ns]; Power [dB]; Fading distribution. 1; 0; -15.5; Rayleigh. 2; 10; 0; Rayleigh. 3; 15; -5.1; Rayleigh. 4; 20; -5.1; Rayleigh. 5; 25; -9.6; Rayleigh. 6; 50; -8.2; Rayleigh. 7; 65; -13.1; Rayleigh. 8; 75; -11.5; Rayleigh. 9; 105; -11.0; Rayleigh. 10; 135; -16.2; Rayleigh. 11; 150; -16.6; Rayleigh. 12; 290; -26.2; Rayleigh. Table B.2.1.2-3 TDLC60 (DS = 60 ns) Tap #; Delay [ns]; Power [dB]; Fading distribution. 1; 0; -7.8; Rayleigh. 2; 15; -0.3; Rayleigh. 3; 40; 0; Rayleigh. 4; 50; -8.9; Rayleigh. 5; 55; -14.5; Rayleigh. 6; 75; -8.5; Rayleigh. 7; 80; -10.2; Rayleigh. 8; 130; -12.1; Rayleigh. 9; 210; -13.9; Rayleigh. 10; 300; -15.2; Rayleigh. 11; 360; -16.9; Rayleigh. 12; 520; -19.4; Rayleigh.

It can be seen that the maximum excess tap delay is around 290ns and 520ns for TDL-A30 and TDL-C60 respectively which is greater than CP length of 480kHz and 960kHz. In order to further analyse the above issue, we perform simulation according to difference SCS and the simulation assumption for PBCH detection is provided in Appendix 4.2. The result is shown in Table 1.

Table 1: PBCH detection delay

| Case |  | SCS (kHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 120 | 240 | 480 | 960 |
| Propagation Condition | SNR(dB) | Number of acquisition attempts | Number of acquisition attempts | Number of acquisition attempts | Number of acquisition attempts |
| TDL-A 30ns | -8 | 6 | 6 | 7 | 16 |
|  | -7 | 4 | 5 | 5 | 10 |
|  | -6 | 3 | 3 | 3 | 5 |
|  | -5 | 2 | 3 | 2 | 4 |
|  | -4 | 2 | 2 | 2 | 3 |
|  | -3 | 2 | 2 | 2 | 2 |
| TDL-C 60ns | -8 | 5 | 6 | 10 | 16 |
|  | -7 | 3 | 4 | 6 | 12 |
|  | -6 | 2 | 3 | 3 | 6 |
|  | -5 | 2 | 2 | 2 | 5 |
|  | -4 | 2 | 1 | 2 | 3 |
|  | -3 | 1 | 1 | 2 | 3 |

It is observed that as the SCS increases, the PBCH detection delay would also increase. Cell detection and SSB measurement may also follow the same rules. Therefore, we propose it may be necessary to perform the simulation to evaluate the requirements for new SCS of 480kHz and 960kHz.

As for channel model, in our understanding, considering the cell coverage is smaller for FR2-2 compared with FR2-1, the delay spread is also reduced for 52.6-71GHz. Therefore, it may be reasonable to use the channel model of [38.808] from RAN1, e.g., AWGN and TDL-A (5/10/20ns delay spread).

Proposal 6: The channel model from RAN1 should be used for FR2-2.

At last, we provide the simulation results of cell detection, PBCH detection for SSB index acquisition and SSB measurement based on the simulation assumptions provided in Appendix 4.1, 4.2 and 4.3 separately.

Cell detection

Our preliminary simulation results for cell detection are provided in Table 2 and Table 3 for case1 and case2, which can be found in the simulation assumptions for cell detection in Appendix, respectively.

Table 2: PSS/SSS detection delay for case1

| SCS | Propagation Condition | 90 percentiles of the PSS/SSS acquisition time |
| --- | --- | --- |
| 480kHz | AWGN | 1 |
|  | TDL-A-5 | 2 |
|  | TDL-A-10 | 2 |
|  | TDL-A-20 | 2 |
|  | TDL-A-30 | 3 |
|  | TDL-C-60 | 3 |
| 960kHz | AWGN | 1 |
|  | TDL-A-5 | 2 |
|  | TDL-A-10 | 2 |
|  | TDL-A-20 | 3 |
|  | TDL-A-30 | 3 |
|  | TDL-C-60 | 6 |

Table 3: PSS/SSS detection delay for case2

| SCS | Propagation Condition | 90 percentiles of the PSS/SSS acquisition time |
| --- | --- | --- |
| 480kHz | AWGN | 1 |
|  | TDL-A-5 | 2 |
|  | TDL-A-10 | 2 |
|  | TDL-A-20 | 2 |
|  | TDL-A-30 | 2 |
|  | TDL-C-60 | 3 |
| 960kHz | AWGN | 1 |
|  | TDL-A-5 | 2 |
|  | TDL-A-10 | 2 |
|  | TDL-A-20 | 3 |
|  | TDL-A-30 | 3 |
|  | TDL-C-60 | 6 |

From the simulation results, it can be seen that at least 6 samples are required for PSS/SSS detection for the 52.6GHz~71GHz under TDL-C 60 channel model. In existing intra-frequency measurement requirements and inter-frequency measurement requirements, 2~3 samples are assumed without considering AGC. If requirements for cell detection are still to be defined under same channel models used for legacy FR2, then cell detection delay should be extended.

However, if channel models in TS 38.808 are used to defined requirements, then cell detection requirements for legacy FR2 range can be reused. 2 samples for 480kHz SCS and 3 samples for 960kHz SCS can be assumed without considering AGC. In our view, with such higher SCS, TDL-C may not be appropriate for defining requirements.

Proposal 7a: Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 can be reused for FR2-2 if using the channel model from RAN1.

Proposal 7b: Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 is extended by 3 samples for FR2-2 if using the channel model of FR2-1.

PBCH detection for SSB index acquisition

Preliminary simulation results for SSB index acquisition are provided in Table 4.

Table 4: PBCH detection delay for FR2-2

| Case |  | SCS(kHz) |  |
| --- | --- | --- | --- |
|  |  | 480 | 960 |
| Propagation Condition | SNR (dB) | Number of acquisition attempts | Number of acquisition attempts |
| AWGN | -8 | 1 | 1 |
|  | -7 | 1 | 1 |
|  | -6 | 1 | 1 |
|  | -5 | 1 | 1 |
|  | -4 | 1 | 1 |
|  | -3 | 1 | 1 |
| TDL-A5 | -8 | 6 | 7 |
|  | -7 | 4 | 6 |
|  | -6 | 3 | 3 |
|  | -5 | 2 | 2 |
|  | -4 | 2 | 2 |
|  | -3 | 2 | 2 |
| TDL-A10 | -8 | 6 | 7 |
|  | -7 | 5 | 4 |
|  | -6 | 3 | 4 |
|  | -5 | 2 | 2 |
|  | -4 | 2 | 2 |
|  | -3 | 2 | 2 |
| TDL-A20 | -8 | 6 | 10 |
|  | -7 | 4 | 6 |
|  | -6 | 3 | 4 |
|  | -5 | 2 | 3 |
|  | -4 | 2 | 3 |
|  | -3 | 2 | 2 |
| TDL-A30 | -8 | 7 | 16 |
|  | -7 | 5 | 10 |
|  | -6 | 3 | 5 |
|  | -5 | 2 | 4 |
|  | -4 | 2 | 3 |
|  | -3 | 2 | 2 |
| TDL-C60 | -8 | 10 | 16 |
|  | -7 | 6 | 12 |
|  | -6 | 3 | 6 |
|  | -5 | 2 | 5 |
|  | -4 | 2 | 3 |
|  | -3 | 2 | 3 |

It can be observed that at least 4 samples and 6 samples are required for SSB index acquisition for channel model from RAN1 and FR2-1 channel model respectively for 960kHz. In the existing inter-frequency measurement requirements for FR2-1, 3 samples are assumed for SSB index acquisition. The requirements cannot be met for 960kHz SCS case. Therefore, the inter frequency SSB index acquisition delay should be extended no matter if the channel model is from RAN1 or FR2-1.

Proposal 8a: SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 1 sample compared with FR2-1 if using the channel model from RAN1.

Proposal 8b: SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 3 samples compared with FR2-1 if using the channel model of FR2-1.

SSB measurements

Preliminary simulation results for SSB measurements are provided in Table 5 and in section 4.4 in Appendix.

Table 5: SSB measurement results

| Channel | Samples | Delta RSRP [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS =480kHz |  |  | SCS=960kHz |  |  |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -0.42 | 0.68 | 1.65 | -0.42 | 0.69 | 1.64 |
|  | 2 | -0.23 | 0.58 | 1.29 | -0.22 | 0.57 | 1.29 |
|  | 3 | -0.12 | 0.52 | 1.13 | -0.11 | 0.53 | 1.11 |
|  | 5 | -0.03 | 0.47 | 0.95 | -0.02 | 0.47 | 0.94 |
| TDL-A | 1 | -2.39 | 0.03 | 2.08 | -2.50 | 0.05 | 2.08 |
|  | 2 | -1.84 | 0.00 | 1.59 | -1.87 | 0.01 | 1.61 |
|  | 3 | -1.57 | -0.02 | 1.34 | -1.61 | -0.02 | 1.31 |
|  | 5 | -1.26 | -0.03 | 1.06 | -1.32 | -0.04 | 1.07 |
| TDL-C | 1 | -2.74 | 0.06 | 2.19 | -2.67 | 0.10 | 2.26 |
|  | 2 | -2.08 | 0.03 | 1.72 | -2.00 | 0.04 | 1.77 |
|  | 3 | -1.81 | -0.01 | 1.47 | -1.73 | 0.02 | 1.52 |
|  | 5 | -1.54 | -0.03 | 1.21 | -1.45 | -0.01 | 1.26 |

It can be seen from the simulation results that 3 samples are enough for SSB measurements for both 480kHz SCS and 960kHz SCS SSBs for FR2-2 under all simulated channel models.

In existing intra-frequency and inter-frequency measurement requirements for FR2-1, 3 samples without considering implementation margin are assumed for SSB measurements. Therefore, the existing requirements are also applicable for FR2-2.

Proposal 9: SSB measurement period requirements for FR2-1 can be reused for FR2-2 for both intra-frequency inter-frequency measurements no matter if the channel model is from RAN1 or FR2-1.

Applicability of requirements for FR2

In the last meeting, the question how to adapt the existing terminology when corresponding requirements are not defined for FR2-2 (e.g., inter band CA scenarios in FR2). We understand it may be feasible to add the applicability of requirements of FR2-2 in 38.133. That is, specify that the requirements for SFTD measurement in clause 9.2.5.4, 9.3.8, 10.1.21 and CGI identification in clause 9.11 are only applicable for FR2-1 and the intra-band CA for FR2 is only applicable for FR2-1.

Proposal 10: Add the applicability of requirements of FR2 in 38.133 for corresponding requirements which are not defined for FR2-2.

Conclusion

Observation 1: It is possible to distinguish the SSB index if the timing difference between reference cell and target cell can be guaranteed less than 3 symbols.

Observation 2: When the coverage is 1km for FR2-2, the timing difference between cells for FR2-2 can be guaranteed within 6.33us.

Proposal 1: Rx beam scaling factor for FR2-2 need to continue waiting for the RF outcome.

Proposal 2: Scheduling restrictions need to continue waiting for the conclusion about beam switching time from RF session.

Proposal 3: DeriveSSB-IndexFromCell should be always enabled for 480kHz of FR2-2. And the deriveSSB-IndexFromCell tolerance requirement should be revised to 3 SSB symbols.

Proposal 4: DeriveSSB-IndexFromCell may be either enabled or not enabled which is up to network configuration for 960kHz of FR2-2. And the deriveSSB-IndexFromCell tolerance requirement should be revised to 3 SSB symbols.

Proposal 5: If deriveSSB-IndexFromCell is not enabled, the requirement of SSB index detection for intra-frequency measurement for FR2-2 need to be specified.

Proposal 6: The channel model from RAN1 should be used for FR2-2.

Proposal 7a: Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 can be reused for FR2-2 if using the channel model from RAN1.

Proposal 7b: Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 is extended by 3 samples for FR2-2 if using the channel model of FR2-1.

Proposal 8a: SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 1 sample compared with FR2-1 if using the channel model from RAN1.

Proposal 8b: SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 3 samples compared with FR2-1 if using the channel model of FR2-1.

Proposal 9: SSB measurement period requirements for FR2-1 can be reused for FR2-2 for both intra-frequency inter-frequency measurements no matter if the channel model is from RAN1 or FR2-1.

Proposal 10: Add the applicability of requirements of FR2 in 38.133 for corresponding requirements which are not defined for FR2-2.

Appendix

The simulation assumptions for the new band are provided in Appendix.

Simulation Assumptions for cell detection

Simulation assumptions for cell detection for the new band are based on R4-1708698 with changes highlighted.

Table 1: Cell-specific Parameters for PSS/SSS Detection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| Carrier frequency |  | 60GHz | The same frequency as for Cell 1 is used |
| Subcarrier spacing | KHz | 480kHz;960kHz | The same values as for Cell 1 are used |
| RB Utilization | % | 100 | 100 |
| Power boosting for PSS and SSS | dB | 0 | 0 |
| Power boosting for Data and Control | dB | 0 | 0 |
| Data Modulation | - | QPSK | QPSK |
| CP Length | - | Normal | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 | 1 |
| SS burst set periodicity | ms | 5 | 5 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | CP/2 |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Es/Noc | dB | 6.4 | 1.25; FSS: 0.25; -0.75 (SINR: -6dB; FFS: -8dB, -7dB) |
| Number of Tx antennas | - | 1 | 1 |
| PSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| SSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| Propagation Condition [38.101-4] Propagation Condition [38.808] | - | TDL-A (30ns delay spread)/TDL-C (60ns delay spread) AWGN/TDL-A (5/10/20ns delay spread) |  |
| UE speed |  | 3km/h |  |
| Noc Model | - | AWGN noise |  |

Table 2: UE-specific Parameters for Cell Detection

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 by the UE | Yes |
| False detect threshold | Required as in a real UE implementation |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence IDs. |
| Receive antennas | 2 |

Table 3: Example Cell ID Combinations that can be simulated

| Case # | Cell 1 (Interferer Cell) |  | Cell 2 (Desired Cell) |  |
| --- | --- | --- | --- | --- |
| 1 | psc1 | ssc1 | psc2 | ssc2 |
| 2 | psc1 | ssc1 | psc1 | ssc3 |

Table 4: Example PSS, SSS indices that can be simulated simulations

| PSS Indices |  | SSS Indices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Label | Code index | Label | Code index (m0, m1) | Cell ID | (,) |
| psc1 | 43 | ssc1 | (20,0) | 337 | (112, 1) |
| psc2 | 86 | ssc2 | (40, 0) | 674 | (224, 2) |
|  |  | ssc3 | (35,0) | 673 | (224, 1) |

Performance Metric

For NR-PSS/SSS acquisition time, the detection is claimed to be successful only when both the PSS and SSS are correctly detected and the timing offset is within half CP length.

Simulation Assumptions of PBCH detection

Simulation assumptions for PBCH detection for the new band are based on R4-1713437 with changes highlighted.

Case1: PBCH reading

Basic SI reading time = the number of SS-blocks required for 99% to successfully decode the PBCH.

Case2: PBCH-DMRS time index reading (Without decode PBCH data)

DMRS time index reading time = the number of SS-blocks required for 99% to successfully reading DMRS time index.

Table 1: Parameters for basic SI reading (PBCH acquisition)

| Parameter | Unit | Value |
| --- | --- | --- |
| Carrier frequency | GHz | 60GHz |
| Subcarrier spacing | KHz | 480kHz;960kHz |
| Number of Tx antennas | - | 1 Assuming only 1Tx port is used |
| Number of Rx antennas | - | 2 |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 |
| SS burst set periodicity | ms | 5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| RB Utilization | % | 100 |
| PBCH symbols within the SS block |  | According to the RAN1 agreements |
| Data and Control Power offset with respect to PSS and SSS | dB | 0 |
| PBCH power offset with respect to PBCH-DMRS | dB | 0 |
| PBCH-DMRS power offset with respect to PSS and SSS | dB | 0 |
| PSS and SSS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS RE positions within the PBCH resource | - | According to the RAN1 agreements |
| PBCH Channel coding |  | Polar code with 512 length and 24bit DCRC |
| PBCH Modulation | - | QPSK |
| PBCH Payload (including the CRC) | bits | 56bit (CRC 24bit) |
| PBCH SNR | dB | -10 : 0 dB, at least including -6dB,-8 dB |
| Propagation Condition [38.101-4] Propagation Condition [38.808] | - | TDL-A (30ns delay spread)/TDL-C (60ns delay spread) AWGN/TDL-A (5/10/20ns delay spread) |
| UE speed |  | 3km/h |
| Detection Method |  | Baseline: on one shot detection (No combination for different PBCHs,) FFS Soft- combining |

Performance Metric

The proposed simulation assumptions are used to derive the Basic SI reading time.

Basic SI reading time = the number of SS-blocks required for 99% to successfully decode the PBCH.

- 4.3 Simulation Assumptions for SSB measurements

Simulation assumptions for SSB measurements for the new band are based on R4-1706904 with changes highlighted.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 60GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes |
| DRX | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions |
| UE receive antennas | 1 |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 1 2 3 5; other number of samples may also be studied upon a need |
| - Subcarrier spacing | 480 kHz; 960 kHz |
| - Number of SS blocks per SS burst set | 1 |
| - SS burst periodicity | 5ms |
| - Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| - SS block BW | 127 subcarriers for SS, 288 subcarriers for PBCH |
| - Actual SS block transmissions | always transmitted |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS and NR-SSS sequences | - | To be indicated by companies | To be indicated by companies |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Relative Delay of 1st Path | µs | 0 | CP/2 |
| SNR | dB | 6 dB | 1 dB |
| Es/IoT (calculated from SNR) | dB | N/A 2.46dB | -5.97 dB |
| Propagation conditions [38.101-4] | - | AWGN/TDL-A (30ns delay spread)/TDL-B (100ns delay spread)/TDL-C (300ns delay spread) for 4GHz TDL-A (30ns delay spread)/TDL-C (60ns delay spread) for 30GHzNote |  |
| UE speed |  | 3km/h |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |

Performance Metrics

For different estimation time periods, the CDF curves and 90th %-iles are to be provided for (without RF impairment margin):

- Delta measurement   = (estimated measurement – ideal measurement) [dB];

- 4.4 Simulation results for SSB measurements

- References

[1] R4-2120316 WF on NR extension to 71 GHz RRM requirements (Part 1)

[2] R4-2120062 WF on 60 GHz UE RF requirements

[3] R4-2210063 WF on 60GHz UE timing