3GPP TSG-RAN4 WG4 Meeting #101-bis-e	R4-2200656

# Electronic meeting, January 17 – January 25, 2022

Source: 	vivo

Title: 	Further discussion on interruption for 52.6-71GHz

Agenda Item:	6.16.7.3

Document for:	Discussion

Introduction

In RAN4 #101e, Interruption requirements from 52.6GHz to 71GHz was discussed. A WF [1] related to the above was duplicated as below:

- For FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps: - Option 1: the interruption requirements to be defined as in table below Interruption length X1, Y1 and Y2 during measurements with autonomous gaps NR Slot length (ms) of victim cell; Interruption length X1 (slots); Interruption length Y1 (slots); Interruption length Y2 (slots). 0; 1; 6; 7; 6. 1; 0.5; 12; 13; 10. 2; 0.25; 24; 25; 19. 3; 0.125; 48; 49; 37. 5; 0.03125; 192; 193; 145. 6; 0.015625; 384; 385; 289. - Option 2: do not define interruption requirements - RAN4 to further discuss whether to extend to NR-DC case the agreements from RAN4 #100-e regarding interruptions at SCell addition/release for intra-band CA, interruptions at Scell activation/deactivation for intra-band CA, interruptions during measurements on deactivated SCC, interruptions due to Active BWP switching, interruptions at NR SRS carrier based switching.

In this contribution, we further discuss the interruption requirements when a UE is identifying CGI of a FR1 serving cell with autonomous gaps for FR2-2 DC with FR1 and whether to extend the agreements from RAN4 #100-e to NR-DC case.

Discussion

In the last meeting, de-prioritizing CGI identification and SFTD measurement requirements for Rel-17 FR2-2 was agreed. However, for the deployment scenario of FR2-2 DC with FR1, if the victim CC is in FR2-2 and the aggressor CC is in FR1, when UE is identifying CGI of the serving cell of FR1, this would still cause an interruption on FR2-2 serving cells. Therefore, Option 2 is not reasonable. So it is necessary to define the interruption requirements when a UE is identifying CGI of a FR1 serving cell with autonomous gaps for FR2-2 DC with FR1.

Proposal 1: The interruption requirements for higher SCS, e.g., 480kHz and 960kHz, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps for FR2-2 DC with FR1 need to be specified.

Another question is whether to extend the agreements from RAN4 #100-e to NR-DC case. In our understanding, the interruption requirements at SCell addition/release or activation/deactivation for intra-band CA, due to Active BWP switching and at NR SRS carrier based switching for Standalone NR Carrier Aggregation is applied to NR-DC case.

Proposal 2: The interruption requirements at SCell addition/release or activation/deactivation for intra-band CA, due to Active BWP switching and at NR SRS carrier based switching for Standalone NR Carrier Aggregation is applied to NR-DC case.

At last, we summarized the interruption requirements in NR-DC case.

Proposal 3:

For interruption requirements at transitions between active and non-active during DRX, update Table 8.2.4.2.6-1 as below:

Table 8.2.4.2.6-1: Interruption length X at transition between active and non-active during DRX

|  | NR Slot length (ms) | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |
| 5 | 0.03125 | 17 |  |
| 6 | 0.015625 | 33 |  |

For interruption requirements at NR SRS carrier based switching, update Table 8.2.4.2.9-1 and Table 8.2.4.2.9-2 as below:

Table 8.2.4.2.9-1: Interruption length X1 (slot)

|  | NR Slot length | SRS carrier | Interruption length X1 (slots) |  |
| --- | --- | --- | --- | --- |
|  | (ms) of victim cell | switching time (us)Note 1 | Sub carrier spacing for agressor cell (kHz) |  |
|  |  |  | 15 | 30 |
| 0 | 1 | ≤ 200 | 2 | 2 |
|  |  | 300, 500 | 2 | 2 |
|  |  | 900 | 2 | 2 |
| 1 | 0.5 | ≤ 200 | 3 | 2 |
|  |  | 300, 500 | 3 | 3 |
|  |  | 900 | 4 | 4 |
| 2 | 0.25 | ≤ 200 | 4 | 3 |
|  |  | 300, 500 | 5 | 4 |
|  |  | 900 | 7 | 6 |
| 3 | 0.125 | ≤ 200 | 7 | 5 |
|  |  | 300, 500 | 9 | 7 |
|  |  | 900 | 12 | 10 |
| 5 | 0.03125 | ≤ 200 | 22 | 15 |
|  |  | 300, 500 | 31 | 24 |
|  |  | 900 | 44 | 37 |
| 6 | 0.015625 | ≤ 200 | 42 | 28 |
|  |  | 300, 500 | 61 | 47 |
|  |  | 900 | 87 | 73 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |

Table 8.2.4.2.9-2: Interruption length X2 (slot)

|  | NR Slot | SRS carrier | Interruption length X2 (slots) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | length (ms) of victim cell | switching time (us) Note 1 | Sub carrier spacing for agressor cell (kHz) |  |  |  |
|  |  |  | 60 | 120 | 480 | 960 |
| 0 | 1 | ≤ 200 | 2 | 2 | 2 | 2 |
| 1 | 0.5 | ≤ 200 | 2 | 2 | 2 | 2 |
| 2 | 0.25 | ≤ 200 | 3 | 3 | 2 | 2 |
| 3 | 0.125 | ≤ 200 | 4 | 4 | 3 | 3 |
| 5 | 0.03125 | ≤ 200 | 11 | 10 | 8 | 8 |
| 6 | 0.015625 | ≤ 200 | 21 | 18 | 15 | 15 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |  |  |

For interruption requirements when identifying CGI of an NR cell with autonomous gaps, update Table 8.2.4.2.11-1 as below:

Table 8.2.4.2.11-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |
| 5 | 0.03125 | 192 | 193 | 145 |
| 6 | 0.015625 | 384 | 385 | 289 |

Conclusion

Proposal 1: The interruption requirements for higher SCS, e.g., 480kHz and 960kHz, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps for FR2-2 DC with FR1 need to be specified.

Proposal 2: The interruption requirements at SCell addition/release or activation/deactivation for intra-band CA, due to Active BWP switching and at NR SRS carrier based switching for Standalone NR Carrier Aggregation is applied to NR-DC case.

Proposal 3:

For interruption requirements at transitions between active and non-active during DRX, update Table 8.2.4.2.6-1 as below:

Table 8.2.4.2.6-1: Interruption length X at transition between active and non-active during DRX

|  | NR Slot length (ms) | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |
| 5 | 0.03125 | 17 |  |
| 6 | 0.015625 | 33 |  |

For interruption requirements at NR SRS carrier based switching, update Table 8.2.4.2.9-1 and Table 8.2.4.2.9-2 as below:

Table 8.2.4.2.9-1: Interruption length X1 (slot)

|  | NR Slot length | SRS carrier | Interruption length X1 (slots) |  |
| --- | --- | --- | --- | --- |
|  | (ms) of victim cell | switching time (us)Note 1 | Sub carrier spacing for agressor cell (kHz) |  |
|  |  |  | 15 | 30 |
| 0 | 1 | ≤ 200 | 2 | 2 |
|  |  | 300, 500 | 2 | 2 |
|  |  | 900 | 2 | 2 |
| 1 | 0.5 | ≤ 200 | 3 | 2 |
|  |  | 300, 500 | 3 | 3 |
|  |  | 900 | 4 | 4 |
| 2 | 0.25 | ≤ 200 | 4 | 3 |
|  |  | 300, 500 | 5 | 4 |
|  |  | 900 | 7 | 6 |
| 3 | 0.125 | ≤ 200 | 7 | 5 |
|  |  | 300, 500 | 9 | 7 |
|  |  | 900 | 12 | 10 |
| 5 | 0.03125 | ≤ 200 | 22 | 15 |
|  |  | 300, 500 | 31 | 24 |
|  |  | 900 | 44 | 37 |
| 6 | 0.015625 | ≤ 200 | 42 | 28 |
|  |  | 300, 500 | 61 | 47 |
|  |  | 900 | 87 | 73 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |

Table 8.2.4.2.9-2: Interruption length X2 (slot)

|  | NR Slot | SRS carrier | Interruption length X2 (slots) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | length (ms) of victim cell | switching time (us) Note 1 | Sub carrier spacing for agressor cell (kHz) |  |  |  |
|  |  |  | 60 | 120 | 480 | 960 |
| 0 | 1 | ≤ 200 | 2 | 2 | 2 | 2 |
| 1 | 0.5 | ≤ 200 | 2 | 2 | 2 | 2 |
| 2 | 0.25 | ≤ 200 | 3 | 3 | 2 | 2 |
| 3 | 0.125 | ≤ 200 | 4 | 4 | 3 | 3 |
| 5 | 0.03125 | ≤ 200 | 11 | 10 | 8 | 8 |
| 6 | 0.015625 | ≤ 200 | 21 | 18 | 15 | 15 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |  |  |

For interruption requirements when identifying CGI of an NR cell with autonomous gaps, update Table 8.2.4.2.11-1 as below:

Table 8.2.4.2.11-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |
| 5 | 0.03125 | 192 | 193 | 145 |
| 6 | 0.015625 | 384 | 385 | 289 |

- References

[1] R4-2120317 WF on NR extension to 71 GHz RRM requirements (Part 2), Intel