3GPP TSG-RAN WG4 Meeting#101-bis-e      	R4-2200723

E-meeting, 17 – 25 Jan, 2022

Title:	TP to TR 38.717-04-02: CA_n2-n5-n66-n77

Source:	Nokia, Verizon

Agenda item:	5.19.1

Document for:	Approval

# 1	Introduction

This contribution introduces CA_n2-n5-n66-n77 combinations into 38.717-04-02

********************************* Start of the TP ******************************************

### 5.x.3	CA_n2-n5-n66-n77

#### 5.x.3.1	Operating bands for CA

Table 5.x.3.1-1: Inter-band CA operating bands of CA_n41-n66-n71-n77

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n2-n5-n66-n77 | n2, n5, n66, n77 |

#### 5.x.3.1	Channel bandwidths per operating bands for CA

Table 5.x.3-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n5A-n66A-n77A | CA_n2A-n5A, CA_n2A-n66A, CA_n2A-n77A, CA_n5A-n66A, CA_n5A-n77A, CA_n66A-n77A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

#### 5.x.3.3		UE co-existence studies

The coexistence studies have been captured into the constituent fallback modes in TR 38.716-03-02, there is no additional harmonic and intermodulation impact for the additional band receiver.

#### 5.x.3.4	∆TIB,c and ∆RIB,c values

For CA_n2-n5-n66-n77, the TIB,c and RIB,c values are derived from DC_2-5-66_n77

Table 5.x.3-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n5-n66-n77 | n2 | 0.5 |
|  | n5 | 0.3 |
|  | n66 | 0.5 |
|  | n77 | 0.8 |

Table 5.x.3-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n2-n5-n66-n77 | n2 | 0.3 |
|  | n5 | 0 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

#### 5.x.3.5	REFSENS requirements

There is no need to specify additional MSD requirement for this UL CA configuration.

********************************* End of the TP ******************************************