3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200742

Electronic Meeting, January 17-25, 2022

Source:	ZTE Corporation

Title:	Other RRM requirements for FR2 inter-band DL CA requirements for CBM

Agenda item:	6.4.6.1.2

Document for:	Discussion

# 1	Introduction

In the last 101-e meeting, RRM requirements for FR2 inter-band CA for CBM have been discussed and a WF document was approved[1]. In this contribution ,we further provide our views on the open issues about SCell activation delay.

- Discussion

The following issues about SCell activation delay are open for discussion:

Issue 1-2-4: SCell activation delay Candidate options: Issue1-2-4-2: Can AGC settling time be reduced for UE owing to following AGC settling in Pcell/PSCell? - Option 1: Yes (Nokia, Huawei, Qualcomm) - If RTD < X (Nokia) - If this is about the number of required SSB bursts for coarse and fine AGC - Option 2: No (Mediatek, OPPO, ZTE, Vivo, Ericsson, Apple) - If RTD > X (Nokia) Issue1-2-4-3: Can TCI state indication and CSI reporting can be skipped for both semi-persistent and periodic CSI reporting? - Option 1: Yes (ZTE, Nokia, Ericsson) - Option 2: No (Mediatek, Qualcomm, Vivo, Apple) - Option 3: Only TCI state indication can be skipped (Huawei) - Option 4: consider defining requirements for following cases - SCell have same TCI with PCell/PSCell (TCI indication is not needed, hence uncertainty mac is not needed) - SCell have different TCI with PCell/PSCell (TCI indication is needed, hence uncertainty mac is needed)

According to RF session discussion and the agreement[2], RAN4 agrees to define CBM requirements in such manner that both single chain and multi chain architectures are possible. Thus it is possible for the UE to implement separate or same RF chain for CBM. If the same RF chain is applied for CBM UE, AGC setting time may be reduced. However, if separate RF chain is applied, AGC setting time cannot be reduced. It is suggested to consider the delay requirement to fulfill the worst case.

Proposal 1:AGC settling time cannot be reduced for SCell activation delay requirement.

UE performs CBM for PCell and target SCell in FR2. Since the beam characteristic for SCell could be regarded as the same as that of the PCell, the TCI state indication and CSI reporting could be reused for the case of periodic and semi-persistent CSI reporting.

Proposal 2: TCI state indication and CSI reporting can be skipped for both semi-persistent and periodic CSI reporting.

- Conclusion

Proposal 1:AGC settling time cannot be reduced for SCell activation delay requirement.

Proposal 2: TCI state indication and CSI reporting can be skipped for both semi-persistent and periodic CSI reporting.

References

- R4-2120284,WF on RRM requirements for FR2 Inter-band DL CA and UL CA，Nokia, Nokia Shanghai Bell

- R4-2107853,WF on CBM UE architecture,Qualcomm Incorporated