3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200837

Electronic Meeting, January 17-25, 2022

Source:	ZTE Corporation

Title:          PDSCH requirements under Bi-directional scenario

Agenda Item:	6.9.5.2.2

Document for:	Approval

- Introduction

In this contribution, the channel model and test issues for scenario-B are discussed.

- Discussion

In RAN4# 101-e meeting, the UE capability was discussed and the following 2 option about scenario-B were reached[1]:

Issue 2-1-3: UE capability Way forward: - Option 1: - Do not introduce the UE capability to differentiate requirement for Bi/Uni-directional. - Option 2: if found to be needed, define UE capability to support operation in HST FR2 bidirectional deployments with higher than 250 km/h speed. Define corresponding performance requirements with up to UE capability

The two options on the UE capability above are not mutually exclusive. For example, when option 1 is supported, option 2 can also be supported.

For option 1, signaling needs to be introduced to indicate whether the UE supports uni directional deployment or bi directional deployment. In our understanding, the HST deployment is relatively fixed. If the network is uni-directional deployment, the network can not provide bi-directional deployment even if the CPE reports capability of bi-directional. Also the network will adjust bore-sight for certain deployment. For example, if the RRH is determined as uni-directional deployment, the bore sight and beam direction will be adjusted for this type of deployment. Figure 1 and figure 2 show the bore-sight setup for uni-directional deployment and bi-directional deployment.

Figure 1 bore sight and beam direction for scenario-B uni-directional deployment

Figure 2 bore sight and beam direction for scenario-B bi-directional deployment

If the network is bi-directional deployed and the HST CPE supports uni-directional operation, the network may not have the opportunity to adjust the bore sight of the panel adapt to the deployment type reported by HST CPE.

On the other hand, UE may experience different deployment type when CPE moves from one place to another. The CPE can not require the network always provide the same deployment type reported by CPE. It’s reasonable for HST CPE to support both uni-directional deployment and bi-directional deployment by default.

Proposal 1: Not to define signaling distinguishing UE capability between uni-directional deployment and bi-directional deployment.

For option 2 we are open to introduce the capability for different speed. If different speed for practical deployment is confirmed the capability signaling can be supported as lower speed may require lower Doppler estimation and compensation.

Proposal 2: To support UE capability signaling differentiating different speed if the need of different speeds is confirmed.

- Conclusion

In this contribution, we provide the following proposals for HST_FR2 scenario-B:

Proposal 1: Not to define signaling distinguishing UE capability between uni-directional deployment and bi-directional deployment.

Proposal 2: To support UE capability signaling differentiating different speed if the need of different speeds is confirmed.

- Reference

R4-2120775 WF on general and UE demodulation requirement for FR2 HST