3GPP TSG-RAN WG4 Meeting # 101bis-e                	R4-2200873

Electronic Meeting, 17th – 25th January 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	LBT impacts on NR RRM requirements for extension to 71GHz

Agenda item: 	6.16.7.6

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to the operation between 52.6 GHz and 71 GHz when LBT is required for channel access.

Discussion

- FR2-2 operation with LBT

One important aspect of FR2-2 operation is the operation with LBT on unlicensed bands. Requirements with LBT already exist in the TS 38.133 specification, as they were introduced during the work of NR_unlic. Since NR_unlic was only specified for FR1, there is need to address requirements with LBT in FR2-2. Some of the requirements with LBT may be understood as band agnostic, and many of those need to be reviewed for FR2-2 since there is no baseline FR2 that can be reused.

The requirements with LBT defined in TS 38.133 are only applicable for FR1.

Before discussing and analysing which requirements need to be defined for operation with LBT in FR2-2, it is important to understand the differences between the channel access mechanism in unlicensed bands in FR1 and FR2-2.

At FR2-2, interference and channel access contention are expected to be less severe than in FR1, due to more directive transmissions and stronger signal attenuation. The use of beamforming in FR2-2 impacts interference as well as co-channel coexistence. Narrow beams, or large antenna gains, will limit the transmitted signal to a certain direction, minimizing the probability of collision, and consequently the LBT failure rate. This can be seen from the results below, firstly presented in [3], which compare the throughput in different channel access cases: without LBT, with omni-directional LBT and per beam LBT. The results on the left are for the median throughput, while in the right the cell-edge throughput is presented. Details and simulation assumptions can be found in [3].

Figure 1. FTP model 3 throughput median and 5th percentile for low, medium and high load

Fig 1 shows that there is no significant difference in the throughput between the modes with per-beam LBT, omni-directional LBT and no LBT for median throughput in FR2-2. Due to its narrower beamforming and channel propagation characteristics, the probability of collision in FR2-2 is not as significant as in FR1and the use of LBT reduces the throughput for uses at the cell-edge.

At 60 GHz narrow beams are expected, which, combined with a stronger signal attenuation, results in a LBT failure probability which is lower than what is experienced at 5 GHz band.

At 60GHz, the performance of directional LBT and omni-directional LBT is similar to the performance with no-LBT.

This is actually reflected in the channel access procedure defined by RAN1 in TS 37.213, which considers a random back-off drawn from a fixed sized window. According to the TR 38.808 [1], the baseline for the LBT procedure in NR_ext_to_71GHz is the adaptivity mechanism defined in EN 302 567 [2]. RAN1 has discussed the adaptivity mechanism and in the last RAN1 meeting, the CR 0022 to TS 37.213 has been approved, compiling the latest agreements from RAN1 regarding the channel access procedures for frequency range 2-2. The channel access procedures defined in TS 37.213 are roughly summarized in Table 1.

Table 1-Summary of DL channel access procedures in TS 37.213 for FR1 and FR2-2

| Channel access procedure | LBT type in FR1 | LBT type in FR2-2 |
| --- | --- | --- |
| Random back-off | Type 1. The back-off is drawn randomly from a contention window (CW) which is determined based on the Channel Access Priority (CAPC) and it is adjusted if a collision is detected. CW is within the range [CWmin, CWmax]. Min(CWmin) = 3 (CAPC1) and max(CWmax) = 1023 (CAPC 4). Defer duration = 16us. | Type 1. The back-off is drawn randomly from a fixed CW size equal to 3. Defer duration = 8 us. The channel occupancy time shall be at most 5ms. |
| Deterministic back-off | Type 2 - The node can transmit if the channel is idle during a sensing interval = 25us (shared channel occupancy or short transmission) - gNB transmissions follow a UE transmission, and the channel is idle for a duration of 16us | Type 2 Used for both DL and UL, the time duration for sensing before a transmission is deterministic. The node can transmit if the channel is idle for 8 us. |
| No LBT | Type 2 C) No sensing required, gNB transmissions follow a UE transmission, and the transmission duration is at most 584us. | Type 3) The node can transmit without sensing the channel. Transmissions of the discovery burst by the gNB are exempted from sensing if the total duration of those transmissions does not exceed 10ms over any 100ms period. Additionally, sensing is not required by the responding device within a channel occupancy initiated by another device using Type 1 LBT. |

As seen in Table 1, the LBT procedure in FR2-2 is much simpler than in FR1. No CW adjustment procedure is defined in FR2-2, and the deferral time is also reduced when compared to FR1. Additionally, transmissions of the discovery burst by the gNB, as well as transmission of the 1st message in random access procedure by the UE are exempted from sensing if the total duration of those transmissions does not exceed 10ms over any 100ms period.

The LBT procedure in FR2-2 is simplified when compared to FR1. There is no adaptation of the contention window based on collision detection, the deferral times are also reduced, and the discovery burst transmissions by the gNB might be exempt from LBT.

- RRM requirements with LBT

In Rel-16 there was an extensive discussion on the parameters used to account for LBT failures in the requirements defined in TS 38.133. By that time, RAN4 decided that the measurement periods for NR-U measurements would be extended by M samples, in case of IDLE and INACTIVE mode measurements, and by L samples, in case of CONNECTED mode measurements. The maximum number of samples, and the exact UE behaviours upon exceeding these samples, varies according to the specific measurements.

RAN4 has decided to extend measurement periods for most of the NR-U RRM requirements in FR1 in order to cope with the high probability of LBT failure due to interference in the 5GHz band.

Additionally, still in Rel-16, RAN1 has specified as part of the NR-U work a feature to make the SSBs more robust against CCA failures. When using this feature, each SSB index may be transmitted in different candidate positions, depending on whether there is an LBT failure or not. That means that if the gNB misses the opportunity to transmit SSB due to LBT failure, it may attempt transmitting it again in the next candidate position. From the RAN4 discussion, it was decided to use a working assumption where the UE is requirement to monitor 2 SSB candidate positions of the same SSB index during RRM measurements, and at least 1 SSB candidate position is assumed to be monitored for cell detection. When FR2-2 is considered, this feature is also defined by RAN1. In the last RAN1 meeting, the agreement below was reached, specifying that for FR2-2 SCS, there is support for 64 candidate SSB positions in a half-frame.

Agreement - Support DBTW with 480 and 960 kHz SCS. - For licensed and unlicensed operation, support 64 candidate SSB positions in a half frame - Working assumption: Use 2 bits for Q: - SubcarrierSpacingCommon - spare bit in MIB - Send LS to RAN2 for confirming the use of the spare bit in MIB - The use of 2 bits for Q can be revisited if RAN2 tells RAN1 that the spare bit cannot be used Agreement Confirm the following working assumptions: - (From #106-bis-e) Support DBTW for 120 kHz. - (From #106-e) For 120kHz SSB, the number of candidates SSBs in a half frame is 64. Agreement For SCS that support DBTW, UE derives the QCL relation between candidate SSBs by the value of , where is the candidate SSB index.

As in NR-U in FR1, RAN1 has specified the transmission of SSBs in different candidate positions in case of LBT failure during the transmission of the SSB burst in FR2-2.

Considering that the interference is less severe for unlicensed operation in FR2-2, when compared unlicensed operation in FR1, it is our view that if the same number of candidate SSB positions from FR1 could be assumed for RRM measurements and cell detection in FR2-2, there is no need to further relax the RRM requirements for unlicensed operation in FR2-2. Additionally, it is important to highlight that currently the FR2 measurement requirements (without LBT) are already scaled by factors to consider beam sweeping. Therefore, the requirements already allow for longer measurement and evaluation periods in FR2, even without considering LBT.

Most of the FR2 RRM requirements are scaled by a scaling factor N1, which allows for longer measurement and evaluation periods when compared to FR1 RRM requirements.

Use the FR2 RRM requirements as baseline for the RRM requirements with LBT in FR2-2, considering that the probability of failure of LBT in FR2-2 is nearly zero.

- TS 38.133 specification impact

In Table 1 below, we present an analysis of the requirements that need to be revisited to take into account CCA in FR2, considering the clauses changed for NR-U in FR1 as baseline.

RAN4 to consider Table 2 for the discussion of the impact of FR2-2 for requirements with CCA, taking into account the timeline of Rel-17 work

Table 2 Analysis of the impact of the operation on FR2-2 for requirements with CCA in TS 38.133

| Requirements and clauses |  | Impact for FR2-2 | Comments |
| --- | --- | --- | --- |
| Cell reselection | 4.2A.2.2 Measurement of serving cell | Yes | Table 4.2A.2.2-1 to be revisited. FR2 scaling factor N1 to be considered for requirements with CCA |
|  | 4.2A.2.3 Intra-frequency | Yes | Table 4.2A.2.3-1 to be revisited. FR2 scaling factor N1 to be considered for requirements with CCA |
|  | 4.2A.2.4 Inter-frequency | Yes | Table 4.2A.2.4-1 to be revisited. FR2 scaling factor N1 to be considered for requirements with CCA |
|  | 4.2A.2.5 Inter-RAT | No | This requirement is band agnostic. |
|  | 4.2A.2.6 Maximum interruption on paging reception when CCA is used in Target Cell | No | This requirement is band agnostic. |
| Handover | 6.1B Handover to target cell using CCA | yes | Discuss for which scenarios the requirements with CCA are to be defined for: - FR2-FR2 handover - FR2-FR1 handover - FR1-FR2 handover - E-UTRAN - NR FR2 handover: if possible, there is need to assess the impact on TS 36.133 as well |
| RRC Connection Mobility Control | 6.2.1A RRC re-establishment with CCA | Yes | Time to identify target NR cell to be defined for FR2 requirements with CCA Revisit Table 6.2.1.2.1-2: |
|  | 6.2.2A Random access when CCA is used | No | Existing requirements with CCA are also applicable for FR2-2 |
|  | 6.2.3.2.3 RRC release with redirection to NR carrier subject to CCA | Yes | Time to identify target NR cell needs to be defined for FR2 requirements with CCA Revisit Table 6.2.3.2.3-1: |
| UE timing | 7.1 UE transmit timing | FFS | Subject to agreements on UE transmit timing in FR2-2. FFS number of SSB candidate positions |
|  | 7.2 UE timer accuracy | No | Reuse FR2 or FR2-2 requirements |
|  | 7.3 Timing advance | No | Reuse FR2 or FR2-2 requirements |
|  | 7.5 UE maximum receive timing difference | No | Reuse FR2 or FR2-2 requirements |
|  | 7.6 UE maximum transmission timing difference | No | Reuse FR2 or FR2-2 requirements |
| Signalling characteristics | 8.3A SCell activation and deactivation delay | Yes | Clarify that the current requirements in this clause are applicable only to FR1 FR2-2 specific requirements to be defined, like the Tactivation_time_withCCA, |
|  | Interruption | FFS | FFS if impact on interruptions due to Active BWP switching Requirement |
|  | PSCell addition and release delay | No |  |
|  | 8.10A Active TCI state switching delay | Yes | Clarify that the current requirements in this clause are applicable only to FR1 Requirements with CCA should include FR2-2 specific timing TL1-RSRP for MAC-CE based TCI state switch delay |
|  | 8.6 Active BWP switching delay | No | Impact is generic and covered with FR2-2 requirements without CCA |
|  | PSCell change | No |  |
|  | Conditional PSCell change | No |  |
|  | 8.1A Radio link monitoring | Yes | The following needs to be defined for FR2-2 with CCA: - Maximum number of RLM resources, in Table 8.1A.1-2 - Include a table with the evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA, in FR2-2 - Measurement restrictions - Scheduling availability of UE during radio link monitoring fir FR2-2 - Discuss whether to define CSI-RS requirements for RLM in FR2-2, since there are no CSI-RS RLM requirements for unlicensed spectrum in FR1. |
|  | 8.5A Link recovery procedures | Yes | Clarify that the current requirements in this clause are applicable only to FR1 Define TEvaluate_BFD_SSB_CCA, TEvaluate_CBD_SSB_CCA for FR2-2 (Clause 8.5A.2.2) Measurement restriction for FR2-2 in clause 8.5A.2.3 Define requirements for scheduling availability of UE performing beam failure detection on FR2-2 with CCA Discuss whether to define CSI-RS requirements for BFD in FR2-2, since there are no CSI-RS RLM requirements for unlicensed spectrum in FR1. |
|  | Uplink spatial relation switch delay | No |  |
|  | UE specific CBW change | No |  |
|  | Pathloss reference signal switch delay | No |  |
| Measurement requirements | Measurement gap | No |  |
|  | UE measurement capability | No |  |
|  | 9.2A Intra-frequency | Yes | Switching time for FR2 SS-RSRP, SS-RSRQ, SS-SINR side conditions for FR2 Number of Cell and SSBs for FR2 Include tables with the requirements for: TPSS/SSS_sync_intra_CCA, T SSB_measurement_period_intra_CCA and TSSB_time_index_intra_CCA in FR2, with and without gaps FFS: RSSI measurement details depend on RAN1 decision. |
|  | 9.3A Inter-frequency | Yes | Switching time for FR2 SS-RSRP, SS-RSRQ, SS-SINR side conditions for FR2 Number of Cell and SSBs for FR2 Include tables with the requirements for: TPSS/SSS_sync_inter_CCA, T SSB_measurement_period_inter_CCA and TSSB_time_index_inter_CCA in FR2. FFS: RSSI measurement details depend on RAN1 decision. |
|  | Inter-RAT | No |  |
|  | 9.5A L1-RSRP measurements for reporting | Yes | Side conditions nrofReportedRS must be defined for FR2 The measurement period of TL1-RSRP_Measurement_Period_SSB_CCA must be defined for FR2-2 |
|  | L1-SINR measurements for reporting | No |  |
|  | Cross link interference measurements | No |  |
|  | CSI-RS based measurement | No | No impact, if RAN4 uses NR-U requirements in FR1 as baseline |

- TS 36.133 specification impact

In the last RAN4 meeting, the following scenarios were agreed to be in the scope of the WI NR_ext_to_71GHz:

Deployment scenarios - To clarify the scope of RRM, RRM requirements for Ext_to_71GHz should be defined for the scenarios below in Rel-17. - FR2-2 single carrier and CA in SA - FR1+FR2-2 CA (FR1 is PCell) - FR1+FR2-2 DC (FR1 is PCell) - Other scenarios are precluded in Rel-17.

In our view it is still unclear whether FR1 refers only to NR or also to LTE. We would like to clarify it.

RAN4 to clarify if EN-DC when the NR carrier in FR2-2 is subject to CCA is within the scope of the requirements defined in Rel-17.

Even if EN-DC (with NR in FR2-2) is not within the scope of the Rel-17 work item, there might be specification impact on TS 36.133 due to CCA requirements in FR2-2:

- Cell reselection

- Handover, if agreed that handover between LTE and NR-U in FR2-2 is within the scope

- RRC connection release with redirection

- Addition and Release Delay of NR PSCell*

- Measurement procedures (E-UTRAN FDD/TDD – NR measurements when CCA is used)

- NR measurements under CCA

Additionally, if it is agreed that EN-DC is within the scope, the following clauses would be affected:

- E-UTRA Inter-RAT NR measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation

- SFN and Frame Timing Difference (SFTD) under CCA

RAN4 to consider Table 3 for the discussion of the impact of FR2-2 for requirements with CCA, taking into account the timeline of Rel-17 work

- Table 3-Analysis of the impact of the operation on FR2-2 for requirements with CCA in TS 36.133

| Requirements and clauses |  | Impact for FR2-2 | Comments |
| --- | --- | --- | --- |
| Cell reselection | 4.2.2.5.7 Measurements of NR cells subject to CCA | Yes | Table 4.2.2.5.7-1 to be revisited. FR2 scaling factor N1 to be considered for requirements with CCA |
| Handover | New clause “5-3-5A E-UTRAN NR FR2 Handover to target cell using CCA” | FFS | If it is agreed to define requirements for handover between E-UTRAN and NR-U in FR2-2, a new clause is needed. |
| RRC Connection Mobility Control | 6.3.2.5 RRC connection release with redirection to NR carrier subject to CCA | Yes | Time to identify target NR cell to be defined for FR2 requirements with CCA |
| Timing and Signalling characteristics | 7.31A Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA – NR Dual Connectivity | No |  |
| Inter-RAT measurements | 8.1.2.4.21A E-UTRAN FDD – NR measurements when CCA is used | Yes | Revisit the tables 8.1.2.4.21A.1.1-1, 8.1.2.4.21A.1.1-2 and 8.1.2.4.21A.1.1-3 to include the FR2-2 requirements: revisit TPSS/SSS_sync_irat_CCA, T SSB_measurement_period_irat_CCA and TSSB_time_index_irat_CCA in FR2-2. |
|  | 8.1.2.4.21A E-UTRAN TDD – NR measurements when CCA is used | Yes | Same as E-UTRAN FDD – NR measurements. |
|  | 8.1.2.4.25.2a SFTD Measurement delay with CCA on target frequency | No |  |
|  | 8.17.2.2.a SFTD Measurement requirements with CCA on target frequency | No |  |
| Measurements for E-UTRA – NR Dual Connectivity | 8.17.4A E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation | FFS | If EN-DC is within the scope of the work item in Rel-17, the following tables need to be revisited: 8.17.4A.1.1-1, 8.17.4A.1.1-2, 8.17.4A.1.2: FR2 scaling factor N1 to be considered for requirements with CCA. |

Conclusion

In this discussion, Nokia’s views on the LBT impacts on NR RRM requirements were discussed. The following observation and proposals were presented:

- The requirements with LBT defined in TS 38.133 are only applicable for FR1.

- At 60 GHz narrow beams are expected, which, combined with a stronger signal attenuation, results in a LBT failure probability which is lower than what is experienced at 5 GHz band.

- At 60GHz, the performance of directional LBT and omni-directional LBT is similar to the performance with no-LBT.

- The LBT procedure in FR2-2 is simplified when compared to FR1. There is no adaptation of the contention window based on collision detection, the deferral times are also reduced, and the discovery burst transmissions by the gNB might be exempt from LBT.

- RAN4 has decided to extend measurement periods for most of the NR-U RRM requirements in FR1 in order to cope with the high probability of LBT failure due to interference in the 5GHz band.

- As in NR-U in FR1, RAN1 has specified the transmission of SSBs in different candidate positions in case of LBT failure during the transmission of the SSB burst in FR2-2.

Most of the FR2 RRM requirements are scaled by a scaling factor N1, which allows for longer measurement and evaluation periods when compared to FR1 RRM requirements.

- Use the FR2 RRM requirements as baseline for the RRM requirements with LBT in FR2-2, considering that the probability of failure of LBT in FR2-2 is nearly zero.

RAN4 to consider Table 1 for the discussion of the impact of FR2-2 for requirements with CCA, taking into account the timeline of Rel-17 work

| Requirements and clauses |  | Impact for FR2-2 | Comments |
| --- | --- | --- | --- |
| Cell reselection | 4.2A.2.2 Measurement of serving cell | Yes | Table 4.2A.2.2-1 to be revisited. FR2 scaling factor N1 to be considered for requirements with CCA |
|  | 4.2A.2.3 Intra-frequency | Yes | Table 4.2A.2.3-1 to be revisited. FR2 scaling factor N1 to be considered for requirements with CCA |
|  | 4.2A.2.4 Inter-frequency | Yes | Table 4.2A.2.4-1 to be revisited. FR2 scaling factor N1 to be considered for requirements with CCA |
|  | 4.2A.2.5 Inter-RAT | No | This requirement is band agnostic. |
|  | 4.2A.2.6 Maximum interruption on paging reception when CCA is used in Target Cell | No | This requirement is band agnostic. |
| Handover | 6.1B Handover to target cell using CCA | yes | Discuss for which scenarios the requirements with CCA are to be defined for: - FR2-FR2 handover - FR2-FR1 handover - FR1-FR2 handover - E-UTRAN - NR FR2 handover: if possible, there is need to assess the impact on TS 36.133 as well |
| RRC Connection Mobility Control | 6.2.1A RRC re-establishment with CCA | Yes | Time to identify target NR cell to be defined for FR2 requirements with CCA Revisit Table 6.2.1.2.1-2: |
|  | 6.2.2A Random access when CCA is used | No | Existing requirements with CCA are also applicable for FR2-2 |
|  | 6.2.3.2.3 RRC release with redirection to NR carrier subject to CCA | Yes | Time to identify target NR cell needs to be defined for FR2 requirements with CCA Revisit Table 6.2.3.2.3-1: |
| UE timing | 7.1 UE transmit timing | FFS | Subject to agreements on UE transmit timing in FR2-2. FFS number of SSB candidate positions |
|  | 7.2 UE timer accuracy | No | Reuse FR2 or FR2-2 requirements |
|  | 7.3 Timing advance | No | Reuse FR2 or FR2-2 requirements |
|  | 7.5 UE maximum receive timing difference | No | Reuse FR2 or FR2-2 requirements |
|  | 7.6 UE maximum transmission timing difference | No | Reuse FR2 or FR2-2 requirements |
| Signalling characteristics | 8.3A SCell activation and deactivation delay | Yes | Clarify that the current requirements in this clause are applicable only to FR1 FR2-2 specific requirements to be defined, like the Tactivation_time_withCCA, |
|  | Interruption | FFS | FFS if impact on interruptions due to Active BWP switching Requirement |
|  | PSCell addition and release delay | No |  |
|  | 8.10A Active TCI state switching delay | Yes | Clarify that the current requirements in this clause are applicable only to FR1 Requirements with CCA should include FR2-2 specific timing TL1-RSRP for MAC-CE based TCI state switch delay |
|  | 8.6 Active BWP switching delay | No | Impact is generic and covered with FR2-2 requirements without CCA |
|  | PSCell change | No |  |
|  | Conditional PSCell change | No |  |
|  | 8.1A Radio link monitoring | Yes | The following needs to be defined for FR2-2 with CCA: - Maximum number of RLM resources, in Table 8.1A.1-2 - Include a table with the evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA, in FR2-2 - Measurement restrictions - Scheduling availability of UE during radio link monitoring fir FR2-2 - Discuss whether to define CSI-RS requirements for RLM in FR2-2, since there are no CSI-RS RLM requirements for unlicensed spectrum in FR1. |
|  | 8.5A Link recovery procedures | Yes | Clarify that the current requirements in this clause are applicable only to FR1 Define TEvaluate_BFD_SSB_CCA, TEvaluate_CBD_SSB_CCA for FR2-2 (Clause 8.5A.2.2) Measurement restriction for FR2-2 in clause 8.5A.2.3 Define requirements for scheduling availability of UE performing beam failure detection on FR2-2 with CCA Discuss whether to define CSI-RS requirements for BFD in FR2-2, since there are no CSI-RS RLM requirements for unlicensed spectrum in FR1. |
|  | Uplink spatial relation switch delay | No |  |
|  | UE specific CBW change | No |  |
|  | Pathloss reference signal switch delay | No |  |
| Measurement requirements | Measurement gap | No |  |
|  | UE measurement capability | No |  |
|  | 9.2A Intra-frequency | Yes | Switching time for FR2 SS-RSRP, SS-RSRQ, SS-SINR side conditions for FR2 Number of Cell and SSBs for FR2 Include tables with the requirements for: TPSS/SSS_sync_intra_CCA, T SSB_measurement_period_intra_CCA and TSSB_time_index_intra_CCA in FR2, with and without gaps FFS: RSSI measurement details depend on RAN1 decision. |
|  | 9.3A Inter-frequency | Yes | Switching time for FR2 SS-RSRP, SS-RSRQ, SS-SINR side conditions for FR2 Number of Cell and SSBs for FR2 Include tables with the requirements for: TPSS/SSS_sync_inter_CCA, T SSB_measurement_period_inter_CCA and TSSB_time_index_inter_CCA in FR2. FFS: RSSI measurement details depend on RAN1 decision. |
|  | Inter-RAT | No |  |
|  | 9.5A L1-RSRP measurements for reporting | Yes | Side conditions nrofReportedRS must be defined for FR2 The measurement period of TL1-RSRP_Measurement_Period_SSB_CCA must be defined for FR2-2 |
|  | L1-SINR measurements for reporting | No |  |
|  | Cross link interference measurements | No |  |
|  | CSI-RS based measurement | No | No impact, if RAN4 uses NR-U requirements in FR1 as baseline |

RAN4 to clarify if EN-DC when the NR carrier in FR2-2 is subject to CCA is within the scope of the requirements defined in Rel-17.

RAN4 to consider Table 3 for the discussion of the impact of FR2-2 for requirements with CCA, taking into account the timeline of Rel-17 work

| Requirements and clauses |  | Impact for FR2-2 | Comments |
| --- | --- | --- | --- |
| Cell reselection | 4.2.2.5.7 Measurements of NR cells subject to CCA | Yes | Table 4.2.2.5.7-1 to be revisited. FR2 scaling factor N1 to be considered for requirements with CCA |
| Handover | New clause “5-3-5A E-UTRAN NR FR2 Handover to target cell using CCA” | FFS | If it is agreed to define requirements for handover between E-UTRAN and NR-U in FR2-2, a new clause is needed. |
| RRC Connection Mobility Control | 6.3.2.5 RRC connection release with redirection to NR carrier subject to CCA | Yes | Time to identify target NR cell to be defined for FR2 requirements with CCA |
| Timing and Signalling characteristics | 7.31A Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA – NR Dual Connectivity | No |  |
| Inter-RAT measurements | 8.1.2.4.21A E-UTRAN FDD – NR measurements when CCA is used | Yes | Revisit the tables 8.1.2.4.21A.1.1-1, 8.1.2.4.21A.1.1-2 and 8.1.2.4.21A.1.1-3 to include the FR2-2 requirements: revisit TPSS/SSS_sync_irat_CCA, T SSB_measurement_period_irat_CCA and TSSB_time_index_irat_CCA in FR2-2. |
|  | 8.1.2.4.21A E-UTRAN TDD – NR measurements when CCA is used | Yes | Same as E-UTRAN FDD – NR measurements. |
|  | 8.1.2.4.25.2a SFTD Measurement delay with CCA on target frequency | No |  |
|  | 8.17.2.2.a SFTD Measurement requirements with CCA on target frequency | No |  |
| Measurements for E-UTRA – NR Dual Connectivity | 8.17.4A E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation | FFS | If EN-DC is within the scope of the work item in Rel-17, the following tables need to be revisited: 8.17.4A.1.1-1, 8.17.4A.1.1-2, 8.17.4A.1.2: FR2 scaling factor N1 to be considered for requirements with CCA. |

References

- 3GPP TR 38.808 v17.0.0, Study on supporting NR from 52.6 GHz to 71 GHz

- EN 302 567 v2.2.0, "Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU", ETSI

- R1-2102563, Channel access mechanism, Nokia, Nokia Shanghai Bell

- R4-2108353, Work plan for NR extension to 71 GHz - RRM, Qualcomm

- R4-2108354, WF on NR extension to 71 GHz - RRM, Qualcomm

- R4-1912662 ,“Outline specification structure for 36.133 and 38.133 NR-U requirements”, Ericsson

- R4-2120316, “WF on NR extension to 71GHz RRM requirements, Part 1” Qualcomm

- R4-2017080, WF on NR-U RRM Core Requirements, Ericsson

- R4-2115351, WF on NR extension to 71 GHz – RRM - Part 1, Qualcomm

- R4-2115352, WF on NR extension to 71 GHz – RRM - Part 2, Intel

- 3GPP TS 38.133: “NR; Requirements for support of radio resource management”

- TS 38.213, NR; Physical layer procedures for control

- TS 38.211, NR; Physical channels and modulation