3GPP TSG-RAN WG4 #101-bis-e	R4-2200879

Electronic meeting, Jan. 17-25, 2022

Source: 	Ericsson

Title:  	Measurement procedure requirements for HST FR2

Agenda Item:	6.9.4.6

Document for:	Discussion

# Introduction

Work on RRM requirements for train-mounted UE in high speed train scenario in FR2 [1] continued during the RAN4#101e meeting, with outcome in terms of agreements and open issues captured in WFs [2, 3].

In this contribution, we focus on issues measurement procedure specifications due to HST FR2 deployment.

# Disucssion

The issue related to L1-RSRP is captured in WF and listed as below:

For L1-RSRP measurement - Reuse the Rel-16 FR1 HST scaling factor K for FR2 HST L1-RSRP measurement requirement, with the same SMTC periodicity bound of 40ms unless technical issues identified - Separate sets of requirements for deployment Scenarios A and B - For scenario A Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*NA)*TSSB). DRX cycle ≤ [80ms]; max(TReport, ceil([1.5]*M*P*NA)*max(TDRX,TSSB)). Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting.. NA = 2 - For scenario B Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*NB)*TSSB). DRX cycle < [80ms]; max(TReport, ceil([1.5]*M*P*NB)*max(TDRX,TSSB)). Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting.. NB = 6

Proposal 1: Agree with separating sets of requirements for deployment Scenarios A and B.

# Conclustion

Proposal 1: Agree with separating sets of requirements for deployment Scenarios A and B.

# References

- RP-202118 “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia

- R4-2120292 “WF on FR2 HST RRM (part 1)”, Nokia, Nokia Shanghai Bell

- R4-2120293 “WF on FR2 HST RRM (part 2)”, Samsung