3GPP TSG-RAN WG4 #101-bis-e	R4-2200891

Electronic meeting, Jan. 17-25, 2022

Source: 	Ericsson

Title:  	Measurement requirements for NTN

Agenda Item:	6.13.5.5

Document for:	Discussion

# 1	Introduction

In RAN4# 101-e a WF on NTN RRM measurement requirements was approved [1]. In this contribution we develop topics on measurement requirements which were agreed to continue study in RRM measurement part in the WF.

# 2	Discussion

## Issue 3-1: Multiple SMTCs and Measurement Gap

The issue related to UE capability regarding the number measurement cell groups is identified as follows:

Issue 3-1-1: The maximum number of SMTCs per Frequency layer Agreement (1st round GTW): - The maximum number of SMTCs configured per measurement object for the same ssbFrequency is 4. - FFS on the assumptions on the number of configured parallel SMTCs to be used for requirements definition - FFS if different UE capabilities to support measurements for multiple configured SMTCs are needed Agreements: RAN4 to send a reply LS to RAN2 “R4-2120308, Reply LS on Multiple SMTCs for NR NTN” as a part of outcome of Issue 3-1. Issue 3-1-2: Capability on the number of Measurement Cell Groups Conclusion: Revisit the issue in the next RAN4 meeting based on RAN2#116 agreements and/or a partial/full response to R4-2120308 “Reply LS on Multiple SMTCs for NR NTN” if made available by/during the next RAN4 meeting.

Revisit the issue in the next RAN4 meeting. - Option 1: (Huawei) - For Connected mode, define a common set of measurement capability requirements for all scenarios: - define the number of carriers UE needs to monitor as 3 - define the number of SSB beams UE needs to monitor per carrier as 8 - For Idle mode, number of carriers UE needs to monitor should be increased to account for TN frequency layers. - Option 2: (CATT) - Reuse total carriers number of inter-frequency measurement, including NTN carriers and TN carriers. - The total number of measurement cells per frequency layer and the total number of measurement beams per cell can be reduced. - The measurement capability of number cells for each layer measured defined in NR RRM specification should be based on assumption for satellite cell architecture. If beams from same satellites on same frequency layer have the same cell ID, the measurement capability of number cells for each layer can be 4; otherwise reuse TN requirements. - Option 3: (Intel) - Reuse the measurement capability requirements in terms of the maximum number of monitored cells, specified in R15, for NTN NR Ues. - Option 4: (MediaTek) - UE shall be capable of monitoring at least K NTN carriers, where K is TBD. - For the NTN UE, the measurement capability in CONNECTED mode regarding TN carrier can be removed. - Option 5: (Ericsson, Nokia) - To complete uncovered items listed in below, not limited in the list, multi-satellite simulation is needed. - the total number of measurement cells per frequency layer - the total number of measurement beams (SSB and/or CSI-RS) per cell - DRX issue

Proposal 1: Current estimations on number are based on single satellite simulation, the feasibility is doubtable.  The final target is Option 5, we understand the difficulties. At this moment, we can use TBD number here.

Issue 3-1-4: Measurement with multiple SMTCs Agreements: The following issues will be discussed in the next RAN4 meeting - Issue 1: - Determine in which way RAN4 should specify the requirements: - Option 1: the UE is required to measure on the target neighbour cell with configured timing offsets and receive/transmit on the serving cell at the same time - Option 2: specify scheduling restrictions to avoid such complexity - Option 3: measurement gaps are used on the target neighbour cells - Issue 2: - FFS on scaling of the measurement period is needed if UE is configured to measure multiple SMTC windows per MO. - Issue 3: - Scaling scheme on multi-SMTC for intra-frequency is needed when simultaneous measurement is invalid in case of partial overlapping. But simultaneous measurement in case of partial overlapping shall not be precluded, instead of optional support. - Issue 4: - Whether and how to define the applicability rule for requirements regarding the number for MOs - Issue 5: - Measurement with Gap-less vs. Gap-based - Issue 6: - for intra-frequency MO without MG and inter-frequency MO without MG - if UE has no scheduling restriction within SMTC, the maximum number of SMTCs per measurement object for the same ssbFrequency used by UE can be equal to 4 - otherwise if UE has scheduling restriction within SMTC, the maximum number of SMTCs per measurement object for the same ssbFrequency used by UE shall meet both of following conditions: - smaller than or equal to 4, and - guarantee the total scheduling restriction length less than or equal to X% of the maximum SMTC periodicity length. - X is FFS - for intra-frequency MO with MG and inter-frequency MO with MG, the maximum number of SMTCs per measurement object for the same ssbFrequency used by UE shall meet both of following conditions: - smaller than or equal to 4, and - guarantee these SMTCs can be contained in active measurement gaps. The concurrent MG number is up to the conclusions in MG enhancement WI.

Figure 1 Possible multi-SMTC overlapping

In RAN2#116-e meeting, following agreements on SMTC number were made after previous agreements to support maximal 4 SMTCs.

Agreements: 1. In NW-based solution, the network can configure up to 2 SMTCs in parallel and the UE uses all of them, i.e. there is no switching between or activation/deactivation of configured SMTCs. FFS whether this (UE support for 2 SMTCs) requires a UE capability. A UE can optionally indicate support for 4 SMTCs (in this case the NW can configure up to 4 SMTCs in parallel)

We suppose NW-based solution isn’t precluded in RRC_IDLE state. The agreements for UE in RRC_IDLE state may be interpreted that when SMTC number is greater than 2, UE isn’t required to measure all SMTCs simultaneously in one measurement periodicity to avoid severe scheduling restriction.

In low activity RRC states (e.g. in RRC_IDLE/RRC_INACTIVE state) UE needs to measure the SSB of neighboring cells to perform cell reselection. In low activity RRC state the UE typically performs measurement on signals (e.g. SSB) of one or more cells (e.g. satellites) e.g. once every DRX cycle. To ensure UE can detect the SSBs of neighboring cells accurately, system information of current serving cell will broadcast the SMTC configurations of neighboring cells per carrier frequency, the configurations of SMTCs e.g. SMTC1, SMTC2 etc.  It may happen that number of SMTC configurations transmitted (e.g. broadcasted) by NW is more than maximal SMTC number supported by UE.

Proposal 2: NW-based measurement in RRC_IDLE state isn’t precluded, even it is problematic in some satellite deployment scenarios. RAN4 shall study NW-based measurement in RRC_IDLE state and determine whether more inputs from RAN2 are necessary.

Another critical issue in RRC_IDLE state is that the SMTC configuration broadcasted isn’t UE specific. Same SMTC configuration would not be applicable to the entire cell’s coverage considering huge size of cell footprint. In this sense, some UEs can not distinguish SSBs in SMTC configured by network. The issue also causes ambiguity between no SSBs in proper SMTC and unproper SMTC, i.e. UE may not have capability to identify the difference between them and results into wrong procedure and more measurement delay. SMTC configured UE-based SMTC which is under discussion in RAN2 is potential solution to solve the problem but without agreement.

Proposal 3: Wait for RAN2’s agreements on detailed mechanism of UE-based SMTC measurement in RRC_IDLE state, which may influence measurement delay.

Observation 1: How to measure M SMTCs configured by NW with N SMTCs capability in RRC_IDLE state isn’t discussed before. There are two possible approaches:

- UE only measures N SMTCs, which are determined by UE or NW. The drawback is that UE may miss measurement report in other SMTCs which may be helpful in some cases.

- UE measures all M SMTCs with sort of sharing scheme, e.g. round robin. The drawback is that additional measurement delay is necessary to ensure power saving.

Proposal 4: RAN4 needs to study UE’s measurements in RRC_IDLE state when M SMTCs which are configured and broadcasted by network upon UE’s capability to support N SMTCs (M>N), for either NW-based measurement or UE-based measurement.

In RRC_CONNECTED state, UE can be configured with multiple SMTCs per frequency layer and can use them all in parallel partially. It shall be interpreted that SMTC configuration is flexible, e.g. partial overlapping between consecutive SMTCs isn’t rare configuration in practice regarding the common case that propagation delay offset between cells belonging to same satellite or cells belonging to different satellites < 5ms.

Recap RAN2’s agreements here again:

Agreements: 1. In NW-based solution, the network can configure up to 2 SMTCs in parallel and the UE uses all of them, i.e. there is no switching between or activation/deactivation of configured SMTCs. FFS whether this (UE support for 2 SMTCs) requires a UE capability. A UE can optionally indicate support for 4 SMTCs (in this case the NW can configure up to 4 SMTCs in parallel)

The agreements for UE in RRC_CONNECTED state may be interpreted that when SMTC number is greater than 2, UE isn’t required to measure all SMTCs simultaneously in one measurement periodicity to avoid severe scheduling restriction.

On contrary, it also can be interpreted that UE needs to measure all SMTCs signaled by network simultaneously in one measurement periodicity, but UE report to network is allowed, e.g. UE requests measurements on certain or certain amount of SMTCs. Meanwhile, network shall be aware of scheduling restriction also.

Proposal 5: When more than two SMTCs are signaled by network for UE in RRC_CONNECTED state. NW-based measurement is still vague. We should align the interpretation on ‘A UE can optionally indicate support for 4 SMTCs (in this case the NW can configure up to 4 SMTCs in parallel)’, where the term ‘support’ can interpret UE behavior like:

- fully measure all SMTCs simultaneously in one measurement periodicity, or

- measure all SMTCs on more measurement periodicities

Another concern is that whether, and with what accuracy, the serving gNB is aware of the UE’s location, e.g. through location reports from the UE. Note that earlier RAN2 agreed that it cannot be assumed that the network will always have accurate information about the UE’s location for SMTC window configuration in NTN specially in RRC_IDLE state.

Proposal 6: It’s rational that the delay resulting from measurement attempts are relevant to SMTC window configuration. More measurement attempts are needed in case of non-fitting SMTC window configuration. RAN4 shall study the probability of imperfect SMTC configuration and how to deal with the issue.

Issue 3-1-6: Measurement Gap Conclusion: Needs further RAN2 progress. - Option 1: (Xiaomi) - the concurrent MG discussed in Rel-17 MG enhancement WI can be applied for the case of multiple SMTC configurations in one frequency layer in NR NTN. - Option 2: (Huawei, Apple, CMCC) - RAN4 to wait for further conclusions from RAN2 on MGs for defining requirements for MG based measurement. - Option 3: (Intel) - Specify if needed, NR NTN UE measurement gap capability requirements based on the outcome of R17 measurement gap enhancement work item - Option 4: (Ericsson) - RAN4 shall check whether restriction is needed when different MG configurations occur. - In consideration of different MG configurations e.g. partial overlapping, scaling scheme among MGs is needed. - Regarding L1-RSRP impact due to multi-MG, P scaling factor shall be further updated by the overlapping with the sets or unions of multi-MG. - Option 5: - Others are not precluded

RAN2#116-e made agreements on SMTC and corresponding MG as below:

Agreements via email - from offline 103: 1. RAN2 will decide which option to choose for NTN assistance information for SMTC/MG once SA3 feedback on user consent is received. 2. If propagation delay based UE assistance information for NTN SMTC is agreed, it is defined in the form of propagation delay difference. 3. RAN2 assumes FL delay is known to and compensated by the network. RAN2 also assumes the UE needs to have neighbour cell ephemeris for the propagation delay estimation. 4. In NW-based SMTC solution the UE is not allowed to apply shifts to configured SMTCs. 5. Measurement gap related aspects for Rel-17 NTN will be addressed in Rel-17 NTN WI. Coordination and avoiding overlap with other WIs and WGs is recommended. 6. RAN2 will reuse at least the SMTC agreements made for UE assistance information reporting also in the area of measurement gaps for NTN

Regarding the content of above agreements, RAN2 didn’t make any agreement on MG configurations, e.g. number and MGL. Meanwhile, Concurrent MG WI has not made agreements on MG configurations including number, proximity condition for overlapping and UE behavior during colliding gap occasion.

Proposal 7: Wait RAN2 and Concurrent MG WI to clarify issues on MG firstly, the issues should include:

- Maximal number of MG

- Matching between SMTC and MG if applicable

- Proximity condition for overlapping

- UE behavior during colliding gap occasion

Another issue on MG is that consideration of propagation delay in TN system may not cover NTN system sufficiently.  In one example, assuming 5ms windows of SS burst set of the serving cell and neighbor cells, NR measurement gap length 6ms shall include the below aspects at least:

- RF switching time including RF switching- to and switching back

- Propagation delay offset between serving cell and neighbor cell

- SSB window (5ms)

Whereas, propagation delay offset only takes 33μs into account which aiming for 10km distance difference between serving cell and neighbor cell.

In NTN system, propagation delay offset between serving cell and neighbor cell, specially for inter-satellite, is in order of hundreds of μs or more. It is probable that present MG won't be able to adequately cover SMTCs correctly, e.g. 1ms between SMTC and MG are not enough to cover propagation delay offset. Furthermore, timing among different satellites isn’t always synchronized which also worsen accuracy of MG. Even if network (serving cell) and UE have sufficient knowledge of ephemeris data of satellites and UE’s position to ensure one MG can match more than one SMTC on inter-frequency, it’s worth noting that the amount of SMTC cover by the MG would be very limited due to difficulty in finding the matching SMTCs (only can measure those SMTCs with short propagation delay offset) for the MG. One of the sequences is that more MGs are required to cover sufficient amount of SMTCs.

Proposal 8:  RAN4 to discuss how MG deals with unalignment,e.g. edge of SMTC window may cross MGL, due to propagation delay offset/timing error between serving cell and neighbor cell.

## Issue 3-2: Measurement relaxation

## Issue 3-2: Measurement relaxation Issue 3-2-1: Cell Service Time based Measurement Relaxation Conclusion: RAN4 can discuss the following aspect further: - Measurement can be relaxed based on cell service time information if exist and applicable. The detail scheme can be studied whereafter, e.g. no mandatory measurements before T2 or relaxed measurement required before T2. T2 is the starting time of [a new cell] measurement.

In RAN2#116e, the agreements on cell service time was made as follows:

Agreements via email - from offline 102 - second round: 1. For quasi-earth fixed cell, UE should start measurements on neighbour cells before the serving cell stops covering the current area, regardless of (the distance between UE and serving cell reference location) or (if legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold).

As currently RAN2 only agree to broadcast stop time of the serving cell for quasi-earth fixed case, so the prerequisite should be that the cell stop time of neighbor cells need to be broadcasted.

Recap our proposal on updating reselection in another clause here:  For quasi-earth fixed cell, update of requirement for reselection is shown as below exemplarily: ‘The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 [1] within Tdetect,NR_Intra when that Treselection= 0 before cell is going to stop serving the area, if applicable .’

Although the exact time to start intra-frequency and inter-frequency measurements is agreed to be UE implementation in RAN2, RAN4 shall define the proper delay of measurements before stop time of the serving cell, for the purpose of power saving and abbreviating data interruption. Otherwise, UE may fail to complete measurement before stop time of the serving cell or measure unnecessarily for a long time.

Essentially, it’s up to UE to start measurement between 2 bounds: the earliest detectable time for a neighbor cell and stop time of the serving cell. earliest detectable time for a neighbor cell is various to different neighbor cells.

It is rational that UE shall avoid too frequent reselection, e.g. UE may stay in serving cell as long as possible even R-criteria is fulfilled already.  Subsequently, for some neighbor cells (e.g. neighbor cell 2 in above figure), UE shall complete measurement as soon as possible; for some neighbor cells (e.g. neighbor cell 2 in above figure), UE can adaptively change measurement time because UE has plenty of time before stop time of the serving cell.

A solution looks right is up to UE to skip neighbor cells which only can be detected short time prior to stop time of serving cell. However, it is probable that these neighbor cells may have longer coverage time until stop time of these neighbor cells. Therefore, we expect that UE shall measure all available neighbor cells but measurement delay may vary.

The earliest detectable time for a neighbor cell has not definition right now, but we suppose at least there are two approaches which can get it: firstly, the earliest detectable time for a neighbor cell can determined by ephemeris data of neighbor cells; secondly, if ephemeris data is unavailable, UE may keep present measurement procedure and UE shall evaluate time difference between stop time of serving cell and the time when UE detects new neighbor cell.

Proposal 9: ‘Relaxation’ maybe is not a good term in this issue because we have not defined what is the regular requirements. Instead and in fact, we suggest conditional measurement requirements with assistance information of stop time of the serving cell if applicable.

- The detailed measurement requirements on different conditions can involve scaling factor, number of frequency layers, number of SMTC or number of cells (similar to Issue ‘Measurement Prioritization during CHO’ in section ‘Mobility requirements for NTN’) .

- Typical conditions are: if very short time is left for measurement before stop time of the serving cell, faster measurement requirements shall be defined, on the contrary, longer measurement requirements delay can be allowed if there is a large amount of time before stop time of the serving cell.

- Adaptive/changeable measurement requirements can be separately defined for RRC_IDLE state and RRC_CONNECTED state.

Issue 3-2-2: Location based Measurement Relaxation Conclusion: RAN4 can discuss the following aspect further: - Measurement period relaxation based on distance between UE and the [reference location].

In addition, In RAN2#116-e meeting, following agreements for IDLE/INACTIVE state were made.

Agreements: 1. Location assisted cell reselection, with the distance between UE and the reference location of the cell (serving cell and/or neighbor cell) taken into account, is supported for quasi-earth fixed cell. FFS on how UE performs location acquisition.

Proposal 10: In similar manner of cell service time information, measurement can be relaxed/conditional with the distance between UE and the reference location of the cell (serving cell and/or neighbor cell) taken into account. Relaxation/ conditional can be separately defined for RRC_IDLE state and RRC_CONNECTED state. But two issues may be relevant to when to relax measurement or condition to relax measurement:

- UE is not mandated to valid location acquisition ‘always’ in idle mode mobility.

- RAN2 only has defined that when can start to measure in the location based reselection, that is different from time based reselection which has a limitation on ending time of measurements.

Regarding L1-mobility, it’s known that RSRP varies in a relatively small range for a long while, it is possible to relax L1-mobility requirements and L1-RSRP compared with TN system, i.e. a frequent measurement play less important role than that in TN system. Also, given validity of timing/location information for CHO, ephemeris data of satellite and UE position,  we can assume no significant RSRP change before the time close to CHO.

Proposal 11: RAN4 shall discuss whether L1 mobility can be relaxed in comparison to present requirements for TN system, provided assistance information.

Issue 3-2-3: TN Cell Measurement Relaxation in RRC Idle mode Conclusion: RAN4 can discuss the following aspect further: - For idle mode, the UE upon detecting any NTN cell, shall meet legacy (non-relaxed) measurement requirements (low mobility relaxation can refer to section 4.2.2.9 in TS38.133) for TN cells even if it is configured with and meets the relaxed measurement criterion for TN cells.

In idle mode, it is reasonable to expect TN cell measurement to be more frequent/intense than NTN cell measurement; otherwise, NTN cell will be given more priority and opportunity, which is undesirable. Besides of RAN2 solution, RAN4 can ensure that once the NTN cell is detected, relaxed measurements on the TN cell will be inhibited.

Proposal 12: For idle mode, the UE upon detecting any NTN cell, shall meet legacy (non-relaxed) measurement requirements (low mobility relaxation can refer to section 4.2.2.9 in TS38.133) for TN cells even if it is configured with and meets the relaxed measurement criterion for TN cells.

# 2	Summary

Observation 1: How to measure M SMTCs configured by NW with N SMTCs capability in RRC_IDLE state isn’t discussed before. There are two possible approaches:

- UE only measures N SMTCs, which are determined by UE or NW. The drawback is that UE may miss measurement report in other SMTCs which may be helpful in some cases.

- UE measures all M SMTCs with sort of sharing scheme, e.g. round robin. The drawback is that additional measurement delay is necessary to ensure power saving.

Proposal 1: Current estimations on number are based on single satellite simulation, the feasibility is doubtable.  The final target is Option 5, we understand the difficulties. At this moment, we can use TBD number here.

Proposal 2: NW-based measurement in RRC_IDLE state isn’t precluded, even it is problematic in some satellite deployment scenarios. RAN4 shall study NW-based measurement in RRC_IDLE state and determine whether more inputs from RAN2 are necessary.

Proposal 3: Wait for RAN2’s agreements on detailed mechanism of UE-based SMTC measurement in RRC_IDLE state, which may influence measurement delay.

Proposal 4: RAN4 needs to study UE’s measurements in RRC_IDLE state when M SMTCs which are configured and broadcasted by network upon UE’s capability to support N SMTCs (M>N), for either NW-based measurement or UE-based measurement.

Proposal 5: When more than two SMTCs are signaled by network for UE in RRC_CONNECTED state. NW-based measurement is still vague. We should align the interpretation on ‘A UE can optionally indicate support for 4 SMTCs (in this case the NW can configure up to 4 SMTCs in parallel)’, where the term ‘support’ can interpret UE behavior like:

- fully measure all SMTCs simultaneously in one measurement periodicity, or

- measure all SMTCs on more measurement periodicities

Proposal 6: It’s rational that the delay resulting from measurement attempts are relevant to SMTC window configuration. More measurement attempts are needed in case of non-fitting SMTC window configuration. RAN4 shall study the probability of imperfect SMTC configuration and how to deal with the issue.

Proposal 7: Wait RAN2 and Concurrent MG WI to clarify issues on MG firstly, the issues should include:

- Maximal number of MG

- Matching between SMTC and MG if applicable

- Proximity condition for overlapping

- UE behavior during colliding gap occasion

Proposal 8:  RAN4 to discuss how MG deals with unalignment,e.g. edge of SMTC window may cross MGL, due to propagation delay offset/timing error between serving cell and neighbor cell.

Proposal 9: ‘Relaxation’ maybe is not a good term in this issue because we have not defined what is the regular requirements. Instead and in fact, we suggest conditional measurement requirements with assistance information of stop time of the serving cell if applicable.

- The detailed measurement requirements on different conditions can involve scaling factor, number of frequency layers, number of SMTC or number of cells (similar to Issue ‘Measurement Prioritization during CHO’ in section ‘Mobility requirements for NTN’) .

- Typical conditions are: if very short time is left for measurement before stop time of the serving cell, faster measurement requirements shall be defined, on the contrary, longer measurement requirements delay can be allowed if there is a large amount of time before stop time of the serving cell.

- Adaptive/changeable measurement requirements can be separately defined for RRC_IDLE state and RRC_CONNECTED state.

Proposal 10: In similar manner of cell service time information, measurement can be relaxed/conditional with the distance between UE and the reference location of the cell (serving cell and/or neighbor cell) taken into account. Relaxation/ conditional can be separately defined for RRC_IDLE state and RRC_CONNECTED state. But two issues may be relevant to when to relax measurement or condition to relax measurement:

- UE is not mandated to valid location acquisition ‘always’ in idle mode mobility.

- RAN2 only has defined that when can start to measure in the location based reselection, that is different from time based reselection which has a limitation on ending time of measurements.

Proposal 11:  RAN4 shall discuss whether L1 mobility can be relaxed in comparison to present requirements for TN system, provided assistance information.

Proposal 12: For idle mode, the UE upon detecting any NTN cell, shall meet legacy (non-relaxed) measurement requirements (low mobility relaxation can refer to section 4.2.2.9 in TS38.133) for TN cells even if it is configured with and meets the relaxed measurement criterion for TN cells.

# References

- R4-2120307 WF on RRM requirements for NTN measurement and mobility	Qualcomm