3GPP TSG-RAN WG4 Meeting #101bis-e	R4-22xxxxx

# Electronic Meeting, 17th - 25th Jan, 2022

Title: 	Discussion on general UE demodulation requirements for FR2 HST

Source: 	Huawei, HiSilicon

Agenda item:	6.9.5.1

Document for:	Discussion

Background

During RAN4#101-e meeting, WF [1] on general and UE demodulation requirement for FR2 HST was approved. In this contribution, we share our views about NR FR2 HST general UE demodulation requirements for FR2 HST.

Discussion

UE FO/TO Tracking

• FFS on introduction higher layer signalling (ie. System Information, RRC)) to inform the UE of the FR2 HST deployment typology (Uni-directional and Bi-directional scenario) for PDSCH demodulation requirement • FFS on introduction MAC-CE signalling to inform the UE of the TCI switching typology (Intra/Inter -RRH) for PDSCH demodulation requirement • Encourage companies to analysis the impact on PDSCH demodulation requirement, the UE receiver processing , as well as the pros and cons with and without NW signalling, from demodulation perspective

From demodulation point of view, the mainly difference between Uni-directional with Scenario A and Bi-directional with Scenario B is whether large frequency jump happens, considering that delay jump is not verified under demodulation test. For our understanding, if UE use SSB+TRS for FO tracking, SSB is always used for coarse estimation to extend the range of maximum FO/TO tracking and infer whether there is large FO jump, we don’t see any necessity to introduce signalling to inform deployment type and we don’t see any different FO processing between inter-RRH based and intra-RRH based TCI switching.

Do not introduce higher layer signalling to inform the UE of the FR2 HST deployment typology (Uni-directional and Bi-directional scenario) for PDSCH demodulation requirement.

Do not introduce MAC-CE signalling to inform the UE of the TCI switching typology (Intra/Inter -RRH) for PDSCH demodulation requirement.

Test procedure

TRS periodic is discussed in last meeting, as following table shows.

- SSB and TRS period configuration - Option 1: 20ms for SSB, 10ms for TRS - Option 2: 20ms for both SSB and TRS - Other options are not precluded

We don’t think it necessary to configure the same transmission period for SSB and TRS as 20ms. It is benefit to configure 10ms TRS in HST scenario for more precise channel estimation.

Select 10ms for TRS for HST FR2 PDSCH requirements definition.

Proposals

In this contribution, we discuss on demodulation performance for NR UE HST FR2 general issues. Our observations and proposals are:

- Do not introduce higher layer signalling to inform the UE of the FR2 HST deployment typology (Uni-directional and Bi-directional scenario) for PDSCH demodulation requirement.

Do not introduce MAC-CE signalling to inform the UE of the TCI switching typology (Intra/Inter -RRH) for PDSCH demodulation requirement.

Select 10ms for TRS for HST FR2 PDSCH requirements definition.

Reference

R4-2120702, WF on general and UE demodulation requirement for FR2 HST, RAN4#101-e, Samsung