3GPP TSG-RAN WG4 Meeting #101-bis-e                                                                 R4-2201034

Online, 17th - 25th, Jan, 2022

Source:	Samsung, KDDI

Title:	TP for TR 38.717-03-01: CA_n18-n28-n77

Agenda item:			5.9.1

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-01 to include CA_n18-n28-n77 according to the request in [1].

2. Reference

- R4-2117386, Revised WID: Rel-17 NR inter-band CA for 3 bands DL with 1 band UL.

# 3. Text Proposal

<Start of Text Proposal>

6.x	CA_n18-n28-n77

6.x.1	Operating bands for CA

Table 6.x.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n18-n28-n77 | n18, n28, n77 |

6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration | NR Band |  |  | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
| CA_n18A-n28A-n77A | - | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 |  |  |  | 40 |  | 50 | 60 |  | 80 | 90 | 100 |  |  |
| NOTE 3: The SCS of each channel bandwidth for NR band refer to Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

6.x.3	Co-existence studies

Co-existence studies can be omitted because fallback combinations cover these requirements and analysis

6.x.4	∆TIB,c and ∆RIB,c values

For CA_n18-n28-n77, the TIB,c and RIB,c values can reuse DC_18-28_n77 defined in TS 38.101-3.

Table 6.X.4-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n18-n28-n77 | n18 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 6.X.4-2: ΔRIB,c  due to NR CA (three bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n18-n28-n77 | n18 | 0 |
|  | n28 | 0 |
|  | n77 | 0.5 |

6.x.5	REFSENS requirements

MSD requirements are captured in lower order combinations.

<End of Text Proposal>