3GPP TSG-RAN WG4 Meeting # 101-bis-e							     R4- 2201072

Electronic Meeting, January 17-25, 2022

Agenda item:			6.13.2.1

Source:	Samsung

Title:	NR-NTN co-ex study results

Document for:	Approval

# Introduction

In this document, we proposed our ACIR study results for co-ex case 3 and case 4.

# Co-ex study results

Following the updates in last RAN4 101-e meeting, the latest agreements about simulation assumptions for NTN, we further updated our simulator and proposed the following results for different scenarios in case 3 and case 4.

## Case 3 NTN DL interfering TN DL

## GEO interfering TN equipped with AAS antenna in Rural

| Required ACIR [dB] |  | Company | 8 | 10 | 12 | 14 | 16 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 5.41 | 3.63 | 2.39 | 1.55 | 1.00 | 0.64 |
|  | 5%-tile |  | 12.77 | 8.49 | 5.55 | 3.58 | 2.30 | 1.46 |

## GEO interfering TN equipped with sector antenna in Rural

| Required ACIR [dB] |  | Company | 6 | 8 | 10 | 12 | 14 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 3.12 | 2.03 | 1.31 | 0.83 | 0.53 | 0.34 |
|  | 5%-tile |  | 11.97 | 7.88 | 5.11 | 3.28 | 2.09 | 1.33 |

## GEO interfering TN equipped with AAS antenna in Urban

| Required ACIR [dB] |  | Company | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 0.50 | 0.32 | 0.21 | 0.13 | 0.08 | 0.05 |
|  | 5%-tile |  | 0.23 | 0.15 | 0.09 | 0.06 | 0.04 | 0.02 |

## GEO interfering TN equipped with sector antenna in Urban

| Required ACIR [dB] |  | Company | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 0.04 | 0.03 | 0.02 | 0.01 | 0.01 | 0.00 |
|  | 5%-tile |  | 0.15 | 0.10 | 0.06 | 0.04 | 0.02 | 0.01 |

## LEO1200 interfering TN equipped with AAS antenna in Rural

| Required ACIR [dB] |  | Company | 18 | 20 | 22 | 24 | 26 | 28 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 5.94 | 4.01 | 2.65 | 1.73 | 1.11 | 0.71 |
|  | 5%-tile |  | 14.04 | 9.40 | 6.17 | 4.00 | 2.57 | 1.64 |

## LEO1200 interfering TN equipped with sector antenna in Rural

| Required ACIR [dB] |  | Company | 16 | 18 | 20 | 22 | 24 | 26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 3.46 | 2.26 | 1.46 | 0.93 | 0.59 | 0.38 |
|  | 5%-tile |  | 13.24 | 8.76 | 5.71 | 3.67 | 2.35 | 1.50 |

## LEO1200 interfering TN equipped with AAS antenna in Urban

| Required ACIR [dB] |  | Company | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 4.24 | 2.94 | 1.99 | 1.32 | 0.87 | 0.56 |
|  | 5%-tile |  | 2.75 | 1.69 | 1.06 | 0.66 | 0.41 | 0.26 |

## LEO1200 interfering TN equipped with sector antenna in Urban

| Required ACIR [dB] |  | Company | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 0.47 | 0.30 | 0.19 | 0.12 | 0.08 | 0.05 |
|  | 5%-tile |  | 1.71 | 1.08 | 0.69 | 0.44 | 0.28 | 0.17 |

## LEO600 interfering TN equipped with AAS antenna in Rural

| Required ACIR [dB] |  | Company | 18 | 20 | 22 | 24 | 26 | 28 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 5.52 | 3.72 | 2.45 | 1.60 | 1.03 | 0.66 |
|  | 5%-tile |  | 13.13 | 8.76 | 5.73 | 3.71 | 2.37 | 1.51 |

## LEO600 interfering TN equipped with sector antenna in Rural

| Required ACIR [dB] |  | Company | 16 | 18 | 20 | 22 | 24 | 26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 3.21 | 2.09 | 1.35 | 0.86 | 0.55 | 0.35 |
|  | 5%-tile |  | 12.33 | 8.12 | 5.28 | 3.40 | 2.17 | 1.38 |

## LEO600 interfering TN equipped with AAS antenna in Urban

| Required ACIR [dB] |  | Company | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 4.07 | 2.81 | 1.90 | 1.26 | 0.83 | 0.54 |
|  | 5%-tile |  | 2.62 | 1.61 | 1.00 | 0.63 | 0.39 | 0.25 |

## LEO600 interfering TN equipped with sector antenna in Urban

| Required ACIR [dB] |  | Company | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 0.45 | 0.28 | 0.18 | 0.11 | 0.07 | 0.05 |
|  | 5%-tile |  | 1.63 | 1.03 | 0.66 | 0.42 | 0.26 | 0.16 |

## Case 4 NTN UL interfering TN UL

## GEO interfering TN equipped with AAS antenna in Rural

| Required ACIR [dB] |  | Company | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 1.48 | 1.05 | 0.74 | 0.51 | 0.35 | 0.24 |
|  | 5%-tile |  | 4.99 | 3.42 | 2.32 | 1.56 | 1.04 | 0.69 |

## GEO interfering TN equipped with sector antenna in Rural

| Required ACIR [dB] |  | Company | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 2.86 | 1.95 | 1.32 | 0.89 | 0.60 | 0.41 |
|  | 5%-tile |  | NaN | NaN | NaN | NaN | NaN | NaN |

## GEO interfering TN equipped with AAS antenna in Urban

| Required ACIR [dB] |  | Company | 24 | 26 | 28 | 30 | 32 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 2.00 | 1.53 | 1.17 | 0.89 | 0.67 | 0.50 |
|  | 5%-tile |  | 9.44 | 7.15 | 5.47 | 4.19 | 3.19 | 2.44 |

## GEO interfering TN equipped with sector antenna in Urban

| Required ACIR [dB] |  | Company | 12 | 14 | 16 | 18 | 20 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 8.39 | 6.50 | 5.06 | 3.96 | 3.11 | 2.43 |
|  | 5%-tile |  | NaN | NaN | NaN | NaN | NaN | NaN |

## LEO1200 interfering TN equipped with AAS antenna in Rural

| Required ACIR [dB] |  | Company | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 1.48 | 1.05 | 0.74 | 0.51 | 0.35 | 0.24 |
|  | 5%-tile |  | 4.99 | 3.42 | 2.33 | 1.56 | 1.04 | 0.69 |

## LEO1200 interfering TN equipped with sector antenna in Rural

| Required ACIR [dB] |  | Company | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 2.86 | 1.95 | 1.32 | 0.89 | 0.60 | 0.41 |
|  | 5%-tile |  | NaN | NaN | NaN | NaN | NaN | NaN |

## LEO1200 interfering TN equipped with AAS antenna in Urban

| Required ACIR [dB] |  | Company | 24 | 26 | 28 | 30 | 32 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 2.00 | 1.53 | 1.17 | 0.89 | 0.67 | 0.51 |
|  | 5%-tile |  | 9.44 | 7.16 | 5.50 | 4.22 | 3.21 | 2.44 |

## LEO1200 interfering TN equipped with sector antenna in Urban

| Required ACIR [dB] |  | Company | 12 | 14 | 16 | 18 | 20 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 8.38 | 6.49 | 5.05 | 3.96 | 3.10 | 2.43 |
|  | 5%-tile |  | NaN | NaN | NaN | NaN | NaN | NaN |

## LEO600 interfering TN equipped with AAS antenna in Rural

| Required ACIR [dB] |  | Company | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 1.48 | 1.05 | 0.74 | 0.51 | 0.35 | 0.24 |
|  | 5%-tile |  | 4.99 | 3.42 | 2.33 | 1.56 | 1.04 | 0.69 |

## LEO600 interfering TN equipped with sector antenna in Rural

| Required ACIR [dB] |  | Company | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 2.86 | 1.95 | 1.32 | 0.89 | 0.60 | 0.41 |
|  | 5%-tile |  | NaN | NaN | NaN | NaN | NaN | NaN |

## LEO600 interfering TN equipped with AAS antenna in Urban

| Required ACIR [dB] |  | Company | 24 | 26 | 28 | 30 | 32 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 1.98 | 1.52 | 1.15 | 0.88 | 0.66 | 0.50 |
|  | 5%-tile |  | 9.32 | 7.07 | 5.44 | 4.16 | 3.17 | 2.41 |

## LEO600 interfering TN equipped with sector antenna in Urban

| Required ACIR [dB] |  | Company | 10 | 12 | 14 | 16 | 18 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 10.72 | 8.28 | 6.41 | 4.99 | 3.91 | 3.07 |
|  | 5%-tile |  | NaN | NaN | NaN | NaN | NaN | NaN |

Proposal 1: It is proposed to include the above co-ex results from our company to the TR 38.863, and also to consider the above results from our company when discussing the required ACIR and ACLR/ACS.

# Conclusion

In this document, we submit our co-ex study results for discussion and considerations:

Proposal 1: It is proposed to include the above co-ex results from our company to the TR 38.863, and also to consider the above results from our company when discussing the required ACIR and ACLR/ACS.

# Reference

[1]. R4-2120671, Simulation assumptions for NTN co-existence study, Moderator.

[2]. R4-2120749, NR-NTN co-ex results template, Moderator.