3GPP TSG-RAN WG4 Meeting #101-bis-e			  									   	 R4-2201078 Electronic Meeting, January 17-25, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 38.863 on HAPS coexistence study

Agenda item: 	6.13.2.2

Document for:	Approval

# Introduction

The discussions in NTN coexistence study during RAN4#101-e are summarized in [1], while the WF for NTN coexistence is captured in [2]. The current draft of TR 38.863 can be found in [3].

This contribution proposes additions of text in the aspect of HAPS coexistence simulation to the TR draft according to work-split agreed at RAN4#101 [4]. The text proposal is based on the agreed simulation assumptions [5] with a small number of TBD values/options marked by [ ].

# Text proposal

******************************* Start of TP ***************************************

# 2	Reference

[xx]		SoftBank, Loon LLC, Nokia, Ericsson, “Proposed deployment and system characteristics of HIBS in the working document towards a preliminary draft new Report ITU-R M.[HIBS-CHARACTERISTICS],” ITU WP-5D contribution, Sep. 28, 2020

**************************** Unchanged Section Omitted ****************************

6	Co-existence study

## 6.1	Co-existence simulation scenario

**************************** Unchanged Section Omitted ****************************

The frequency and bandwidth are listed in table 6.1-3.

Table 6.1-3.  Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Rural | 2 GHz | 20MHz | FDD, TDD | 1 |
| TN Urban macro | 2 GHz | 20MHz | FDD, TDD | 1 |
| GEO | 2 GHz | 5/10/15/20 MHz for FR1 | FDD | 1, 31 |
| LEO | 2 GHz | 5/10/15/20 MHz for FR1 | FDD | 1, 31 |
| HAPS | 2 GHz | 20MHz | FDD | 1 |
| Note 1: 2 phases will be considered for FRF: FRF=1 in phase 1 for simplification; FRF=3 in phase 2 or it is found FRF=1 is too stringent. |  |  |  |  |

## 6.2	Co-existence simulation assumption

### 6.2.1	Network layout model

**************************** Unchanged Section Omitted ****************************

#### 6.2.1.2	Co-existence between HAPS and TN

For simulations of HAPS and TN co-existence, a cluster of TN cells is randomly dropped in the HAPS coverage area as shown in Figure 6.2.1.2-1. The TN cell cluster consists of 19 sites, 57 sectors, with the same assumption described in Section 6.2.2.4. HAPS coverage and cell layout are described in Section 6.2.2.3.

Figure 6.2.1.2-1 Simulation layout between HAPS and TN

#### 6.2.1.3	Co-existence between HAPS and HAPS

HAPS and HAPS co-existence can be characterized by “center-to-center” inter-system distance (i.e., the distance between two coverage centers) as depicted in Figure 6.2.1.2-2. Since HAPS location is at the center of its coverage area, the inter-system distance is the separation of the two HAPS. Different inter-system distances may be simulated to evaluate the interference caused by antenna gain variation in the elevation domain.

Figure 6.2.1.2-2 Simulation layout between HAPS and HAPS

### 6.2.2	System parameters

**************************** Unchanged Section Omitted ****************************

#### 6.2.2.3	HAPS parameters

Using the antenna model in Section 6.2.3.3, HAPS deployed at 20 km altitude has a coverage radius of 100 km in a 7-cell layout as shown in Figure 6.2.2.3-1. The UEs in the HAPS system are assumed to be outdoor and uniformly distributed in the coverage area in the rural environment. HAPS system parameters for the co-existence study are listed in Table 6.2.2.3-1.

Figure 6.2.2.3-1 HAPS cell layout [Final layout TBD]

Table 6.2.2.3-1 HAPS system parameters

| HAPS altitude | 20 Km |
| --- | --- |
| Carrier frequency | 2 GHz |
| Duplex scheme | FDD |
| Channel bandwidth | 20 MHz |
| Subcarrier spacing (SCS) | 15 KHz |
| Number of cells | 7 |
| Coverage area (7 cells combined) | A 100 Km radius circular area centered by the serving HAPS |
| Frequency reuse | 1 |
| Environment (Note1) | Rural |
| UE distribution | Uniformly distributed in the coverage area |
| Indoor UE percentage | 0% |
| Number of DL scheduled UEs per cell | 1 |
| Number of UL scheduled UEs per cell | [3] |
| DL scheduled bandwidth per UE | 20 MHz |
| UL scheduled bandwidth per UE | [6 RBs] |
| DL power control | No |
| UL power control | See section 6,2.6.3 |
| HAPS antenna | See section 6.2.3.3 |
| HAPS noise figure | 5 dB |
| HAPS-UE pathloss model | See section 6.2.5.4 |
| HAPS UE assumption | Same as TN UE in Table 6.2.2.4-1 |
| Note 1: HAPS is assumed to serve UEs in the rural environment, but the co-existed TN may be in the rural or urban macro environment. |  |

### 6.2.3	Antenna and beamforming pattern modelling

**************************** Unchanged Section Omitted ****************************

#### 6.2.3.3	HAPS antenna model

HAPS antenna model is based on the proposal for HIBS (HAPS as IMT base stations) study in ITU WP-5D [xx]. The antenna array is composed of seven antenna panels (six side panels and one downward facing panel) as shown in Figure 6.2.3.3-1. Antenna elements on each panel are co-phased to form one beam in two crossed linear polarizations to serve one cell. There are a total of seven cells in two layers, one cell in the 1st layer and six cells in the 2nd layer. The antenna parameters are listed in Table 6.2.3.3-1. Note that a fixed beam in the direction of the panel boresight is formed by each panel to serve one cell. Figure 6.2.3.3-1 shows the antenna gain of the 1st layer cell and a 2nd layer cell projected on the ground.

#### Figure 6.2.3.3-1 HAPS antenna model

Table 6.2.3.3-1 HAPS antenna parameters

| Number of cells | 7 |
| --- | --- |
| Antenna array configuration (row x column) | 2 x 2 for 1st layer cell 4 x 2 for 2nd layer cell |
| Antenna polarization | Linear ±45° |
| Element gain (Note 1) | 7.8 dBi |
| Element HPBW horizontal/vertical | 65⁰ for both H/V |
| Element front-to-back ratio horizontal/vertical | 30 dB for both H/V |
| Element spacing horizontal/vertical | 0.7 wavelength for both H/V |
| Antenna panel tilt (from the horizon) (Note 2) | 90⁰ for 1st layer cell 23⁰for 2nd layer cell |
| EIPR/cell | 56.8 dBm (1st layer cell), 59.8 dBm (2nd layer cell) |
| EIRP spectral density/cell | 43.8 dBm/MHz (1st layer cell), 46.8 dBm/MHz (2nd layer cell) |
| Tx power per antenna panel | 43 dBm |
| Conducted power (before ohmic loss) per antenna element (dBm) | 34 dBm for 2 x 2 (x 2 polarizations for 1st layer cell) 31 dBm for 4 x 2 (x 2 polarizations for 2nd layer cell) |
| Antenna weight (Note 3) wm,n | 12 for each of 1st layer cell element 122 for each of 2nd layer cell element |
| Polarization gain for SINR | 3 dB |
| Note 1: The element gain includes the Ohmic loss of 2 dB. Note 2: This is mechanical tilt only. Note 3: These antenna weights create a fixed cell beam in the antenna panel’s boresight direction. |  |

| (a) | (b) |
| --- | --- |

Figure 6.2.3.3-2 HAPS antenna gain (in dB) in a 100 km radius area on the ground. (a) Antenna gain of the 1st layer cell. (b) Antenna gain of a 2nd layer cell.

**************************** Unchanged Section Omitted ****************************

### 6.2.5	Propagation model

**************************** Unchanged Section Omitted ****************************

#### 6.2.5.4	Propagation model between HAPS and UE

Propagation model between HAPS and UE, regardless of whether the UE is connected to HAPS or TN, should reference to section 6.6 in TR 38.811.

**************************** Unchanged Section Omitted ****************************

### 6.2.6	Transmission power control model

**************************** Unchanged Section Omitted ****************************

#### 6.2.6.4	HAPS UL TPC

HAPS uplink follows the TPC model in Section 6.2.6.1 with X = [1.08] (scheduled bandwidth [6] RBs) and Y = 5 (HAPS noise figure 5 dB).

#### 6.2.6.5	DL TPC

For downlink scenario, no power control scheme is applied.

******************************* End of TP ***************************************

# References

- R4-2120741, Email discussion summary for [101-e][310] NTN_Solutions_Part2, RAN4#101-e

- R4-2120670, WF on [310] NTN_Solutions_Part2, RAN4#101-e.

- R4-2120776, Draft TR 38.863, RAN4#101-e.

- R4-2120670, WF on [310] NTN_Solutions_Part2, Samsung

- R4-2120672, Simulation assumptions for HAPS co-existence, RAN4#101-e.