3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2201118

Electronic Meeting, Jan. 17th – 26th, 2021

Title:	TP to TR 38.717-03-02: Addition of CA_n2-n29-n30

Source:	Nokia, AT&T

Agenda item:	5.11.1

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-03-02 to add CA_n2-n29-n30 with 2UL.

************************************* Start of TP*****************************************

### 5.1.x	CA_n2-n29-n30

#### 5.1.x.1		Operating bands for CA

Table 5.1.x.1-1: Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n2A-n29A-n30A | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n29 | N/A |  |  | 717 MHz | – | 728 MHz | SDL |
|  | n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |

#### 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n29A-n30A | CA_n2A-n30A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n29A-n30A | CA_n2A-n30A | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.1.x.3	UE co-existence studies

For 3DL/2UL NR CA, only the IMD issues due to dual uplink operation of two bands falling into the DL of the third band shall be verified.

The co-existence studies for dual uplink operation of two bands is therefore limited to 2UL in CA_n2A-n30A from TR 38.717-02-01, since n29 is SDL.

- No IMD products produced by Band n2 and Band n30 impact the reference sensitivity of NR band n29.

#### 5.1.x.4	REFSENS requirements

There is no impact on the REFSENS requirements.

************************************* End of TP*****************************************