3GPP TSG-RAN WG4 Meeting # 101-bis-e							      R4- 2201126

Electronic Meeting, January 17-25, 2022

Agenda item:			6.13.2.3

Source:	Samsung

Title:	NR-NTN co-ex results analysis and ACLR ACS proposal

Document for:	Approval

# Introduction

In the document R4-2201072, we proposed our ACIR study results for co-ex case 3 and case 4. And in this document, we propose the methodology to conclude the required ACIR from all the studied results and to derive the ACLR for NTN BS and UE.

# Methodology to conclude the required ACIR

Following the updates in last RAN4 101-e meeting, the latest agreements about simulation assumptions for NTN, we further updated our simulator and proposed the following results for different scenarios in case 3 and case 4.

## Methodology for results analysis

Since the meeting has agreed to consider the different factors, such as TN antenna type (AAS, non-AAS), TN deployed scenario (Rural, Urban), NTN deployed scenario (GEO, LEO-600, LEO-1200), NTN BS location (90-degree, 45-degree), etc., the co-ex study results varies from different combinations of the above conditions.

Considering the objective of these studies is to determine the ACLR or ACS for the NTN BS and UE to make sure the adjacent channel deployment of NTN will not cause un-acceptable interference to the existing TN network, we suggest to first review all the results for each case and to eliminate or skip those case which are not representing the worst co-ex scenario between NTN and TN.

Proposal 1: It is proposed to consider following methodology to determine the required ACIR for each case:

Step 1: Discuss and agree on the worst case option for each case (Case 1, 2, 3…,6);

Step 2: Discuss and determine the required ACIR from all calibrated results for each case;

Step 3: Use equation to derive corresponding ACLR or ACS from the agreed ACIR for each case.

## Adopt the methodology to analyse our results

By reviewing all the results from our company, we identified the following option as the worst-case option for Case 3 and Case 4.

For Case 3, the worst case is:

LEO600 interfering TN equipped with AAS antenna in Rural

| Required ACIR [dB] |  | Company | 18 | 20 | 22 | 24 | 26 | 28 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 5.52 | 3.72 | 2.45 | 1.60 | 1.03 | 0.66 |
|  | 5%-tile |  | 13.13 | 8.76 | 5.73 | 3.71 | 2.37 | 1.51 |

For Case 4, the worst case is:

GEO interfering TN equipped with AAS antenna in Urban

| Required ACIR [dB] |  | Company | 24 | 26 | 28 | 30 | 32 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average | Samsung | 2.00 | 1.53 | 1.17 | 0.89 | 0.67 | 0.50 |
|  | 5%-tile |  | 9.44 | 7.15 | 5.47 | 4.19 | 3.19 | 2.44 |

Proposal 2: It is proposed to consider our derived interpolate required ACIR for Case 3 and Case 4. For Case 3, our results suggested 22.72 dB as required ACIR; for Case 4, our results suggested 28.73 dB as required ACIR.

With the equation and the agreed TN ACLR and ACS values below, we can derive the required ACLR or ACS for NTN directly.

Table ACLR/ACS for TN (2GHz)

|  |  | NR | NB-IOT |
| --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB |
|  | ACS | 46 dB | 46 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) | 37 |
|  | ACS | 33 | 28 |

For Case 3, the aggressor is NTN BS while the victim is the TN UE. Given the required ACIR for case 3 is 22.72 dB and TN UE ACS is 33 dB, the required ACLR for NTN BS is 23.15 dB.

For case 4, the aggressor is NTN UE while the victim is the TN BS. Given the required ACIR for case 4 is 28.73 dB and TN BS ACS is 46 dB, the required ACLR for NTN UE is: ACLR1 as 28.81 dB, ACLR2 as 41.81 dB.

Proposal 3: It is proposed to consider our derived ACLR and ACS for Case 3 and Case 4. For Case 3, our results suggested 23.15 dB as NTN BS ACLR; for Case 4, our results suggested 28.81 dB as ACLR1 and 41.81 dB as ACLR2 for NTN UE ACLR.

## Summary

Considering the above results of case 3 and 4, we find out the worst interfering scenarios and their corresponding required interpolated ACIR are shown below.

| Case # | Scenario | Worst case option | Required ACIR | TN | NTN |
| --- | --- | --- | --- | --- | --- |
| Case 3 | NTN DL to TN DL | LEO 600 interfering TN equipped with AAS antenna in Rural | 22.72 | TN UE ACS 33dB | NTN BS ACLR 23.15 |
| Case 4 | NTN UL to TN UL | GEO interfering TN equipped with AAS antenna in Urban | 28.73 | TN BS ACS 46dB | NTN UE ACLR1 28.81 ACLR2 41.81 |

# Conclusion

In this document, we proposed the methodology to post-processing and analyse the NTN co-ex results. And also, we adopt the methodology to process our results and then proposed the required ACLR for NTN BS and UE of Case 3 and Case 4.

Proposal 1: It is proposed to consider following methodology to determine the required ACIR for each case:

Step 1: Discuss and agree on the worst case option for each case (Case 1, 2, 3…,6);

Step 2: Discuss and determine the required ACIR from all calibrated results for each case;

Step 3: Use equation to derive corresponding ACLR or ACS from the agreed ACIR for each case.

Proposal 2: It is proposed to consider our derived interpolate required ACIR for Case 3 and Case 4. For Case 3, our results suggested 22.72 dB as required ACIR; for Case 4, our results suggested 28.73 dB as required ACIR.

Proposal 3: It is proposed to consider our derived ACLR and ACS for Case 3 and Case 4. For Case 3, our results suggested 23.15 dB as NTN BS ACLR; for Case 4, our results suggested 28.81 dB as ACLR1 and 41.81 dB as ACLR2 for NTN UE ACLR.

# Reference

[1]. R4-2120671, Simulation assumptions for NTN co-existence study, Moderator.

[2]. R4-2120749, NR-NTN co-ex results template, Moderator.

[3]. R4-2201072 NR-NTN co-ex study results, Samsung.