3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201191

Electronic Meeting, January 17-25, 2022

Title: 	Discussion on RRM measurement relaxations for RedCap UE

Source: 	Huawei, HiSilicon

Agenda item:	6.20.3.3

Document for:	Discussion

Introduction

The revised WID on support of reduced capability (RedCap) NR devices has been approved in [1]. RAN4 has started work on the support of RedCap NR devices, where the related study item has been concluded in TR 38.875. The WID has the following objectives on introduction of RRM measurement relaxations:

- Specify support for the following RRM measurement relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected [RAN2, RAN4]: - Specify RSRP/RSRQ based stationary criterion, which is based on Rel-16 low mobility criterion, for RRM measurement relaxation [RAN2] - For RRC_Idle/Inactive, the stationary criterion allows the UE to perform RRM measurement relaxation when fulfilled. - For RRC_Connected, the stationary criterion triggers the UE to send a report to the gNB when fulfilled. - Enabling/disabling of RRM measurement relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM measurement relaxation. - Specify provision of thresholds for the Rel-16 not-at-cell-edge criterion, alternatively rely on the existing thresholds [RAN2] - Specify RRM measurement relaxation [RAN4] - No RRM measurement relaxations are specified for the serving cell.

This contribution provides further analysis on the RRM measurement relaxation for RedCap UE.

Discussion

RRM relaxation in idle and inactive mode

- Relaxation when R17 stationary criterion is satisfied

RAN2 has the following agreements in RAN2#114e [2].

Agreements in RAN2#114-e:

•	Reuse R16 low mobility criterion, as part or whole of Rel-17 stationary criterion in RRC_IDLE/INACTIVE. When NW configures both Rel-17 stationary criterion and Rel-16 low mobility criterion, NW configures different Rel-17 thresholds (i.e., SSearchDeltaP_stationary/TSearchDeltaP_stationary) from Rel-16 (SSearchDeltaP / TSearchDeltaP). How to configure the criterion (e.g. more stringent) is left to NW implementation (i.e. no specification impact to RAN2).

It is observed that network configures different threshold for stationary criterion from R16 low mobility criterion. In R16 power saving, when only low mobility criterion is satisfied, fixed relaxation factor (e.g., 3) is applied. In our understanding, the relaxation for stationary scenario is supposed to be more “relaxed” than low mobility scenario. Therefore the measurement relaxation factor when stationary criterion is satisfied can be larger than low mobility. Furthermore to simplify UE implementation, the relaxation factor shall be fixed.

Proposal 1: When stationary criteria is fulfilled, RRM measurement relaxation with larger fixed scaling factor than R16 low mobility is applied, Kstationary>3.

According to RAN2 understanding, there is no motivation to introduce a separate R17 not-at-cell-edge criterion for RedCap UE as not-at-cell-edge criterion is already defined in R16. Separate Rel-17 not-at-cell-edge threshold is introduced which is only configured together with R17 stationary criterion. The agreements in RAN2 are duplicated as below[2].

Agreements in RAN2#115-e: - Introduce separate Rel-17 not-at-cell-edge threshold, and the new threshold is only associated with Rel-17 stationary criterion (if configured). If configured with a not-at-cell-edge criterion, the R17 stationary criterion can only be configured together with the R17 not-at-cell-edge criterion, not with the R16 one.

This means that there is no separate R17 not-at-cell-edge criterion. In other words, there are two criteria for R17 Redcap measurement relaxation:

- Relaxation criterion 1: R17 stationary criterion

- Relaxation criterion 2: R17 stationary and R17 not-at-cell-edge criterion

Proposal 2: There is no separate R17 not-at-cell-edge criterion.

- Relaxation when R17 stationary and R17 at-at-cell edge criterion is satisfied

When R17 stationary and R17 at-at-cell edge criterion (Relaxation criterion 2) is satisfied, UE can stop measurement for a longer interval, e.g., more than 1 hour.

Proposal 3: When R17 stationary and R17 at-at-cell edge criterion is fulfilled, UE can stop measurement for a longer interval, e.g., more than 1 hour.

- Relaxation when mixed R17 criterion and R16 criterion are satisfied

Table 1 lists the cases where UE may fulfil mixed R16 and R17 criteria. It shall be noted that any combination of R17 not-at-cell-edge only and R16 relaxation criterion is not valid.

Table 1. UE fulfils mixed R16 and R17 criteria

| #Case | UE fulfils R17/R16 mixed criteria |  |
| --- | --- | --- |
| #1 | R-17 stationary | R16 low mobility |
| #2 | R-17 stationary | R16 not at cell edge |
| #3 | R-17 stationary | R16 low mobility& R16 not at cell edge |
| #4 | R-17 stationary& R17 not at cell edge | R16 not at cell edge |
| #5 | R-17 stationary& R17 not at cell edge | R16 low mobility |
| #6 | R-17 stationary& R17 not at cell edge | R16 low mobility& R16 not at cell edge |

In RAN2 #114e, there are two options on how to define Rel-17 not-at-cell-edge criterion [2]. It shall be noted that no separate Rel-17 not-at-cell-edge criterion is allowed. Rel-17 not-at-cell-edge criterion is only associated with Rel-17 stationary criterion.

- Continue discussion on Rel-17 not-at-cell-edge criterion in RRC_IDLE/INACTIVE within two options: - Option 1) Reuse Rel-16 not-at-cell-edge criterion with the same thresholds (i.e., SSearchThresholdP / SSearchThresholdQ) - Option 2) Reuse Rel-16 not-at-cell-edge criterion with the different thresholds

In RAN2 #115e, the conclusion was reached to introduce separate Rel-17 not-at-cell-edge threshold. It means that option 1) in RAN2 is precluded. It also implicitly means that the mixed combination of R-17 stationary and Rel-16 not-at-cell-edge criterion is not valid. Therefore case #2 in Table.1 is supposed not to be considered.

For other mixed cases, RAN2 decided to postpone the discussion and wait for RAN4’s conclusion [2].

Agreements in RAN2#114-e: - Postpone the following discussion until RAN4 defines RRM relaxation method for Rel-17: When NW configures both R16/R17 relaxation criteria and the UE fulfills both, UE performs: - Option 1) UE performs Rel-17 RRM relaxation method - Option 2) It is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation

From RAN4 perspective,

- For case#1, measurement requirements when R-17 stationary criterion is fulfilled are supposed to be relaxed than that of R16 low mobility.

- For case#3, in our understanding, a larger factor Kstationary may be applied for the measurement relaxation requirements when R-17 stationary criterion is satisfied, where Kstationary >Klow_mobility. However we think the relaxed measurement period for R-17 stationary criterion may be still smaller than 1hour.

- For case #4, #5, #6, in our understanding, the measurement requirements when UE fulfils R-17 stationary& R17 not-at-cell-edge criterion are the most relaxed.

Based on the above discussion, if UE selects the more relaxed requirements when both mixed R16/R17 relaxation criteria are fulfilled, the requirements will be distinguished case by case (see the green highlight in Table 1). It is, to some extent, complicated to specify requirements. From UE perspective, whether UE would like to choose the relaxed requirements is up to UE implementation. It is better to provide more room for implementation freedom. Therefore option 2) in RAN2 discussion, i.e., it is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation, is a better way.

Proposal 4: When UE fulfils both mixed R16 and R17 relaxation criteria, it is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation.

RRM relaxation in connected mode

For the RRM relaxation in connected mode, RAN2 has reached the below agreements.

Agreements:

- An RSRP/RSRQ based stationarity criterion (Working Assumption: the same as in idle/inactive) can be configured for UEs in RRC Connected. If the criterion is met, this is reported to the network (FFS how/when). It is FFS whether, based on this, besides possibly reconfiguring RRM measurements (up to network implementation), the network can enable RRM measurement relaxation (FFS whether same method as in Idle/Inactive)

Agreements online: (RAN2#116e) - RAN2 assume that the existing RRM measurement framework can be used as baseline for enabling and disabling RRM relaxations for UEs in RRC Connected. Other methods can be considered too based on relaxation methods agreed by RAN4. - RAN2 understands that no prohibit timer is needed, if legacy measurement reporting framework is reused by UE to report its relaxation status - The granularity of RRM measurement relaxations (i.e. whether it should be specified per beam, per cell or per frequency) should be handled by RAN4

In RRC connected mode, if the stationary criterion is met, UE reported the information to network. Enabling/disabling of RRM measurement relaxation is under network’s control.

According to RAN2 agreement, the existing RRM measurement framework can be used as baseline for enabling and disabling RRM relaxations for UEs in RRC Connected. In other words, network can configure related parameters to relax UE’s measurement if needed. For example, network can configure long SMTC periodicity or large DRX cycle length to UE. The agreements in RAN2 also mean that from RAN2 perspective, the measurement relaxation in connected mode is completed.

Furthermore for Redcap UE, typically the duration UE in connected mode is short. Redcap UE can quickly enter to RRC idle/inactive mode to save more power. The strong justification of introducing new relaxation methods is not foreseen.

If new UE behaviour of RRM relaxation measurement is introduced, UE is allowed to skip some SSB measurement occasions, however the RLM/BFD procedure is still performed on the same SSB symbols, the power saving resulting from only relaxation on RRM measurement is limited.

Consequently no new UE behaviour of RRM measurement relaxation is needed for RedCap UE in connected mode.

Proposal 5: No new UE behaviour of RRM measurement relaxation is needed for RedCap UE in connected mode.

Conclusions

This contribution provides our analysis on RRM measurement relaxation for RedCap UE. The following proposals are provided:

Proposal 1: When stationary criteria is fulfilled, RRM measurement relaxation with larger fixed scaling factor than R16 low mobility is applied, Kstationary>3.

Proposal 2: There is no separate R17 not-at-cell-edge criterion.

Proposal 3: When R17 stationary and R17 at-at-cell edge criterion is fulfilled, UE can stop measurement for a longer interval, e.g., more than 1 hour.

Proposal 4: When UE fulfils both mixed R16 and R17 relaxation criteria, it is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation.

Proposal 5: No new UE behaviour of RRM measurement relaxation is needed for RedCap UE in connected mode.

Reference

- RP-211038, Revised WID on support of reduced capability NR devices, Ericsson

- R2-2109197, LS on RRM Relaxations for RedCap, RAN2