3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2201192

Electronic Meeting, January 17-25, 2022

Title: 	Discussion on NCD-SSB and CSI-RS

Source: 	Huawei, HiSilicon

Agenda item:	6.20.3.4

Document for:	Discussion

Introduction

In last meeting, RAN4 replied LS on use of NCD-SSB for RedCap UE [1]. This paper provides further considerations on NCD-SSB based measurement. In addition, RAN4 received another LS on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE [2]. This paper also provides some analysis on the work assumption of CSI-RS.

Discussion

- RRC IDLE/INACTIVE state

In RAN#94-e, it is agreed that RedCap UE in IDLE and INACTIVE monitors paging in an initial BWP associated with CD-SSB in Rel-17, and idle/inactive mobility is only based on measurements on the CD-SSB. These procedures in IDLE /INACTIVE are same as legacy ones. Therefore the legacy measurement requirements in IDLE/INACTIVE state can be reused for RedCap.

Proposal 1: Legacy measurement requirements in IDLE/INACTIVE state can be reused for RedCap.

- RRC CONNECTED state

Both RAN1 and RAN4 have conclusion on the periodicity of NCD-SSB. Taking together the below agreements, the periodicity of NCD-SSB shall be larger than periodicity of CD-SSB.

RAN4#101e agreement - Periodicities of NCD-SSB are up to network configuration and can be same or different from those of CD-SSB, if both NCD-SSB and CD-SSB are transmitted on the serving cell of RedCap UE. Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB.

RAN1#107b-e Agreement:

Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB

Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB

For RedCap UEs, UE may work on a BWP without CD-SSB. With the introduction of NCD-SSB, RedCap UEs can directly evaluate the serving cell quality based on NCD-SSB without retuning to CD-SSB under the measurement gap. Therefore NCD-SSB capable UE can perform serving cell RRM measurement based on NCD-SSB in connected mode.

Proposal 2: Support RRM measurement on serving cell based on NCD-SSB in connected state.

From NCD-SSB configuration perspective, it is not possible to guarantee all NCD-SSB in serving cell and neighbour cells are aligned in frequency domain. One example is presented in Figure 1 where NCD-SSB1 is in active BWP1, neighbour cell 1 has NCD-SSB in the same frequency position but neighbour cell 2 doesn’t have. If UE performs cell detection and measurement on NCD-SSB1, although no measurement gap is needed, neighbour cell2 is not able to be identified. It means that the cells observed on NCD-SSB are only a sub-set of all neighbour cells. If UE could not identify a full set of the neighbour cells, there is risk for mobility performance. For example, if UE is moving to neighbour cell2, but this cell is not detectable by the UE, the handover would face enormous risk, and the case shall be highly not expected. Therefore to guarantee UE can observe full set of neighbour cells and avoid mobility performance degradation, RRM measurement on neighbour cell is supposed to be performed on CD-SSB. In other words, do not support neighbour cell measurement based on NCD-SSB in connected state.

Figure 1. NCD-SSB topo

Proposal 3: For RedCap UE, RRM measurement on neighbour cell is supposed to be performed on CD-SSB in connected mode.

- Working assumption on CSI-RS

In RAN1#107-e, RAN1 has made the following agreements and working assumptions related to DL BWP operation for RedCap UEs in [2]. RAN2 and RAN4 are request to provide feedback on below highlighted RAN1 working assumption. As the working assumption on paging belongs to RAN2 scope, RAN4 focus on CSI-RS assumption.

Agreement: - For both FR1 and FR2, for a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. At least the case when the separate initial DL BWP includes CD-SSB and the entire CORESET#0 is supported - It can be used in idle/inactive mode (including paging) and during and after initial access, when applicable - It is no wider than the maximum RedCap UE bandwidth. - This applies to both TDD and FDD (including FD FDD and HD FDD) cases. Agreement: - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. - Note: if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB Agreement: - For FR2, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. - Note: For SSB and CORESET#0 multiplexing pattern 1, if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB

It shall be noted RAN1 already agreed that for an RRC-configured active DL BWP in connected mode, a RedCap UE can indicate optional capability which represents “not need NCD-SSB”. How to represent “not need NCD-SSB” is what RAN4 need to be discussed. As FG 6-1a has reached consensus in RAN1, the key point of the discussion is whether CSI-RS capability is needed or not.

From timing perspective, although the legacy requirements for timing are based on SSB, it doesn’t mean it is not feasible to perform CSI-RS/TRS based fine time tracking. In NR initial discussion phase, the simulation evaluations in RAN1 had been verified that TRS based T/F tracking can achieve good performance. From RAN4 perspective, as long as the bandwidth of TRS is not less than SSB bandwidth, the existing Te, Tq requirements can be guaranteed by TRS based T/F tracking.

CSI-RS based RLM, BFD and L1-RSRP requirements are already specified in TS38.133. And supporting CSI-RS based layer 1 related procedure has respective UE capability.

For L3 measurement, as FG 6-1a is supported by UE, UE can perform SSB based L3 measurement with RF retuning. As UE would like to report FG6-1a, it means that the possible impact due to RF retuning can be acceptable to UE.

bwp-WithoutRestriction Indicates support of BWP operation without bandwidth restriction. The Bandwidth restriction in terms of DL BWP for PCell and PSCell means that the bandwidth of a UE-specific RRC configured DL BWP may not include the bandwidth of CORESET #0 (if configured) and SSB. For SCell(s), it means that the bandwidth of DL BWP may not include SSB.

Alternative of L3 measurement is based on CSI-RS. As discussed in last meeting, CSI-RS are not used as a standalone mechanism for RRM measurements and the existing requirements rely on the presence of SSB signals. As L3 CSI-RS measurement resources are associated with SSB, Redcap UE which also support FG 6-1a can switch to CD-SSB with gap for cell detection, and performs CSI-RS measurement on its active BWP without retuning.

Based on the above analysis, a RedCap UE can optionally support relevant operation based on CSI-RS and FG 6-1a by reporting optional capabilities.

Proposal 4: A RedCap UE can indicate not need NCD-SSB: a RedCap UE can in addition optionally support relevant operation based on CSI-RS and FG 6-1a by reporting optional capabilities.

Conclusions

This contribution provides our analysis on NCD-SSB related open issues. The following proposals are provided:

Proposal 1: Legacy measurement requirements in IDLE/INACTIVE state can be reused for RedCap.

Proposal 2: Support RRM measurement on serving cell based on NCD-SSB in connected state.

Proposal 3: For RedCap UE, RRM measurement on neighbour cell is supposed to be performed on CD-SSB in connected mode.

Proposal 4: A RedCap UE can indicate not need NCD-SSB: a RedCap UE can in addition optionally support relevant operation based on CSI-RS and FG 6-1a by reporting optional capabilities.

Reference

- RP-211038, Revised WID on support of reduced capability NR devices, Ericsson

- R1-2112802, LS on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE