3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201213

Electronic Meeting, January 17-25, 2022

Agenda Item:	6.11.2.2

Source:	Ericsson

Title:	Discussion on multiple concurrent MG patterns

Document for:	Discussion

# Introduction

In last RAN4 meeting, a WF for multiple concurrent and independent gap patterns was approved [1]. There is a reply LS [2] from RAN2 which also have some further questions need to be clarification.

In this contribution, we continue to discuss the concurrent gaps requirement.

# Applicability and configurations

In last meeting, the remaining issues is whether to allow concurrent gap in the case when only E-UTRAN measurement objectives are configured.

Issue 2-1-1: Whether concurrent gaps are allowed in the case when only E-UTRAN measurement objectives are configured - Open issue - Option 1: Yes - Option 2: No - Option 3: No need to further discuss - Option 4: Yes, provided that UE supports LTE measurement with concurrent MGs, which is Up to UE capability - Companies are encouraged to provide reasons for the benefits or difficulty to support this configuration.

This issue is similar as whether to limit the NW’s configuration on the type of measurement objectives. From our understanding, it’s up to NW to configure one gap or two gaps for E-UTRAN measurements once UE supports concurrent gaps. Furthermore, if UE supports concurrent gaps, there is no technical reason to NOT support two inter-RAT measurements.

Proposal 1: It’s up to NW to decide whether to configure only E-UTRAN RAT measurement objectives.

# UE capability related issues

In last meeting, one of the remaining issues related UE capability is whether supporting simultaneous configuring per-UE gap and per-FR gap.

Issue 2-2-1: Whether to allow simultaneous configuration of per-UE gap and per-FR gap to FR gap capable UEs - Open issue - FFS the use case of simultaneous configuring per-UE gap and per-FR gap. Consider the identified use cases to make decision in RAN4#101b-e meeting.

Some companies argue that only considering the per-UE gap for positioning because no other use case for per-UE gap and per-FR gap. However, in Rel-17, MU-SIM gaps are also introduced in RAN2 which are also only per-UE gap. From our understanding, MU-SIM gap can be seen as one type of gaps within the concurrent gaps framework. Thus, RAN4 should at least consider per-UE gap and per-FR gap scenarios for positioning and MU-SIM.

Scenarios and supported gap types … - Only per UE level scheduling gap is supported in Rel-17 for non-DC. FFS if we support MR-DC. - Do not support autonomous gaps for MUSIM in Rel-17.

In addition, we don’t see any technique obstacle for UE to support per-UE gap and per-FR gap simultaneously when UE supports both per-FR gap and concurrent gaps. Therefore, to consider forward compatibility, UE can be configured with per-UE gap and per-FR gap simultaneously when UE is capable of per-FR gap and concurrent gaps.

Proposal 2: UE can be configured with per-UE gap and per-FR gap when UE is capable of per-FR gap and concurrent gaps.

Combination of per-UE gap and/or per-FR gap

In Rel-17, when UE is capable of per-FR gap and concurrent gaps, several use cases for different combinations of per-UE gap and/or per-FR gap has been discussed before. Thus, we propose to support the following gap combinations for concurrent gaps. Without considering MU-SIM and NTN gaps, the maximum number of supported concurrent gaps can be 4.

| Index | # of simultaneous MG |  |  | RAN4 conclusion |
| --- | --- | --- | --- | --- |
|  | Per-FR1 | Per-FR2 | Per-UE |  |
| 0 | 2 | 1 | 0 | Supported |
| 1 | 1 | 2 | 0 | Supported |
| 2 | 0 | 0 | 2 | Supported |
| 3 | 1 | 0 | 1 | Supported |
| 4 | 0 | 1 | 1 | Supported |
| 5 | 1 | 1 | 1 | Supported |
| 6 | 2 | 2 | 0 | Supported |
| 7 | 0 | 0 | 1 | Supported |
| 8 | 1 | 1 | 0 | Supported |
| 9 | 1 | 0 | 0 | Supported |
| 10 | 0 | 1 | 0 | Supported |

Proposal 3: When UE supports both per-FR gap and concurrent gaps, except the legacy gap combination, the combination of the per-UE gap and/or per-FR gap to be configured can be as follow.

| Index | # of simultaneous MG |  |  | RAN4 conclusion |
| --- | --- | --- | --- | --- |
|  | Per-FR1 | Per-FR2 | Per-UE |  |
| 0 | 2 | 1 | 0 | Supported |
| 1 | 1 | 2 | 0 | Supported |
| 2 | 0 | 0 | 2 | Supported |
| 3 | 1 | 0 | 1 | Supported |
| 4 | 0 | 1 | 1 | Supported |
| 5 | 1 | 1 | 1 | Supported |
| 6 | 2 | 2 | 0 | Supported |
| 7 | 0 | 0 | 1 | Supported |
| 8 | 1 | 1 | 0 | Supported |
| 9 | 1 | 0 | 0 | Supported |
| 10 | 0 | 1 | 0 | Supported |

Proposal 4: The max number of supported concurrent gap can be 4 when UE supports both per-FR gap and concurrent gaps without considering MUSIM and NTN gaps.

# Overlapping

Condition of overlapping

The proximity condition to apply the gap overlapping rule can base on the comparison between time difference of the ending point in time of one gap and the starting point in time of the other gap with a defined threshold. In last meeting, the agreed potential values are listed as follow.

Issue 2-3-1: Proximity condition for overlapping - Agreements (from GTW session on Nov 4th) - Two measurement gap occasions are defined as colliding (overlapping) if at least one of the following conditions apply - Condition #1: The gaps are physically fully or partially overlapping in time domain - Condition #2: The gaps are not physically overlapping in time domain but the minimal distance between the two gap instances is equal or less to X - X = 1 or 4 ms for FR1 - X = [1, 2, or 4] ms for FR2 - FFS if split between FR1/FR2 is needed

When RAN4 considers the values to define the proximity condition, two important technical aspects should be considered. On the one hand, the proximity condition shall guarantee UE to perform the consecutive measurements within two gaps which are close to each other. On the other hand, the proximity condition can avoid UE not to receive the DL or/and transmit the UL during a long period.

Furthermore, the value for proximity condition is highly related to the overlapping rule. The proximity condition will consider the scheduling limitation once data scheduling during dropped gap occasions is assumed. Thus, the values can be 4ms for FR1 and 1ms for FR2. However, if RAN4 agrees no data scheduling permitted during dropped gap occasions, to avoid too long outage, a shorter proximity condition which equals the upper bound of UE processing delay can be defined which is 1ms for both FR1 and FR2.

Proposal 5: The proximity condition X = 4ms for FR1 and 1ms for FR2 once data scheduling is assumed during dropped gap occasions.

Overlapping rule

In last meetings, some rules(priority/sharing/canceling) for overlapping instances were discussed. Two options are agreed to further study in this meeting.

Issue 2-3-2: UE behavior during colliding gap occasion - Open issue - Option 1: Priority rule - UE will only do the measurement w.r.t. the gap with higher priority on all colliding occasions - The priority can be configurable or fixed - FFS whether to resume data scheduling during dropped gap occasions - Option 5: Compromised proposal from moderator - Introduce gap sharing rule. - Request RAN2 to reserve some RRC signaling for different sharing factors. - The signalling design may consider the possibility of resuming data scheduling on dropped gaps - Rel-17 requirements will only consider sharing ratios 0% and 100%. - The requirements for other sharing factors are FFS in later releases. - FFS whether the resume scheduling on those dropped gaps as well as the impact to other intra-frequency measurements

From our understanding, it’s better to find a good compromise solution to consider the possible forward compatibility to introduce enhanced mechanism for gap overlapping, especially, when RAN4 considers a greater number of gaps will be supported in the future release, such as MU-SIM gaps and NTN gaps. Thus, the most important issue for gap overlapping is that both NW and UE should have the clear understanding of each gap where collision happens. On the one hand, UE can easily schedule the measurements based on the NW’s gap indication. On the other hand, NW can schedule the data on the unused gap occasion when collision happens.

When RAN4 defines the requirement based on the overlapping rule, some gap occasions will be disabled.  Data scheduling on the disabled gap occasions should be permitted since both NW and UE have the same understanding on which gap occasion should be disabled. More importantly, we can anticipate more and more gaps will be defined in the future release which will have a big impact to the system performance. Both UE and network’s performance will be degraded too much once data scheduling is not permitted when multiple gaps will be introduced. Thus, data scheduling must be permitted on the dropping gap occasions.

Proposal 6: Data scheduling is assumed on the dropping gap occasions.

Considering the pressure on WI timeline, RAN4 can only define requirement for UE to perform measurement in the gap with higher priority on all colliding occasions in Rel-17 and further consider the enhancement on gap overlapping issue in the future release which we can foresee more gaps will be introduced.

Proposal 7: As a compromised solution, RAN4 to only define requirement for UE to perform measurement in the gap with higher priority on all colliding occasions in Rel-17.

Currently, the traditional gap is applied for CONNECTED mode mobility, or positioning measurements. When additional new gaps will be introduced, such as MU-SIM gap or NTN gap, network can indicate the priority of the gap or gap group.  Current traditional MGs can be believed as a gap group for CONNECTED mobility; the new type of measurements, such as MU-SIM gaps can be believed as another gap group. Network can further indicate the gap overlapping rule based on the gap groups. For example, as we mentioned before, the gaps can be grouped by the usage configured from NW. The gap overlapping rule can be used to indicate which group of gaps shall be prioritized once overlapping happens. Thus, to consider forward compatibility with other type of gaps, NW needs to configure the priority indication together with the configured measurement gaps. For example, NW can configure one gap for legacy L3 mobility with priority as ‘1’, one gap for positioning with priority as ‘0’ and two gaps for MUSIM with priority as ‘2’. (‘0’ – the highest priority, ‘2’ – the lowest priority)

Figure 1: Gap overlapping between MG for L3 mobility and the new MGs for MU-SIM

Proposal 8: The concurrent gaps can support new type of gaps by indicating the gaps based on the usage, such as MU-SIM gaps.

Proposal 9: NW needs to configure each measurement gap together with the priority indication.

# Overhead

Whether to define an overhead cap is related to restrict the configuration from network side. On the one hand, network can manage this cap and tradeoff between the throughput loss and measurement gaps’ configuration. On the other hand, there is no significant throughput loss for UE compared with the legacy MG once data scheduling is assumed on the dropping gap occasions.

Proposal 10: RAN4 not to define an overhead cap for concurrent gaps.

# Transition period and measurement requirement

In last meeting, a discussion on UE’s behaviour after transition is on-going.

Issue 2-7-2: UE measurement behavior after transition - Open issue - Option 1: - The UE will continue and complete the ongoing measurement on MO1 using MGP1 and meet the corresponding measurement requirement based on MGP1 during this measurement period even if the MO1 is reconfigured to be measured using MGP2. - UE will perform the measurement on MO2 using MGP2 immediately after the concurrent gaps reconfiguration, if MO2 can’t be measured by MGP1 due to gap offset or if gap length is not enough. - After one of concurrent gaps deconfiguration, data scheduling is expected on this disabled MG’s time occasions. - Option 2: - FFS whether/how to define UE measurement behaviour after transition.

When UE previously performing measurement on MO1 using MGP1 will continue the measurement by MGP2 once the MO1 is configured to be measured in MGP2. In addition, the measurement will be performed within MG2 immediately after the concurrent gaps reconfiguration for the MO2 which could not be performed within MG1 but can be performed with MG2. Similarly, RAN4 should also discuss UE’s behaviour during the concurrent gap deconfiguration. Generally, when NW has received the measurement report or measurement gap release indication, NW will send the concurrent gap deconfiguration command. Thus, after concurrent gap deconfiguration, these on-going measurements should be stopped immediately. NW will schedule the data on the disabled MG’s time occasions after concurrent gap deconfiguration.

Proposal 11: The following UE behaviours after transition should be defined.

- UE will continue the measurement by MGP2 and meet the corresponding measurement requirement based on MGP2 during this measurement period once the MO1 is reconfigured to be measured using MGP2.

- UE will perform the measurement on MO2 using MGP2 immediately after the concurrent gaps’ reconfiguration, if MO2 can’t be measured by MGP1 due to gap offset or if gap length is not enough.

- After one of concurrent gaps deconfiguration, data scheduling is expected on this disabled MG’s time occasions.

# Requirements for concurrent gaps

CSSF within gap

An additional scaling factor Kgap due to gap overlapping rule shall be introduced. This factor can be directly multiplexed in the CSSF similar as legacy intra-frequency and inter-frequency sharing factor. The detail Kgap calculation can be as follow.

- When none of the MG occasions of the MGP with lower priority are overlapped by other MGP(s), Kgap = 1.

- When all of the MG occasions of the MGP with lower priority are overlapped by other MGP(s), the MGP with lower priority will be disabled.

- Otherwise, Kgap will be applied to the MG with lower priority and equals 11-MGRPhigh priorityMGRPlow priority.

Proposal 12: Additional scaling factor Kgap shall be introduced on top of CSSF within gap, where:

- When none of the MG occasions of the MGP with lower priority are overlapped by other MGP(s), Kgap = 1.

- When all of the MG occasions of the MGP with lower priority are overlapped by other MGP(s), the MGP with lower priority will be disabled.

- Otherwise, Kgap will be applied to the MG with lower priority and equals11-MGRPhigh priorityMGRPlow priority.

CSSF outside gap

The scaling factor Kp for measurement outside gap shall also be updated to consider the union of concurrent gaps.

- When none of the MG occasions of the MGP with lower priority are overlapped by other MGP(s), Kp= 11- i=1NSMTCMGRPi .

- When all of the MG occasions of the MGP with lower priority are overlapped by other MGP(s), the MGP with lower priority will be disabled.

- Otherwise,

  - When both MGs are overlapping with the SMTC, Kp= 11- SMTCMGRPlower priority

  - When MG with lower priority is overlapping with the SMTC, but MG with higher priority isn’t overlapping with the SMTC, Kp= 11- SMTCMGRPlower priority×Kgap

  - When MG with lower priority isn’t overlapping with the SMTC, but MG with higher priority is overlapping with the SMTC, Kp= 11- SMTCMGRPhigher priority

Proposal 13: The scaling factor Kp for measurement outside gap shall be updated as follow.

- When all the MG occasions of the MGP with lower priority are overlapped by other MGP(s), the MGP with lower priority will be disabled.

- When none of the MG occasions of the MGP with lower priority are overlapped by other MGP(s), Kp= 11- i=1NSMTCMGRPi .

- Otherwise,

  - When both MGs are overlapping with the SMTC, Kp= 11- SMTCMGRPlower priority

  - When MG with lower priority is overlapping with the SMTC, but MG with higher priority isn’t overlapping with the SMTC, Kp= 11- SMTCMGRPlower priority×Kgap

  - When MG with lower priority isn’t overlapping with the SMTC, but MG with higher priority is overlapping with the SMTC, Kp= 11- SMTCMGRPhigher priority

# Reply LS

In last RAN2 meeting, a LS for concurrent gaps was approved and send to RAN4. There are following questions in the LS:

In addition, RAN2 would like to ask the following questions. RAN2 signaling could ensure to always provide an association between Rel-17 concurrent MGs and the frequency layers to be measured (at least in NR SA). However, since legacy MG do not provide this association: Q1 – Can Rel-17 concurrent gaps be configured together with legacy gap? If ‘yes’, what would be the UE behavior? Q2 – How many concurrent gaps could be configured simultaneously? Q3 – Could concurrent gaps be configured with different gap types (i.e., some gaps are per-UE while some gaps are per-FR)? If so, what is the maximum number of gaps that could be configured simultaneously for each gap type (per-UE /per-FR1/per-FR2)? Q4 – Is the legacy gap sharing configuration (configured in MeasGapSharingConfig) applicable to Rel-17 concurrent gaps? If ‘yes’, could RAN4 clarify how this would work? Q5 – Could RAN4 help to clarify whether UTRAN-FDD measurement (configured in MeasObjectUTRA-FDD) is also applicable in concurrent gap operation?

We will try to analyze each question and reply the LS to RAN2.

Proposal 14: Reply RAN2’s LS as follow.

Q1 – Can Rel-17 concurrent gaps be configured together with legacy gap?

In Rel-17, concurrent gaps can be believed as multiple legacy gaps. NW can configure one legacy gap firstly. After that how to configure additional gaps for concurrent gaps is up to RAN2’s design. NW can also configure multiple gaps simultaneously, where at least one of the gaps shall be legacy gap.

Q2 – How many concurrent gaps could be configured simultaneously?

When UE only supports per-UE gap and concurrent gaps, up to 2 gaps can be configured to UE. When UE supports per-FR gap and concurrent gaps, up to 2 gaps in each FR can be configured to UE.

Q3 – Could concurrent gaps be configured with different gap types?

RAN4 agrees to support per-UE gap and per-FR gap configuration in Rel-17.

Q4 – Is the legacy gap sharing configuration (configured in MeasGapSharingConfig) applicable to Rel-17 concurrent gaps?

RAN4 confirms that the legacy gap sharing configuration for intra-frequency and inter-frequency is applicable in each measurement gap. The network can configure MeasGapSharingConfig for each legacy gap within concurrent gaps separately.

Q5 – Could RAN4 help to clarify whether UTRAN-FDD measurement (configured in MeasObjectUTRA-FDD) is also applicable in concurrent gap operation?

It has already captured in previous RAN4 LS.

# Conclusion

In the contribution, we discuss the open issues for concurrent gap. We have the following proposals:

Proposal 1: It’s up to NW to decide whether to configure only E-UTRAN RAT measurement objectives.

Proposal 2: UE can be configured with per-UE gap and per-FR gap when UE is capable of per-FR gap and concurrent gaps.

Proposal 3: When UE supports both per-FR gap and concurrent gaps, except the legacy gap combination, the combination of the per-UE gap and/or per-FR gap to be configured can be as follow.

| Index | # of simultaneous MG |  |  | RAN4 conclusion |
| --- | --- | --- | --- | --- |
|  | Per-FR1 | Per-FR2 | Per-UE |  |
| 0 | 2 | 1 | 0 | Supported |
| 1 | 1 | 2 | 0 | Supported |
| 2 | 0 | 0 | 2 | Supported |
| 3 | 1 | 0 | 1 | Supported |
| 4 | 0 | 1 | 1 | Supported |
| 5 | 1 | 1 | 1 | Supported |
| 6 | 2 | 2 | 0 | Supported |
| 7 | 0 | 0 | 1 | Supported |
| 8 | 1 | 1 | 0 | Supported |
| 9 | 1 | 0 | 0 | Supported |
| 10 | 0 | 1 | 0 | Supported |

Proposal 4: The max number of supported concurrent gap can be 4 when UE supports both per-FR gap and concurrent gaps without considering MUSIM and NTN gaps.

Proposal 5: The proximity condition X = 4ms for FR1 and 1ms for FR2 once data scheduling is assumed during dropped gap occasions.

Proposal 6: Data scheduling is assumed on the dropping gap occasions.

Proposal 7: As a compromised solution, RAN4 to only define requirement for UE to perform measurement in the gap with higher priority on all colliding occasions in Rel-17.

Proposal 8: The concurrent gaps can support new type of gaps by indicating the gaps based on the usage, such as MU-SIM gaps.

Proposal 9: NW needs to configure each measurement gap together with the priority indication.

Proposal 10: RAN4 not to define an overhead cap for concurrent gaps.

Proposal 11: The following UE behaviours after transition should be defined.

- UE will continue the measurement by MGP2 and meet the corresponding measurement requirement based on MGP2 during this measurement period once the MO1 is reconfigured to be measured using MGP2.

- UE will perform the measurement on MO2 using MGP2 immediately after the concurrent gaps’ reconfiguration, if MO2 can’t be measured by MGP1 due to gap offset or if gap length is not enough.

- After one of concurrent gaps deconfiguration, data scheduling is expected on this disabled MG’s time occasions.

Proposal 12: Additional scaling factor Kgap shall be introduced on top of CSSF within gap, where:

- When none of the MG occasions of the MGP with lower priority are overlapped by other MGP(s), Kgap = 1.

- When all of the MG occasions of the MGP with lower priority are overlapped by other MGP(s), the MGP with lower priority will be disabled.

- Otherwise, Kgap will be applied to the MG with lower priority and equals11-MGRPhigh priorityMGRPlow priority.

Proposal 13: The scaling factor Kp for measurement outside gap shall be updated as follow.

  - When all the MG occasions of the MGP with lower priority are overlapped by other MGP(s), the MGP with lower priority will be disabled.

  - When none of the MG occasions of the MGP with lower priority are overlapped by other MGP(s), Kp= 11- i=1NSMTCMGRPi .

  - Otherwise,

    - When both MGs are overlapping with the SMTC, Kp= 11- SMTCMGRPlower priority

    - When MG with lower priority is overlapping with the SMTC, but MG with higher priority isn’t overlapping with the SMTC, Kp= 11- SMTCMGRPlower priority×Kgap

    - When MG with lower priority isn’t overlapping with the SMTC, but MG with higher priority is overlapping with the SMTC, Kp= 11- SMTCMGRPhigher priority

Proposal 14: Reply RAN2’s LS as follow.

Q1 – Can Rel-17 concurrent gaps be configured together with legacy gap?

In Rel-17, concurrent gaps can be believed as multiple legacy gaps. NW can configure one legacy gap firstly. After that how to configure additional gaps for concurrent gaps is up to RAN2’s design. NW can also configure multiple gaps simultaneously, where at least one of the gaps shall be legacy gap.

Q2 – How many concurrent gaps could be configured simultaneously?

When UE only supports per-UE gap and concurrent gaps, up to 2 gaps can be configured to UE. When UE supports per-FR gap and concurrent gaps, up to 2 gaps in each FR can be configured to UE.

Q3 – Could concurrent gaps be configured with different gap types?

RAN4 agrees to support per-UE gap and per-FR gap configuration in Rel-17.

Q4 – Is the legacy gap sharing configuration (configured in MeasGapSharingConfig) applicable to Rel-17 concurrent gaps?

RAN4 confirms that the legacy gap sharing configuration for intra-frequency and inter-frequency is applicable in each measurement gap. The network can configure MeasGapSharingConfig for each legacy gap within concurrent gaps separately.

Q5 – Could RAN4 help to clarify whether UTRAN-FDD measurement (configured in MeasObjectUTRA-FDD) is also applicable in concurrent gap operation?

It has already captured in previous RAN4 LS.

# Reference

[1] R4-2120414, “WF on R17 NR MG enhancements - Multiple concurrent and independent MG patterns”, MediaTek Inc.

[2] R2-2111472, “Reply LS on R17 NR MG enhancements – Concurrent MG”, RAN2