3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201220

Electronic Meeting, January 17-25, 2022

Agenda Item:	6.12.2.2

Source:	Ericsson

Title:	Simulation results on PDSCH performance for intra-cell inter-user interference

Document for:	Information

# 1	Introduction

In this contribution, the MMSE-IRC for suppressing inter-cells’ interference performance simulation results based will be shown. The detailed simulation assumptions are listed in table below.

Table 1. Common Parameters

| Parameters | Serving cell FDD |  | Serving cell TDD |
| --- | --- | --- | --- |
| Network scenario | Sync |  | Always in Sync |
| Number of carriers | single carrier |  |  |
| Carrier frequency | [2 GHz] | [4GHz] |  |
| PDSCH allocation in frequency domain | Full PRB |  |  |
| Physical cell ID | 0 |  | 0 |
| Active DL BWP index | 1 |  | 1 |
| Channel bandwidth | 10MHz |  | 40MHz |
| TDD configuration | N/A |  | 7DS2U, S=6D+4G+4U |
| SCS | 15kHz |  | 30kHz |
| PDCCH allocation in time domain | symbols #0 and #1 of each slot for PDCCH |  |  |
| SSB position in burst | SSB index #0 |  |  |
| SSB periodicity | 20ms |  |  |
| TRS configuration | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Density 3, 1 port Periodicity 20 slots Offset: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |  | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Density 3, 1 port Periodicity 40 slots Offset: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| CSI-RS configuration | Not necessary of PDSCH Demod |  |  |
| Propagation condition | TDLC300-100 for rank 1+1; TDLA30-10 for rank 2+2 |  |  |
| Time Offset relative to serving cell | 0 us |  |  |
| Metric | SNR at 70% of max TP |  |  |
| Traffic model | Full buffer |  |  |
| Number of HARQ Processes | 4 | 8 |  |
| Maximum HARQ transmission | 4 |  |  |

Table 2. UE specific parameters

| Parameters | Target UE | Interf. UE |  |
| --- | --- | --- | --- |
| PDSCH configuration | Type A, Start symbol 2, Duration 12​ ​ |  |  |
| PRB bundling size for target PDSCH | 2 |  |  |
| Antenna configuration | Rank 1: 2x2, 2x4 Rank 2: 4x4 |  | Rank 1: 2x2, 2x4 Rank 2: 4x4 |
| Rank | (1, 1), (2, 2) |  |  |
| PDSCH DMRS configuration | Type 1 |  | Type 1 |
| PDSCH DMRS ports | Option 1-1: port 0 for rank (1, 1) Option 1-2: port 0 for rank (1, 1) Option 2-1: port 0 and 1 for rank (2, 2) |  | Option 1-1: port 1 for rank (1, 1) Option 1-2: port 2 for rank (1, 1) Option 2-1: port 2 and 3 for rank (2, 2) |
| PDSCH DMRS scrambling ID | Option 1-1-1: same within CDM group Option 1-1-2: different within CDM group |  |  |
| Maximum number of OFDM symbols for DL front loaded DMRS | 1 |  | 1 |
| Number of additional DMRS | 1 |  | 1 |
| MCS | 16QAM, MCS 13 |  | Random 16 QAM |
| Precoding granularity | 2 |  | 2 |
| PRB bundling size | 2 |  | 2 |
| DMRS Precoding configuration | Single Panel Type I |  | Single Panel Type I |
| PDSCH precoder | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination |  | Random precoding, but not identical with target UE |
| Antenna Correlation | ULA Low for 2Tx and 4Tx |  |  |

# 2	Simulation Results

The simulation results are summarized as follow.

Table 3. simulation results for MU-MIMO FDD TDLA30-10

| Case Index | Rank combination | PMI | Antenna configuration | DMRS ports | CDM group | MCS | SNR [dB] @ 70% of max T-put |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | MRC | IRC | Gain |
| #1 | (2,2) | Random | 4*4 | (0,1; 2,3) | different | 13 | N/A | 13.0 | Inf |
| #2 | (2,2) | Orthogonal | 4*4 | (0,1; 2,3) | different | 13 | N/A | 11.56 | Inf |

Table 4. simulation results for MU-MIMO FDD TDLC300-100

| Case Index | Rank combination | PMI | Antenna configuration | DMRS ports | CDM group | MCS | SNR [dB] @ 70% of max T-put |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | MRC | IRC | Gain |
| #1 | (1,1) | Random | 2*2 | (0; 1) | same | 13 | N/A | 13.57 | inf |
| #2 | (1,1) | Random | 2*4 | (0; 1) | same | 13 | 16.41 | 6.84 | 9.57 |
| #3 | (1,1) | Random | 2*2 | (0; 2) | different | 13 | N/A | 13.53 | inf |
| #4 | (1,1) | Random | 2*4 | (0; 2) | different | 13 | 14.87 | 6.76 | 8.11 |
| #5 | (1,1) | Orthogonal | 2*2 | (0; 1) | same | 13 | N/A | 13.39 | inf |
| #6 | (1,1) | Orthogonal | 2*4 | (0; 1) | same | 13 | 13.84 | 6.66 | 7.18 |
| #7 | (1,1) | Orthogonal | 2*2 | (0; 2) | different | 13 | N/A | 13.35 | inf |
| #8 | (1,1) | Orthogonal | 2*4 | (0; 2) | different | 13 | 13.21 | 6.57 | 6.64 |

Observation 1: Compared with MRC, IRC receiver can have obvious gain in 70% of maximum throughput SNR point.

Detail simulation results are shown in the appendix.

# References

- R4-2115733, “Way Forward on MMSE-IRC for intra-cell inter-user interference”, Huawei, HiSilicon

# Appendix

TDLA30-10

Figure 1. IRC and MRC performance for rank2+2 (Random PMI)

Figure 2. IRC and MRC performance for rank2+2 (Orthogonal PMI)

TDLA300-100

Random PMI selection

Figure 3. IRC and MRC performance for rank1+1 same CDM group (Random PMI, 2Rx, 4Rx)

Figure 4. IRC and MRC performance for rank1+1 different CDM groups (Random PMI, 2Rx, 4Rx)

Orthogonal PMI selection

Figure 5. IRC and MRC performance for rank1+1 same CDM group (Orthogonal PMI, 2Rx, 4Rx)

Figure 6. IRC and MRC performance for rank1+1 different CDM groups (Orthogonal PMI, 2Rx, 4Rx)