3GPP TSG-RAN WG4 Meeting #101bis-e	R4-2201279

Electronic Meeting, 17-25 Jan, 2022

Title:	R17 FR1 Redcap UE

Source:	OPPO

Agenda item:	6.20.2.1.1

Document for:	Approval

- Introduction

In RAN4#101e, it was agreed that for RedCap UE power class PC2 is supported based on operator request, and regarding UE architectures still keep open. This paper discusses on this aspect.

Issue 1-2: PC2 support Agreement: Support PC2 RedCap UE based on operator request. - Use 1 Tx as baseline - Further discussion whether the antenna isolation assumptions for existing 2Tx requirements is valid for RedCap Issue 1-3: UE architecture for FR1 RedCap UE Agreement: to define the Tx RF requirements for RedCap - Use 1 Tx as baseline - 2Tx is kept open for discussion

- Discussion

In last meeting, the Tx antennas that Redcap UE can apply was discussed, and it was agreed that 1Tx is baseline and 2Tx is keep open for discussion including the antenna isolation assumptions, etc.

There is no explicit description/discussion of Tx antennas in Redcap WID nor RAN1. And it can be considered as that both 1Tx and 2Tx capabilities also apply to Redcap UE. For example, in the heavy UL services or UL/DL balanced services like video surveillance, 1Tx might not be enough. This makes 2Tx is useful in this case.

For small form factor devices like wearable devices, currently we don’t see the demands of 2Tx. Therefore, in general it can be considered that 2Tx Redcap UE could be devices with equal to or larger than smart phone sizes. And the antenna isolation assumptions could be reused.

Observation 1:    2Tx might be useful in heavy UL services or UL/DL balanced services which typically have equal to or larger than smart phone sizes.

Proposal 1:         2Tx is allowed for Redcap UE, and existing antenna isolation assumptions can be reused.

From Tx requirement definition perspective, current requirements can be reused for UEs with 1Tx or 2Tx antennas.

Proposal 2:         Reuse current 1Tx or 2Tx requirements according to UE Tx capabilities.

- References

[1] R4-2120067, WF on the RedCap RF, Ericsson