3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201320

# Electronic meeting, January 17-25, 2022

Source:		Ericsson

Title:			NTN: Satellite Access node RF Tx OTA

Agenda item:		6.13.3.1

Document for:		Approval

# Introduction

The NTN WI ([1]) has been approved in RAN#88e meeting to specify requirements for the support of NTN. It has been revised in last RAN#94-e meeting [2].

In last RAN4 meeting, a Way Forwards related to satellite access node RF Tx requirements ([3]) was agreed. No OTA requirement has been discussed so far but at least some of them are needed to finalize the specifications of the satellite access node type 1-H.

In the following contribution, we are checking which OTA Tx requirements would be needed depending on the satellite access node type, and how it could be derived from the conducted requirement when relevant.

# Discussion

Following Table 1 lists all OTA Tx requirements and their applicability for satellite access node type 1-H and 1-O.

| Requirement | BS type 1-H | BS type 1-O |
| --- | --- | --- |
| Radiated transmit power | Yes | Yes |
| OTA base station output power |  |  |
| OTA output power dynamics |  |  |
| OTA transmit ON/OFF power |  |  |
| OTA transmitted signal quality |  |  |
| OTA occupied bandwidth |  |  |
| OTA ACLR | NA |  |
| OTA out-of-band emission |  |  |
| OTA transmitter spurious emission |  |  |
| OTA transmitter intermodulation |  |  |

Table 1: OTA Tx requirements applicability for NTN satellite access node type 1-H and 1-O

Observation1: OTA radiated transmit power requirement shall be specified for NTN satellite access node type 1-H.

Following Table 2 goes through the OTA Tx requirements, describing how they could be derived from the conducted requirements when relevant.

| Requirement | Way forward |
| --- | --- |
| Radiated transmit power | This requirement is based on manufacturer declaration of an EIRP level for each supported beam. |
| OTA base station output power | This requirement checks the accuracy (+/-2dB) of the declared TRP output power |
| OTA output power dynamics |  |
| OTA RE power control dynamic range | Same limits than 1-H |
| OTA total power dynamic range | Same limits than 1-H |
| OTA transmit ON/OFF power | NA, only for TDD operations |
| OTA transmitted signal quality |  |
| OTA frequency error | Same limits than 1-H |
| OTA modulation quality | Same limits than 1-H |
| OTA time alignment error | Same limits than 1-H |
| OTA occupied bandwidth | Same limits than 1-H |
| OTA ACLR | Same limits than 1-H, the X scaling should not be relevant here. |
| OTA out-of-band emission | Same limits than 1-H, the X scaling should not be relevant here. |
| OTA transmitter spurious emission | Same limits than 1-H |
| OTA transmitter intermodulation | Not needed for 1-H and 1-O |

Table 2: OTA Tx requirements specification

Observation2: Satellite access node OTA Tx requirements could be specified following a similar approach than for TN OTA Tx requirements.

Based on those observations, we make the following proposal:

Proposal: Specify satellite access node OTA x requirements for type 1-H and 1-O following a similar approach as for TN OTA Tx requirements, as proposed in Table 2.

# Conclusion

In this contribution, we further discussed the satellite access node RF OTA Tx requirements. We made the following observations and proposal:

Observation1: OTA radiated transmit power requirement shall be specified for NTN satellite access node type 1-H.

Observation2: Satellite access node OTA Tx requirements could be specified following a similar approach than for TN OTA Tx requirements.

Proposal: Specify satellite access node OTA x requirements for type 1-H and 1-O following a similar approach as for TN OTA Tx requirements, as proposed in Table 2.

| Requirement |  |
| --- | --- |
| Radiated transmit power | This requirement is based on manufacturer declaration of an EIRP level for each supported beam. |
| OTA base station output power | This requirement checks the accuracy (+/-2dB) of the declared TRP output power |
| OTA output power dynamics |  |
| OTA RE power control dynamic range | Same limits than 1-H |
| OTA total power dynamic range | Same limits than 1-H |
| OTA transmit ON/OFF power | NA, only for TDD operations |
| OTA transmitted signal quality |  |
| OTA frequency error | Same limits than 1-H |
| OTA modulation quality | Same limits than 1-H |
| OTA time alignment error | Same limits than 1-H |
| OTA occupied bandwidth | Same limits than 1-H |
| OTA ACLR | Same limits than 1-H, the X scaling should not be relevant here. |
| OTA out-of-band emission | Same limits than 1-H, the X scaling should not be relevant here. |
| OTA transmitter spurious emission | Same limits than 1-H |
| OTA transmitter intermodulation | Not needed for 1-H and 1-O |

# References

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-213691, Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2120674, WF on Tx RF requirements for satellite access node, CATT