3GPP TSG-RAN WG4 Meeting #101bis-e                                                    R4-2201356

Electronic Meeting, Jan. 17-25, 2022

Source:	ZTE Corporation

Title:	TP for TR 37.717-21-11:  DC_1A-38A_n78A

Agenda Item:	5.15.1

Document for:	Approval

- Introduction

This contribution provides a text proposal on  DC_1A-38A_n78A for TR37.717-21-11[1] .

- Reference

- TR 37.717-21-11, Rel-17 Dual Connectivity (DC) of 2 LTE band (1DL/1UL) and 1 NR band (1DL/1UL), v0.6.0

- Text Proposal

----- Start of TP -----

  - 5.x			DC_1-38_n78

    - 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_1A-38A_n78A | DC_1A_n78A | CA_1A-38A | n78A |

    - 5.x.2	Co-existence studies

Co-existence analysis for DC_1_n78 UL in TR 37.863-01-01 shows IMD4 produce may fall into LTE Band 38 DL.

    - 5.x.3	∆TIB and ∆RIB values

For DC_1-38_n78, same TIB,c and RIB,c values of DC_1_n38-n78 are reused which are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-38_n78 | 1 | 0.5 |
|  | 38 | 0.5 |
|  | n78 | 0.8 |

Table 5.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-38_n78 | 1 | 0 |
|  | 38 | 0 |
|  | n78 | 0.5 |

    - 5.x.4	Reference sensitivity exceptions

Based on co-existence studies additional MSD is needed to be defined. The MSD value is given in table 6.x.5-1, where same MSD values as DC_1_n38-n78 with UL configuration of DC_1_n78 are reused .

Table 6.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-38A_n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 38 | 2590 | 5 | 25 | 2590 | 12.7 | IMD4 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |

----- End of TP -----