3GPP TSG RAN WG4 Meeting #101-bis-e	R4-2201365

Electronic Meeting, Jan. 17-25, 2022

Agenda item:			6.15.5.1

Source:	vivo

Title:	Further discussion on Intra-band con-current V2X operation RRM requirements

Document for:	Discussion

- Introduction

In the previous RAN4 meetings, RRM impacts of sidelink enhancements were discussed. The agreements/conclusions on intra-band con-current V2X operation RRM requirements in the RAN4#101-e meeting is captured in [1] as follows.

WF – 1.1 NTA_offset & NTA,SL when NR Cell is configured as synchronization reference source - Consider that SL NTA_offset = UL NTA_offset & NTA,SL = 0 WF – 1.2.1 Scheduling restriction calculation when switching TDM based intra-band con-current SL operation - Define the scheduling availability requirements based on one slot as baseline - If RF’s decision on the switching time impacts on it, it will be revisited. WF – 1.2.2 Scheduling restriction location when switching TDM based intra-band con-current SL operation - Reuse the principle defined in section 12.9.1 of TS38.133 for transmission switching between SL and Uu (No priority rules in RRM requirements, which are specified by covering all possible cases) WF – 1.3 Interruption on SL due to Uu BWP switch for FDM based intra-band con-current SL operation - Interruption on SL due to Uu BWP switch for FDM based intra-band con-current SL operation - Define interruption similar to the interruption of NR intra-band CA/DC

We provided our initial views on RRM requirements related to new operation scenarios for sidelink enhancement in the previous meetings [2, 3, 4]. In this contribution, we further provide our views on remaining issues on RRM requirements for intra-band con-current V2X operation for R17 NR sidelink enhancements.

- Discussion

In Rel-16 NR V2X, the RRM requirements are specified for V2X SL operation on a V2X carrier which is dedicated to only V2X SL operation and configured with only a PCell on WAN carrier. Only inter-band con-current operation of V2X SL and Uu are supported.

Intra-band con-current sidelink operation in a licensed band are supported in Rel-17. Following scenarios could be supported.

- Focus on intra-band con-current V2X operation with adjacent carrier for TDD band and deprioritize FDD band. FDD band can be studied once operator has request. - Prioritize the scenario for Uu and SL in both the same carrier and different carrier for intra-band V2X operation (TDM).

Scheduling availability

For intra-band con-current sidelink TDM operation, the switching between NR SL and NR Uu would lead to scheduling availability from RRM requirements perspective due to switching timing is needed. In the last RRM session, it was agreed that scheduling availability requirements are based on one slot as baseline. It is also agreed to check if there would RF decision that may have impact on the assumption.

In the last meeting, RF session proposals for switching length are as follows as captured in [5].

Issue 1-1-3: Switching time for different carriers - Proposals - Option 1: RF switching time is 140us. - Option 2: Total switching time equal to PUSCH preparation time for SL to Uu switching and PSSCH preparation time for Uu to SL switching + RF switching time(140us) (R4-2117648) - Observation - For LTE SL and NR LTE switching in ITS band, only RF retuning time is considered, i.e. 150us. - One company would like to further check whether the RF switching time occurs after PSSCH/PUSCH preparation time or simultaneously with PSSCH/PUSCH preparation time. - WF - RF switching time is different from PUSCH/PSSCH preparation time. - Further discuss whether the RF switching time occurs after PSSCH/PUSCH preparation time or simultaneously with PSSCH/PUSCH preparation time. If RAN4 agree RF switching time is separate from preparation time and can occur either sequentially or simultaneously to the preparation time, the switching time, i.e. 140us can be agreed.

So firstly, it can be concluded from conclusions in RF session that 140us RF switching time could be agreeable. The remaining issue is whether PUSCH/PSSCH preparation time should be included in total switching time or not.

From RRM requirements perspective, we think PUSCH/PSSCH preparation time should be separated from RF switching time. The scheduling availability is due to RF switching from Uu to SL or from SL to Uu vice versa. The PUSCH/PSSCH preparation time, which is started from the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH/PUSSH, has already be considered in UE PUSCH/PSSCH procedure time. In our understanding, the scheduling availability is to give UE additional time for RF switching. Thus, the scheduling availability requirements are defined with one slot length by considering 140us RF switching time.

Proposal 1: it is confirmed that 1 slot is restricted from scheduling when switching happens between NR sidelink and NR for TDM based intra-band con-current sidelink operation.

FDM based intra-band con-current SL operation

For FDM based intra-band con-current SL operation, it was agreed to define interruption requirements on SL due to Uu BWP switch.

Interruption on SL due to Uu BWP switch for FDM based intra-band con-current SL operation - Interruption on SL due to Uu BWP switch for FDM based intra-band con-current SL operation - Define interruption similar to the interruption of NR intra-band CA/DC

However, it is not very clear how the interruption requirements should be specified. Defining interruption similar to the interruption of NR intra-band CA/DC is not clear. In our understanding, the interruption requirements should be similar to interruptions due active BWP switching for NR operation, which is not necessarily different for intra-band CA/DC or inter-band CA/DC. Our view is that interruption length should be the same as X in Table 8.2.2.2.5-1 in TS 38.133.

Proposal 2: Interruption length X in Table 8.2.2.2.5-1 is used for defining interruption requirements on SL due to BWP switching on Uu for FDM based intra-band con-current operation.

- Summary

In this contribution we further provided our views on intra-band con-current V2X operation for R17 NR sidelink enhancements. Based on analysis following observations and proposals are present.

Proposal 1: it is confirmed that 1 slot is restricted from scheduling when switching happens between NR sidelink and NR for TDM based intra-band con-current sidelink operation.

Proposal 2: Interruption length X in Table 8.2.2.2.5-1 is used for defining interruption requirements on SL due to BWP switching on Uu for FDM based intra-band con-current operation.

- References

- R4-2120315	WF on NR SL enhancements RRM requirements, LG Electronics

- R4-2109946	RRM impacts overview for sidelink enhancement, vivo

- R4-2112555	Further discussion on RRM impacts for sidelink enhancement, vivo

- R4-2117762	Further discussion on Intra-band con-current V2X operation RRM requirements, vivo

- R4-2119988	WF on RF requirements and sync issues for intra-band operation, CATT