3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201425

Electronic Meeting, 17 – 25 January 2022

Agenda Item:	6.9.5.2.1

Source:	Ericsson

Title:	PDSCH demodulation requirements for HST FR2

Document for:	Discussion

# 1	Introduction

RAN4#101-e agreed with the way forward on the channel model and UE demodulation requirements for HST FR2 [1]. This contribution discusses the open issues on PDSCH demodulation requirements.

# 2	Discussion

## 2.1	TCI switch scheduling for uni-directional scenario

Figure 1 illustrates the activated RRH (TCI state) number according to the UE location for the Uni-directional deployment scenario. From the figure, the active TCI state switch happens every Ds, and the active RRH number is given as follows:

- Switch from RRH #(k-1) to RRH #(k+1) at the location of k-1⋅Ds-Ds_offset, k=0,1,2,…

Figure 1	Active RRH (TCI state) number according to the location of UE for the uni-directional deployment scenario.

Proposal 1: Schedule the active TCI switching for PDSCH demodulation test with the channel model assuming the Uni-directional Scenario A as follows:

- Switch from RRH #(k-1) to RRH #k at the location of k-1⋅Ds-Ds_offset, k=0,1,2,…

## 2.2	PDSCH scheduling at TCI switching [common for both uni-directional and bi-directional deployment]

- Option 1: Do not consider throughput statistics until the UE has received and processed at least one instance of SSB, followed by one instance of TRS and including respective RS processing time. - THARQ + TMAC Proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS pro - THARQ is the number of slots between PDSCH and corresponding HARQ-ACK information, - TMAC Proc is the number of slots for MAC CE processing - TfirstSSB is the number of slots to the first SSB transmission occasion after MAC CE command is decoded by the UE - TSSB proc is the number of slots for SSB processing - TfirstTRSafterSSB is the number of slot to the first TRS transmission occasion available after (TfirstSSB + TSSB proc) - TTRS pro is the number of slots for TRS processing - Option 2: - THARQ + TMAC Proc + TfirstRS + TRS_proc - TfirstRS is the larger number of slots to the first SSB transmission and the first TRS transmission after MAC CE command is decoded by the UE - TRS_proc is the larger number of slots for SSB processing and TRS processing - Option 3: - Bi-directional scenario DPS scheme1a, THARQ + TMAC Proc + TfirstSSB + TSSB proc - Uni-directional scenario DPS scheme 1b, THARQ + TMAC Proc - Option 4 - Bi-directional scenario DPS scheme1a, THARQ + TMAC Proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc - Uni-directional scenario DPS scheme1b, THARQ + TMAC Proc

For FR1 HST-DPS, RAN4 demodulation requirements do not consider the following period after receiving MAC CE active TCI switching to calculate the throughput statistics:

- DPS Scheme 1a (one active TCI states): THARQ + TMAC proc + TfirstTRS + TTRS proc

- DPS Scheme 1b (two or more active TCI states): THARQ + TMAC proc

  - THARQ: Number of slots between PDSCH and corresponding HARQ-ACK information

  - TMAC proc: Number of slots for MAC CE processing

  - TfirstTRS: Number of slots to first TRS transmission occasion after MAC CE command is decoded by the UE

  - TTRS proc: Number of slots for TRS processing

In our understanding, FR1 HST-DPS requirements assume UE performs time/frequency tracking based on TRS. On the other hand, RAN4 agreed frequency offset tracking is up to UE implementation for FR2 HST. This means it is up to UE implementation whether to use SSB or TRS or PTRS. Since PTRS is associated with PDSCH, we only need to consider the time when the first SSB and/or TRS is scheduled. In our understanding Option 1 assumes UE need to receive both SSB and TRS and it also assumes UE processes SSB first, followed by TRS. On the other hand, Option 2 does not care the order which RS is received first, SSB or TRS.

Our preference is to minimize the DTX period due to the time/frequency adjustment. We therefore to propose to set the DTX period based on the larger number of slots to the first SSB transmission and the first TRS transmission after MAC CE command is decoded by the UE.

Proposal 2: RAN4 demodulation requirements do not consider the following period after receiving MAC CE active TCI switching from the throughput statistics:

- DPS Scheme 1a (UE capable of one active TCI state): THARQ + TMAC proc + TfirstRS + TRS proc

- DPS Scheme 1b (UE capable of two or more active TCI states): THARQ + TMAC proc

  - THARQ: Number of slots between PDSCH and corresponding HARQ-ACK information

  - TMAC proc: Number of slots for MAC CE processing

  - TfirstTRS: Larger number of slots to the first SSB transmission and the first TRS transmission after MAC CE command is decoded by the UE

  - TRS proc: Larger number of slots for SSB processing and TRS processing

## 2.3	Indication of HST deployment topology [common for both uni-directional and bi-directional deployment]

- FFS on introduction higher layer signalling (ie. System Information, RRC)) to inform the UE of the FR2 HST deployment typology (Uni-directional and Bi-directional scenario) for PDSCH demodulation requirement - FFS on introduction MAC-CE signalling to inform the UE of the TCI switching typology (Intra/Inter -RRH) for PDSCH demodulation requirement - Encourage companies to analysis the impact on PDSCH demodulation requirement, the UE receiver processing, as well as the pros and cons with and without NW signalling, from demodulation perspective

RAN4#101 agreed that the FR2 HST UE demodulation test assumes that UE receives the signals with single panel even for the channel model assuming the Bi-directional deployment scenario [2].

OTA test for UE and BS - Assuming single panel for FR2 HST UE and BS Bi-directional RRH deployment scenario testing

This agreement implies the single transmitter sends the signal by changing the Doppler shift according to the TCI switch scheduling discussed in 2.2. This means UE is not required to adjust Rx beams even if TCI switch happens, and from the demodulation test perspective, UE only need to adjust the time/frequency based on SSB and/or TRS as discussed in 2.3. Therefore, we do not see any necessity to indicate the deployment topology and/or Doppler changes at the TCI switching.

We also point out the UE demodulation requirements for FR2 HST is only applicable for the high-speed train roof-mounted UE, i.e., PC6 UE. We believe such a UE should be optimized for the FR2 HST channel condition RAN4 discussed so far.

Proposal 3: No signaling is needed to indicate the deployment topology for FR2 HST UE demodulation requirements.

## 2.4	Applicability of test cases [common for both uni-directional and bi-directional deployment]

Table 1 summarizes the current assumption of test cases defined for FR2 HST UE demodulation requirements. The remaining open issue is the applicability of the test cases.

Table 1	Possible test cases for FR2 HST UE demodulation requirements.

| Test number | Channel model and active TCI switching scheduling | DPS Tx scheme | Channel model parameters |
| --- | --- | --- | --- |
| 1 | HST-DPS-FR2-B (Bi-directional Scenario B) | 1a | v: 350km/h Ds: 700ms Dmin: 150m |
| 2 | HST-DPS-FR2-A (Uni-directional Scenario A) | 1b | v: 350km/h Ds: 700m Dmin: 10m Ds_offset: 10m |

If we follow the applicability rule defined in FR1 HST-DPS, Test 1 is applicable for UEs capable of handling single active TCI state and Test 2 is applicable for UEs capable of handling two or more active TCI states. Since RAN4 is going to use the different channel model for scheme 1a and 1b, UE capable of two or more active TCI states is not verified the huge Doppler jump realized by the Bi-directional deployment. One possibility is to apply both channel models for UE capable of two or more active TCI states. According to our simulation results, however, no performance difference is observed between two tests as far as the maximum Doppler shift is same. To minimize the number of test cases, our preference is to define one set of requirements based on the Bi-directional Scenario B as the same agreement as BS demodulation requirements for HST FR2.

Proposal 4: RAN4 define UE demodulation requirements with transmission schemes DPS 1a and 1b only with the channel model based on Bi-directional Scenario B.

| Test number | Channel model and active TCI switching scheduling | DPS Tx scheme | Channel model parameters |
| --- | --- | --- | --- |
| 1 | HST-DPS-FR2 (derived based on Bi-directional Scenario B) | 1a / 1b according to UE capability of the number of active TCI states. | v: 350km/h Ds: 700ms Dmin: 150m |

## 2.5	SSB/TRS configuration [common for both uni-directional and bi-directional deployment]

- SSB and TRS period configuration - Option 1: 20ms for SSB, 10ms for TRS - Option 2: 20ms for both SSB and TRS - Other options are not precluded

We prefer the same SSB/TRS configuration assumption as FR1 HST scenario, that is, the periodicity of 20ms for SSB and 10ms for TRS.

Proposal 5: Configure SSB with the periodicity of 20ms and TRS with the periodicity of 10ms.

# 3	Summary

Proposal 1: Schedule the active TCI switching for PDSCH demodulation test with the channel model assuming the Uni-directional Scenario A as follows:

- Switch from RRH #(k-1) to RRH #k at the location of k-1⋅Ds-Ds_offset, k=0,1,2,…

Proposal 2: RAN4 demodulation requirements do not consider the following period after receiving MAC CE active TCI switching from the throughput statistics:

- DPS Scheme 1a (UE capable of one active TCI state): THARQ + TMAC proc + TfirstRS + TRS proc

- DPS Scheme 1b (UE capable of two or more active TCI states): THARQ + TMAC proc

  - THARQ: Number of slots between PDSCH and corresponding HARQ-ACK information

  - TMAC proc: Number of slots for MAC CE processing

  - TfirstTRS: Larger number of slots to the first SSB transmission and the first TRS transmission after MAC CE command is decoded by the UE

  - TRS proc: Larger number of slots for SSB processing and TRS processing

Proposal 3: No signaling is needed to indicate the deployment topology for FR2 HST UE demodulation requirements.

Proposal 4: RAN4 define UE demodulation requirements with transmission schemes DPS 1a and 1b only with the channel model based on Bi-directional Scenario B.

| Test number | Channel model and active TCI switching scheduling | DPS Tx scheme | Channel model parameters |
| --- | --- | --- | --- |
| 1 | HST-DPS-FR2 (derived based on Bi-directional Scenario B) | 1a / 1b according to UE capability of the number of active TCI states. | v: 350km/h Ds: 700ms Dmin: 150m |

Proposal 5: Configure SSB with the periodicity of 20ms and TRS with the periodicity of 10ms.

# References

- R4-2120775, “WF on general and UE demodulation requirement for FR2 HST”, Samsung.

- R4-2120066, “WF on FR2 HST UE requirements”, Samsung.