3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201431

Electronic Meeting, 17 – 25 January 2022

Agenda Item:	8.9.5

Source:	Ericsson

Title:	Introduction of channel quality report table for NB-IoT supporting 16QAM

Document for:	Discussion

# 1	Introduction

RAN1 agreed with the channel quality reporting table used for NB-IoT UE supporting 16QAM and sent LS to RAN4 to capture the table in RAN4 spec [1].

RAN1 has also endorsed the following table for channel quality reporting in DL for 16-QAM NB-IoT UEs: Reported value; NPDCCH repetition level; NPDSCH transport block error probability not exceeding 0.1. Modulation; Code rate x 1024; Repetition; Efficiency. noMeasurement; No measurement reporting; Out of range. candidateRep-A; 1; QPSK; 221; 1; 0.4316. candidateRep-B; 2; QPSK; 280; 1; 0.2737. candidateRep-C; 4; QPSK; 81; 1; 0.1579. candidateRep-D; 8; QPSK; 81; 2; 0.0789. candidateRep-E; 16; QPSK; 81; 4; 0.0395. candidateRep-F; 32; QPSK; 81; 8; 0.0198. candidateRep-G; 1; QPSK; 336.8; 1; 0.6579. candidateRep-H; 1; QPSK; 453.6; 1; 0.8860. candidateRep-I; 1; QPSK; 579.4; 1; 1.1316. candidateRep-J; 1; QPSK; 759; 1; 1.4825. candidateRep-K; 1; 16QAM; 487.3; 1; 1.9035. candidateRep-L; 1; 16QAM; 541.2; 1; 2.1140. candidateRep-M; 1; 16QAM; 658; 1; 2.5702. candidateRep-N; 1; 16QAM; 783.7; 1; 3.0614. candidateRep-O; 1; 16QAM; 837.6; 1; 3.2719. It is RAN1’s assumption that the channel quality reporting table will be captured in spec by RAN4, and the corresponding signalling will be captured in spec by RAN2.

3GPP has introduced the channel quality reporting for NB-IoT UE in Rel-14, where UE in IDLE mode reports the minimum NPDCCH repetition number which corresponds to hypothetical BLER of 1% in MSG3 at the random access. In Rel-16, UE in CONNECTED mode can also report the same channel quality with MAC CE.

Although the agreed table in RAN1 is used for reporting the modulation order and coding rate for PDSCH, the reporting framework is based on Rel-14/16 channel quality report, and thus RAN1 has asked RAN4 to capture the mapping table in the RAN4 specification. Since RAN4 has defined the channel quality mapping table in TS38.133 9.1.22.15, we propose to add the new CQI table in TS38.133 9.1.22.15. The detailed TP is provided in Appendix.

Regarding the channel quality reporting requirements, it should be specified in UE demodulation performance part.

Proposal 1: RAN4 capture the channel quality reporting mapping table for NB-IoT UE supporting 16QAM in TS38.133 9.1.22.15.

Proposal 2: RAN4 discuss the CQI reporting requirements for NB-IoT UE supporting 16QAM in UE demodulation performance part.

# References

- R1-2112971, “LS on channel quality reporting for NB-IoT”, RAN1.

# Text proposal to TS36.133

#### 9.1.22.xx	Channel Quality Report Mapping for UE Category NB2

Table 9.1.22.xx-1: Downlink channel quality measurement report mapping of TBD when the reception of unicast NPDSCH modulated with 16-QAM is supported [7]

| Reported value | NPDCCH repetition level | NPDSCH transport block error probability not exceeding 0.1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Modulation | Code rate x 1024 | Repetition | Efficiency |
| noMeasurement | No measurement reporting | Out of range |  |  |  |
| candidateRep-A | 1 | QPSK | 221 | 1 | 0.4316 |
| candidateRep-B | 2 | QPSK | 280 | 1 | 0.2737 |
| candidateRep-C | 4 | QPSK | 81 | 1 | 0.1579 |
| candidateRep-D | 8 | QPSK | 81 | 2 | 0.0789 |
| candidateRep-E | 16 | QPSK | 81 | 4 | 0.0395 |
| candidateRep-F | 32 | QPSK | 81 | 8 | 0.0198 |
| candidateRep-G | 1 | QPSK | 336.8 | 1 | 0.6579 |
| candidateRep-H | 1 | QPSK | 453.6 | 1 | 0.8860 |
| candidateRep-I | 1 | QPSK | 579.4 | 1 | 1.1316 |
| candidateRep-J | 1 | QPSK | 759 | 1 | 1.4825 |
| candidateRep-K | 1 | 16QAM | 487.3 | 1 | 1.9035 |
| candidateRep-L | 1 | 16QAM | 541.2 | 1 | 2.1140 |
| candidateRep-M | 1 | 16QAM | 658 | 1 | 2.5702 |
| candidateRep-N | 1 | 16QAM | 783.7 | 1 | 3.0614 |
| candidateRep-O | 1 | 16QAM | 837.6 | 1 | 3.2719 |