3GPP TSG-RAN WG4 Meeting #101-bis-e 	      R4-2201460

# E-meeting, 17th Jan– 25th Feb, 22

Agenda item:	6.5.2.2

Source: 	ZTE Corporation

Title: 	Further discussions on emission related requirements for conducted repeater

Document for:	Approval

Introduction

In the past RAN4 meetings, there were lots of discussion on repeater conducted output power and emission requirements and there were lots of open issues left for further discussions. Therefore in this contribution, we want to share some further considerations on those remaining open issues.

Discussion

- 2.1.UL ACLR

Agreement: For Local area class, UL ACLR follow ACLR from UE side, FFS which PC (PC3 or PC2) should be referred For wide area class, UL ACLR follow gNB requirement

Regarding the UL ACLR requirements, to follow PC2 ACLR requirement 31dBc is more preferred compared with PC3 ACLR requirement 30dBc since its upper output power limit 24dBm is relatively higher than PC3. In addition, since repeater should be infra-equipment, it should be more capable than normal handled UE, therefore we propose to reuse FR1 PC2 UE ACLR requirements for Local area class;

Proposal 1: to reuse FR1 PC2 UE ACLR requirements for local area class;

- 2.2. OBUE

Agreement: on Nov.4 GTW FFS whether inside passband OBUE requirements or other requirements needed for DL and UL for the case with non-full passband transmission.

- As agreed in last RAN4 meeting, it was allowed to have some empty carriers within pass-band from other collaborative operators, however sometimes it might be quite difficult to define the collaborative operators from network deployment perspective. Therefore we think that OBUE requirement within pass-band is still necessary, otherwise there are no insurance on how to guarantee the coexistence performance with other operators.

Proposal 2: to define the OBUE requirement with pass-band if there are any empty carriers within it.

- 2.3. CACLR

- Apply CACLR requirement in the gaps between passbands for both DL and UL - FFS the multi operating band cases - FFS whether there are some exceptions that doesn’t need CACLR requirements, e.g. narrow gaps

- The Cumulative Adjacent Channel Leakage power Ratio (CACLR) in the Inter passband gap is the ratio of: a) the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the Inter passband gap, and b) the filtered mean power centred on a frequency channel adjacent to one of the respective repeater passband edges. - For DL and WA UL, CACLR follows the same relative CACLR and absolute CACLR requirements as gNB. The least stringent requirement could be applicable - For LA UL, CACLR follows the same value as ACLR requirement. i.e. 30 or 31dB should be referred - No CACLR inside passband.

The Wgap defined for CACLR limit in non-contiguous spectrum or multiple bands, this is mainly depending on ACLR definition for adjacent carrier for measurement and the component carriers on each side of sub-block gap. Since repeater will forward the the same carrier configurations from parent BS, therefore Wgap could also been applicable for repeater. Regarding the CACLR limit for local area class, we propose to use 31dBc for CACLR requirements.

Proposal 3: propose CACLR requirement as 31dBc with its Wgap configuration inherited from TS38.104 Table 6.6.3.2-3

- 2.4. Spurious emission

Additional regional emission requirements as in UE spec should be [defined in repeater spec] or [declared by vendor to compliant with regional requirements]. - No A-MPR requirement is required for repeater. - FFS Whether to define NS signaling. - FFS whether would the manufacturer declare one power level for general requirements and another level for more stringent requirement

Regarding the additional regional emission requirement defined in UE spec, we don’t think that it’s necessary to define it for repeater in UL. In Rel-16 IAB topic, MPR and A-MPR requirement is also not defined for it, therefore we don’t see strong motivation to define it for repeater. In addition, to define MPR and A-MPR based on the PA assumption and ACLR/SEM, it is very time consuming. Meanwhile, repeater is also not aware of PRB allocation and modulation orders, therefore it’s not necessary to define MPR or A-MPR for repeater in UL.

Proposal 4: additional regional emission requirements should be declared by vendors to comply with regional requirements;

Conclusions

In this contribution, we want to share some further considerations on those remaining open issues of conducted repeater and proposals are made as following:

Proposal 1: to reuse FR1 PC2 UE ACLR requirements for local area class;

Proposal 2: to define the OBUE requirement with pass-band if there are any empty carriers within it.

Proposal 3: propose CACLR requirement as 31dBc with its Wgap configuration inherited from TS38.104 Table 6.6.3.2-3

Proposal 4: additional regional emission requirements should be declared by vendors to comply with regional requirements;

References

- R4-2120666, WF on repeater conducted requirements, approved.