TSG-RAN Working Group 4 (Radio) meeting #101bis-e	R4-2201486

Electronic Meeting, 17th – 25th January 2022

Source:	Ericsson

Title:	TP to TR 38.844: Section 6.1.2 Signalling for Larger Channel BW Approach

Agenda item:	7.2.2.2

Document for:	Approval

# 1	Introduction

Based upon the LS Reply from RAN2 the following is a text proposal to include inputs for completeness.

# 2	Text Proposal

### 6.1.2	Signalling and configuration aspects

In this section we provide further signaling details on how to support irregular channels given the 7MHz allocation as an example.

The gNB broadcasts the DL carrier bandwidth and the bandwidth of the initial BWP (BWP#0) in SIB1. For the 7MHz allocation, SIB1 can indicate DL next larger standard channel bandwidth, i.e. 10 MHz, and that the initial DL BWP can be set to 5 MHz:

-	SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> frequencyInfoDL-> scs-SpecificCarrierList-> carrierBandwidth = 52 PRBs / subcarrierSpacing = 15 kHz

-	SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> initialDownlinkBWP-> genericParameters-> locationAndBandwidth = 25 PRBs

Once the UE established the RRC connection, the gNB can account for the UE capabilities and re-configure the UE accordingly. At this point the gNB may override the carrier bandwidth value that the UE obtained from SIB1 and configure a dedicated BWP with a bandwidth that differs from the bandwidth of BWP#0. gNB may configure a larger bandwidth part that will cover the whole 7MHz allocation.

-	ServingCellConfig-> downlinkChannelBW-PerSCS-List-> carrierBandwidth = 52 PRBs, subcarrierSpacing = 15 kH-	ServingCellConfig-> downlinkBWP-ToAddModList-> bwp-Common-> genericParameters-> locationAndBandwidth = TBD PRBs

It should be noted that for uplinkChannelBW-PerSCS-List and scs-SpecificCarrierList symmetric operation bands with fixed duplex distance and asymmetric UL/DL channel bandwidth is band specific aspect which is defined in TS 38.101-1 and TS 38.101-2 and bands which wish to consider wider channel bandwidth approach for the irregular bandwidths would need to be adopted in the specific bands.

The network only configures channel bandwidth according to UE capabilities. UE behaviour is unknown when the channel bandwidth configuration exceeds the frequency band borders.

# 4	References

[1]	R4-2111597, “Reply LS on specification impact for methods on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths” RAN2