3GPP TSG-RAN WG4 Meeting #101bis-e													R4-2201600

Electronic meeting, January 17 – 25, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Sensing beam for LBT in FR2-2

Agenda item:	6.16.8

Document for:	Approval

# Introduction

RAN4 received a LS from RAN1 regarding listen-before-talk (LBT) operation in FR2-2 and the relationship between the transmission beam and the sensing beam used for LBT for the transmission [1]. In the LS RAN1 lists the agreements they have made and asks RAN4 to take those into account when considering if test or requirement is needed. In this contribution we discuss LBT and sensing beam from RAN4 perspective and propose a how to handle it in RAN4.

# Discussion

LBT procedure exists in some unlicensed technologies to facilitate fair co-existence when multiple devices and/or network operate in the same frequency channel. The intention of LBT is that a device listens to the frequency channel first to detect whether there are on-going transmissions, and does not transmit in case the channel is already occupied.

Within FR2-2, the co-existence between Wigig and other technologies have been extensively studied in ETSI BRAN, where three standards are within scope of the work: 302 567, 303 753 and 303 722 each targeting different scenarios in 57 – 71 GHz band from indoor operation to high power outdoor operation with fixed point-to-point links. None of these standards intended for regulative purposes necessitates usage of a specific channel raster or channel bandwidth. The same applies for FCC 47 CFR § 15.255 governing operation within the band 57-71 GHz.

From the ETSI standards work on 303 753 and 303 722 include or will include beamforming as a co-existence mechanism, requiring no listen-before-talk (LBT) operation to be able to access the channel. Therefore. LBT and channel alignment are not mandatory features from regulatory perspective. In case channel alignment and/or LBT operation would be required within some administration, network implementation can provide the support even without specific 3GPP requirements for it.

Observation 1: Current ETSI (303 753, 303 722) and FCC (FCC 47 CFR § 15.255) rules do not mandate usage of specific channel bandwidths or channel rasters and therefore also channel alignment is optional.

So far RAN4 has not discussed LBT for FR2-2. Given that there is only 2 meetings left for the work, it would be extremely challenging to come up with requirements for LBT, which have not been specified for FR2 in the past. However, one option could still be to get guidance from the requirements in lower frequencies.

For lower frequencies where LBT has been considered the LBT requirements and tests are not part of the RF requirement specification TS 38.104. Instead, they have been captured in TS 37.107, “Base Station (BS) requirements and conformance tests for shared spectrum channel access”. However, in the WID [2] this specification has not been listed as being impacted, and change is not possible to be done. Therefore, already from RAN the guidance is not to specify LBT requirements for FR2-2.

Observation 2: Shared spectrum channel access requirements are not specified in TS 38.104.

Observation 3: Specification impact to requirements and conformance tests for shared spectrum channel access is not part of the WID and therefore LBT requirements cannot be specified.

In the LS it has also been recognized that “RAN4 can further decide for gNB or UE separately if such test or requirement is not needed or not practical and leave it to gNB or UE implementation”. Given the observations above the most reasonable thing to do is to leave sensing beam related aspects up to gNB implementation.

Proposal 1: Leave sensing beam and its relationship to transmission beam to gNB implementation.

# Conclusion

In this contribution sensing beam and its relationship to transmission beam was discussed. The following observations and proposal were made.

Observation 1: Current ETSI (303 753, 303 722) and FCC (FCC 47 CFR § 15.255) rules do not mandate usage of specific channel bandwidths or channel rasters and therefore also channel alignment is optional.

Observation 2: Shared spectrum channel access requirements are not specified in TS 38.104.

Observation 3: Specification impact to requirements and conformance tests for shared spectrum channel access is not part of the WID and therefore LBT requirements cannot be specified.

Proposal 1: Leave sensing beam and its relationship to transmission beam to gNB implementation.

# References

[1] R1-2112806, “LS on sensing beam selection”, RAN1

[2] RP-213540, “Extending current NR operation to 71 GHz”, Qualcomm, Intel