3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201637

Electronic Meeting, 17 – 25 January, 2022

Title: 	Simulation results for reduced sample number for PRS measurement

Source: 	Huawei, HiSilicon

Agenda item:	6.21.2.2

Document for:	Discussion

Introduction

RRM requirements related to latency reduction for positioning measurement were discussed in RAN4#101-e, and the outcomes are captured in the WF [1]. One aspect discussed is the reduction of sample number in PRS measurement requirements.

Based on [1], RAN4 has agreed that M1=1 sample (without considering AGC) is to be supported, and how to address M1=2 or 3 is FFS. On the other hand, the conditions under which the sample number can be reduced are FFS. As a result, simulation assumption [2] has been approved.

In this paper we will provide our simulation results for reduced number of samples for positioning measurement.

Discussion

Simulation assumption

The simulations are conducted based on assumptions in [2]. The simulation results in the following sections are shown for the following Es/Iot conditions.

- RSTD

  - Es/Iot condition #1: (-6, -13, -13) dB

  - Es/Iot condition #2: (-3, -6, -6) dB

- PRS-RSRP

  - Es/Iot condition #1: -13 dB

  - Es/Iot condition #2: -6 dB

  - Es/Iot condition #3: -3 dB

- UE Rx-Tx

  - Es/Iot condition #1: -13 dB

  - Es/Iot condition #2: -6 dB

  - Es/Iot condition #3: -3 dB

Simulation results for RSTD

The simulation results for RSTD are shown in Table 1-3, for AWGN, TDL-A/C and TDL-D channels. Same as for Rel-16, TDL-A channel is used for FR1 and TDL-C is used for FR2. The results are for the maximum between 5% and 95% of RSTD error in absolute value.

Table 1: RSTD performance for AWGN channel

| SCS (kHz) | RB | Es/Iot = (-6, -13, -13) dB |  | Es/Iot = (-3, -6, -6) dB |  | Rel-16 requirements |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 1 | 2 |  |
| 15 | 24 | 85 | 73 | 69 | 61 | 132 |
|  | 52 | 60 | 56 | 52 | 50 | 98 |
|  | 104 | 26 | 26 | 24 | 24 | 42 |
| 30 | 24 | 152 | 44 | 42 | 39 | 75 |
|  | 48 | 88 | 29 | 25 | 24 | 48 |
|  | 132 | 92 | 15 | 14 | 14 | 24 |
| 60, FR1 | 24 | 81 | 30 | 29 | 27 | 50 |
|  | 64 | 95 | 15 | 15 | 15 | 24 |
|  | 132 | 51 | 8 | 7 | 8 | 10 |
| 60, FR2 | 24 | 89 | 22 | 22 | 20 | 35 |
|  | 64 | 79 | 15 | 15 | 15 | 24 |
|  | 132 | 57 | 8 | 9 | 9 | 11 |
| 120 | 32 | 84 | 15 | 15 | 15 | 24 |
|  | 64 | 54 | 9 | 10 | 9 | 13 |
|  | 128 | 58 | 6 | 6 | 6 | 6 |

Table 2: RSTD performance for TDL-A/C channel

| SCS (kHz) | RB | Es/Iot = (-6, -13, -13) dB |  | Es/Iot = (-3, -6, -6) dB |  | Rel-16 requirements |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 1 | 2 |  |
| 15 | 24 | 2469 | 2389 | 166 | 166 | 247 |
|  | 52 | 2776 | 2335 | 97 | 97 | 140 |
|  | 104 | 1627 | 2106 | 67 | 70 | 86 |
| 30 | 24 | 2683 | 1708 | 96 | 92 | 118 |
|  | 48 | 1775 | 2909 | 74 | 75 | 109 |
|  | 132 | 2173 | 2086 | 27 | 29 | 28 |
| 60, FR1 | 24 | 1833 | 1622 | 95 | 101 | 147 |
|  | 64 | 1877 | 2609 | 22 | 22 | 27 |
|  | 132 | 1559 | 1031 | 19 | 20 | 21 |
| 60, FR2 | 24 | 1395 | 2183 | 76 | 60 | 83 |
|  | 64 | 1628 | 60 | 66 | 51 | 64 |
|  | 132 | 2046 | 69 | 71 | 66 | 46 |
| 120 | 32 | 2049 | 1497 | 64 | 49 | 48 |
|  | 64 | 1740 | 65 | 69 | 56 | 54 |
|  | 128 | 2489 | 62 | 62 | 61 | 36 |

Table 3: RSTD performance for TDL-D channel

| SCS (kHz) | RB | Es/Iot = (-6, -13, -13) dB |  | Es/Iot = (-3, -6, -6) dB |  | Rel-16 requirements |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 1 | 2 |  |
| 15 | 24 | 1172 | 138 | 138 | 134 | 247 |
|  | 52 | 1767 | 83 | 81 | 79 | 140 |
|  | 104 | 2436 | 50 | 49 | 49 | 86 |
| 30 | 24 | 1003 | 72 | 70 | 70 | 118 |
|  | 48 | 2612 | 64 | 61 | 62 | 109 |
|  | 132 | 1723 | 17 | 18 | 18 | 28 |
| 60, FR1 | 24 | 1770 | 82 | 82 | 81 | 147 |
|  | 64 | 2772 | 17 | 16 | 16 | 27 |
|  | 132 | 1374 | 13 | 13 | 13 | 21 |
| 60, FR2 | 24 | 2996 | 50 | 49 | 49 | 83 |
|  | 64 | 1808 | 36 | 35 | 35 | 64 |
|  | 132 | 1542 | 26 | 26 | 26 | 46 |
| 120 | 32 | 2331 | 27 | 28 | 28 | 48 |
|  | 64 | 1663 | 30 | 30 | 30 | 54 |
|  | 128 | 2081 | 21 | 22 | 21 | 36 |
| Note: The Rel-16 requirements are for fading channel. |  |  |  |  |  |  |

Observation 1: For RSTD accuracy, with 1-sample

- it is feasible to achieve similar accuracy as Rel-16 requirements at -6dB Es/Iot under all propagation conditions assumed in Rel-16

- it is feasible to achieve similar accuracy as Rel-16 requirements at -13dB Es/Iot for some but not all cases under AWGN channel

- it is NOT feasible to achieve similar accuracy as Rel-16 requirements for all cases at -13dB Es/Iot under TDL-A/C or TDL-D channel

Observation 2: For RSTD accuracy, with 2-sample

- it is feasible to achieve similar accuracy as Rel-16 requirements at -6dB Es/Iot under all propagation conditions assumed in Rel-16

- it is feasible to achieve similar accuracy as Rel-16 requirements at -13dB Es/Iot for all cases under AWGN and TDL-D channel

- it is NOT feasible to achieve similar accuracy as Rel-16 requirements for most cases at -13dB Es/Iot under TDL-A/C channel

Simulation results for PRS-RSRP

The simulation results for PRS-RSRP are shown in Table 4-6, for AWGN, TDL-A/C and TDL-D channels. Same as for Rel-16, TDL-A channel is used for FR1 and TDL-C is used for FR2. The results are for absolute and relative accuracy derived from 5% and 95% of PRS-RSRP error.

Table 4: PRS-RSRP performance for AWGN channel

| SCS (kHz) | RB | -13 dB |  |  |  | -6 dB |  |  |  | -3 dB |  |  |  | Rel-16 requirements @ -13dB |  | Rel-16 requirements @ -3dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 |  | 2 |  | 1 |  | 2 |  | 1 |  | 2 |  |  |  |  |  |
|  |  | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel |
| 15 | 24 | 2.6 | 3.5 | 1.9 | 2.4 | 2.1 | 2.7 | 2.0 | 2.4 | 2.1 | 2.7 | 2.0 | 2.3 | 6 | 7.5 | 1 | 1.5 |
|  | 52 | 2.5 | 4.0 | 1.7 | 2.7 | 2.0 | 3.2 | 1.7 | 2.6 | 1.9 | 3.1 | 1.6 | 2.5 | 3.5 | 4.5 |  |  |
|  | 104 | 2.1 | 3.6 | 1.4 | 2.3 | 1.6 | 2.7 | 1.4 | 2.4 | 1.4 | 2.5 | 1.2 | 2.1 | 2 | 3 |  |  |
| 30 | 24 | 2.6 | 3.6 | 1.9 | 2.4 | 2.1 | 2.7 | 1.9 | 2.4 | 2.1 | 2.6 | 1.9 | 2.2 | 6 | 7.5 |  |  |
|  | 48 | 2.5 | 4.1 | 1.7 | 2.8 | 2.0 | 3.2 | 1.7 | 2.7 | 1.9 | 3.2 | 1.6 | 2.6 | 3.5 | 4.5 |  |  |
|  | 132 | 2.1 | 3.3 | 1.4 | 2.3 | 1.5 | 2.6 | 1.3 | 2.3 | 1.3 | 2.3 | 1.2 | 2.0 | 2 | 3 |  |  |
| 60, FR1 | 24 | 2.6 | 3.5 | 1.9 | 2.4 | 2.1 | 2.7 | 1.9 | 2.3 | 2.1 | 2.6 | 2.0 | 2.3 | 6 | 7.5 |  |  |
|  | 64 | 2.3 | 3.8 | 1.6 | 2.6 | 1.9 | 3.1 | 1.6 | 2.6 | 1.7 | 2.9 | 1.5 | 2.3 | 3.5 | 4.5 |  |  |
|  | 132 | 2.0 | 3.4 | 1.4 | 2.3 | 1.5 | 2.6 | 1.4 | 2.3 | 1.4 | 2.3 | 1.2 | 2.0 | 2 | 3 |  |  |
| 60, FR2 | 24 | 2.6 | 3.5 | 1.9 | 2.4 | 2.1 | 2.7 | 1.9 | 2.3 | 2.1 | 2.6 | 2.0 | 2.3 | 4.5 | 6 |  | 1 |
|  | 64 | 2.3 | 3.8 | 1.6 | 2.6 | 1.9 | 3.1 | 1.6 | 2.6 | 1.7 | 2.9 | 1.5 | 2.3 | 2 | 3.5 |  |  |
|  | 132 | 2.0 | 3.4 | 1.4 | 2.3 | 1.5 | 2.6 | 1.4 | 2.3 | 1.4 | 2.3 | 1.2 | 2.0 | 2 | 3.5 |  |  |
| 120 | 32 | 2.7 | 3.7 | 2.0 | 2.4 | 2.2 | 2.8 | 2.0 | 2.4 | 2.3 | 2.7 | 2.1 | 2.4 | 4.5 | 6 |  |  |
|  | 64 | 2.3 | 3.9 | 1.6 | 2.6 | 1.9 | 3.1 | 1.6 | 2.6 | 1.8 | 3.0 | 1.6 | 2.4 | 2 | 3.5 |  |  |
|  | 128 | 2.1 | 3.4 | 1.4 | 2.3 | 1.5 | 2.6 | 1.3 | 2.3 | 1.5 | 2.4 | 1.3 | 2.0 | 2 | 3.5 |  |  |

Table 5: PRS-RSRP performance for TDL-A/C channel

| SCS (kHz) | RB | -13 dB |  |  |  | -6 dB |  |  |  | -3 dB |  |  |  | Rel-16 requirements @ -13dB |  | Rel-16 requirements @ -3dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 |  | 2 |  | 1 |  | 2 |  | 1 |  | 2 |  |  |  |  |  |
|  |  | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel |
| 15 | 24 | 5.4 | 6.4 | 4.3 | 4.7 | 4.9 | 5.6 | 4.4 | 4.8 | 4.7 | 5.3 | 4.2 | 4.5 | 6 | 7.5 | 1 | 1.5 |
|  | 52 | 3.0 | 5.1 | 2.0 | 3.7 | 2.6 | 4.9 | 2.2 | 4.0 | 2.3 | 4.5 | 2.0 | 3.7 | 3.5 | 4.5 |  |  |
|  | 104 | 2.3 | 4.1 | 1.6 | 2.9 | 1.8 | 3.5 | 1.5 | 2.9 | 1.6 | 3.2 | 1.3 | 2.6 | 2 | 3 |  |  |
| 30 | 24 | 5.8 | 6.7 | 4.8 | 5.3 | 4.2 | 4.9 | 4.5 | 5.0 | 4.0 | 4.6 | 4.3 | 4.7 | 6 | 7.5 |  |  |
|  | 48 | 2.9 | 4.9 | 1.9 | 3.3 | 2.5 | 4.3 | 2.0 | 3.6 | 2.2 | 3.9 | 1.9 | 3.3 | 3.5 | 4.5 |  |  |
|  | 132 | 2.2 | 3.7 | 1.5 | 2.6 | 1.6 | 2.9 | 1.4 | 2.5 | 1.4 | 2.7 | 1.2 | 2.3 | 2 | 3 |  |  |
| 60, FR1 | 24 | 5.5 | 6.4 | 4.9 | 5.3 | 3.8 | 4.4 | 4.7 | 5.1 | 3.6 | 4.1 | 4.5 | 4.8 | 6 | 7.5 |  |  |
|  | 64 | 2.8 | 4.6 | 1.9 | 3.1 | 2.3 | 3.7 | 2.1 | 3.3 | 2.1 | 3.4 | 1.9 | 2.9 | 3.5 | 4.5 |  |  |
|  | 132 | 2.2 | 3.8 | 1.5 | 2.5 | 1.7 | 2.9 | 1.6 | 2.6 | 1.5 | 2.5 | 1.4 | 2.4 | 2 | 3 |  |  |
| 60, FR2 | 24 | 4.3 | 5.1 | 3.6 | 3.9 | 3.8 | 4.3 | 3.5 | 3.7 | 3.6 | 4.0 | 3.3 | 3.5 | 4.5 | 6 |  | 1 |
|  | 64 | 2.7 | 4.3 | 2.0 | 3.0 | 2.3 | 3.5 | 2.0 | 2.9 | 2.1 | 3.2 | 1.8 | 2.6 | 2 | 3.5 |  |  |
|  | 132 | 2.3 | 3.7 | 1.6 | 2.5 | 1.7 | 2.8 | 1.6 | 2.4 | 1.6 | 2.5 | 1.4 | 2.1 | 2 | 3.5 |  |  |
| 120 | 32 | 4.4 | 5.1 | 3.2 | 3.4 | 3.7 | 4.1 | 3.3 | 3.6 | 3.4 | 3.8 | 3.1 | 3.3 | 4.5 | 6 |  |  |
|  | 64 | 2.9 | 4.3 | 2.1 | 2.9 | 2.4 | 3.5 | 2.2 | 3.0 | 2.2 | 3.2 | 1.9 | 2.7 | 2 | 3.5 |  |  |
|  | 128 | 2.4 | 3.7 | 1.7 | 2.5 | 1.8 | 2.7 | 1.7 | 2.4 | 1.6 | 2.4 | 1.5 | 2.2 | 2 | 3.5 |  |  |

Table 6: PRS-RSRP performance for TDL-D channel

| SCS (kHz) | RB | -13 dB |  |  |  | -6 dB |  |  |  | -3 dB |  |  |  | Rel-16 requirements @ -13dB |  | Rel-16 requirements @ -3dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 |  | 2 |  | 1 |  | 2 |  | 1 |  | 2 |  |  |  |  |  |
|  |  | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel | Abs | Rel |
| 15 | 24 | 3.0 | 4.0 | 2.1 | 2.6 | 2.3 | 3.0 | 2.2 | 2.6 | 2.1 | 2.7 | 2.0 | 2.3 | 6 | 7.5 | 1 | 1.5 |
|  | 52 | 2.6 | 4.1 | 1.7 | 2.8 | 2.1 | 3.5 | 1.8 | 2.8 | 1.9 | 3.1 | 1.6 | 2.5 | 3.5 | 4.5 |  |  |
|  | 104 | 2.1 | 3.6 | 1.4 | 2.5 | 1.7 | 2.8 | 1.4 | 2.4 | 1.4 | 2.5 | 1.2 | 2.1 | 2 | 3 |  |  |
| 30 | 24 | 2.9 | 3.8 | 2.2 | 2.6 | 2.3 | 2.9 | 2.1 | 2.5 | 2.1 | 2.6 | 1.9 | 2.2 | 6 | 7.5 |  |  |
|  | 48 | 2.6 | 4.2 | 1.8 | 2.9 | 2.1 | 3.5 | 1.8 | 2.9 | 1.9 | 3.2 | 1.6 | 2.6 | 3.5 | 4.5 |  |  |
|  | 132 | 2.1 | 3.5 | 1.5 | 2.3 | 1.5 | 2.6 | 1.4 | 2.3 | 1.3 | 2.3 | 1.2 | 2.0 | 2 | 3 |  |  |
| 60, FR1 | 24 | 4.3 | 5.1 | 4.9 | 5.3 | 3.8 | 4.3 | 4.7 | 5.1 | 3.6 | 4.0 | 4.5 | 4.8 | 6 | 7.5 |  |  |
|  | 64 | 2.5 | 3.2 | 2.2 | 3.5 | 2.0 | 2.3 | 2.3 | 3.5 | 1.8 | 2.0 | 2.1 | 3.2 | 3.5 | 4.5 |  |  |
|  | 132 | 1.9 | 3.1 | 1.5 | 2.4 | 1.5 | 2.3 | 1.6 | 2.4 | 1.3 | 2.0 | 1.4 | 2.1 | 2 | 3 |  |  |
| 60, FR2 | 24 | 3.0 | 3.9 | 2.2 | 2.6 | 2.3 | 2.9 | 2.2 | 2.6 | 2.1 | 2.6 | 2.0 | 2.3 | 4.5 | 6 |  | 1 |
|  | 64 | 2.4 | 4.0 | 1.7 | 2.7 | 1.9 | 3.2 | 1.7 | 2.6 | 1.7 | 2.9 | 1.5 | 2.3 | 2 | 3.5 |  |  |
|  | 132 | 2.1 | 3.5 | 1.4 | 2.3 | 1.6 | 2.6 | 1.4 | 2.3 | 1.4 | 2.3 | 1.2 | 2.0 | 2 | 3.5 |  |  |
| 120 | 32 | 3.0 | 3.9 | 2.3 | 2.6 | 2.5 | 3.0 | 2.3 | 2.7 | 2.3 | 2.7 | 2.1 | 2.4 | 4.5 | 6 |  |  |
|  | 64 | 2.6 | 4.1 | 1.8 | 2.8 | 2.0 | 3.2 | 1.8 | 2.7 | 1.8 | 3.0 | 1.6 | 2.4 | 2 | 3.5 |  |  |
|  | 128 | 2.2 | 3.5 | 1.5 | 2.3 | 1.7 | 2.7 | 1.5 | 2.3 | 1.5 | 2.4 | 1.3 | 2.0 | 2 | 3.5 |  |  |

Observation 3: For PRS-RSRP accuracy,

- It is NOT feasible to achieve Rel-16 accuracy for -3dB Es/Iot with 1 or 2 samples even at same -3dB Es/Iot.

- It is feasible to achieve Rel-16 accuracy for -13dB Es/Iot with 1 sample for some but not all cases at -13dB Es/Iot. It is feasible to achieve Rel-16 accuracy for -13dB Es/Iot with 2 samples for all cases at -13dB Es/Iot.

- It is feasible to achieve Rel-16 accuracy for -13dB Es/Iot with 1 sample for all cases at -6dB Es/Iot.

Simulation results for UE Rx-Tx

The simulation results for RSTD are shown in Table 7-9, for AWGN, TDL-A/C and TDL-D channels. Same as for Rel-16, TDL-A channel is used for FR1 and TDL-C is used for FR2. The results are for the 90% of Rx-Tx error.

Note: the results for 120kHz are currently missing, and we will add them when results are ready.

Table 7: Rx-Tx performance for AWGN channel

| SCS (kHz) | RB | -13 dB |  | -6 dB |  | -3 dB |  | Rel-16 requirements @ -13dB | Rel-16 requirements @ -3dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 1 | 2 | 1 | 2 |  |  |
| 15 | 24 | 89 | 67 | 79 | 43 | 55 | 55 | 101 | 78 |
|  | 52 | 80 | 44 | 50 | 27 | 42 | 38 | 75 | 59 |
|  | 104 | 97 | 21 | 22 | 12 | 20 | 20 | 37 | 30 |
| 30 | 24 | 60 | 37 | 39 | 24 | 31 | 31 | 58 | 57 |
|  | 48 | 74 | 26 | 30 | 17 | 22 | 21 | 39 | 30 |
|  | 132 | 66 | 9 | 9 | 5 | 8 | 8 | 16 | 15 |
| 60, FR1 | 24 | 81 | 22 | 26 | 16 | 22 | 20 | 36 | 29 |
|  | 64 | 63 | 9 | 10 | 5 | 9 | 9 | 16 | 15 |
|  | 132 | 67 | 5 | 5 | 3 | 4 | 4 | 8 | 7 |
| 60, FR2 | 24 | 50 | 22 | 26 | 16 | 22 | 20 | 35 | 22 |
|  | 64 | 91 | 9 | 9 | 5 | 8 | 8 | 15 | 15 |
|  | 132 | 89 | 4 | 4 | 2 | 4 | 4 | 7 | 7 |
| 120 | 32 | 97 | 8 | 9 | 5 | 8 | 8 | 14 | 12 |
|  | 64 | 80 | 5 | 5 | 3 | 5 | 5 | 9 | 7 |
|  | 128 | 60 | 2 | 2 | 1 | 2 | 2 | 4 | 4 |

Table 8: Rx-Tx performance for TDL-A/C channel

| SCS (kHz) | RB | -13 dB |  | -6 dB |  | -3 dB |  | Rel-16 requirements @ -13dB | Rel-16 requirements @ -3dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 1 | 2 | 1 | 2 |  |  |
| 15 | 24 | 2170 | 1235 | 146 | 129 | 146 | 146 | 180 | 137 |
|  | 52 | 2632 | 2704 | 101 | 97 | 101 | 101 | 98 | 96 |
|  | 104 | 2636 | 1486 | 83 | 82 | 87 | 82 | 68 | 62 |
| 30 | 24 | 1730 | 2531 | 92 | 92 | 92 | 94 | 87 | 87 |
|  | 48 | 2877 | 2652 | 68 | 68 | 66 | 71 | 85 | 68 |
|  | 132 | 2994 | 1884 | 39 | 38 | 39 | 40 | 44 | 44 |
| 60, FR1 | 24 | 1105 | 1804 | 105 | 82 | 89 | 99 | 139 | 59 |
|  | 64 | 1061 | 1486 | 40 | 37 | 40 | 40 | 46 | 42 |
|  | 132 | 1784 | 2553 | 37 | 36 | 35 | 36 | 30 | 36 |
| 60, FR2 | 24 | 2486 | 2802 | 102 | 90 | 102 | 86 | 92 | 75 |
|  | 64 | 1134 | 80 | 87 | 80 | 82 | 68 | 70 | 72 |
|  | 132 | 1793 | 85 | 84 | 85 | 84 | 85 | 57 | 57 |
| 120 | 32 | 1633 | 1585 | 89 | 87 | 88 | 77 | 60 | 61 |
|  | 64 | 2292 | 80 | 81 | 80 | 80 | 78 | 66 | 64 |
|  | 128 | 1883 | 77 | 78 | 77 | 77 | 77 | 62 | 55 |

Table 9: Rx-Tx performance for TDL-D channel

| SCS (kHz) | RB | -13 dB |  | -6 dB |  | -3 dB |  | Rel-16 requirements @ -13dB | Rel-16 requirements @ -3dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 1 | 2 | 1 | 2 |  |  |
| 15 | 24 | 1937 | 109 | 113 | 72 | 105 | 101 | 180 | 137 |
|  | 52 | 2101 | 62 | 68 | 43 | 56 | 58 | 98 | 96 |
|  | 104 | 2262 | 42 | 42 | 28 | 42 | 42 | 68 | 62 |
| 30 | 24 | 1763 | 57 | 59 | 40 | 55 | 55 | 87 | 87 |
|  | 48 | 2923 | 53 | 55 | 35 | 53 | 50 | 85 | 68 |
|  | 132 | 2854 | 26 | 26 | 16 | 26 | 26 | 44 | 44 |
| 60, FR1 | 24 | 1760 | 77 | 81 | 46 | 77 | 75 | 139 | 59 |
|  | 64 | 1783 | 27 | 28 | 17 | 27 | 27 | 46 | 42 |
|  | 132 | 1552 | 19 | 19 | 13 | 18 | 18 | 30 | 36 |
| 60, FR2 | 24 | 2229 | 53 | 57 | 35 | 53 | 51 | 92 | 75 |
|  | 64 | 2963 | 39 | 40 | 23 | 39 | 39 | 70 | 72 |
|  | 132 | 2228 | 32 | 32 | 19 | 32 | 32 | 57 | 57 |
| 120 | 32 | 2871 | 35 | 35 | 22 | 34 | 34 | 60 | 61 |
|  | 64 | 1325 | 36 | 37 | 21 | 36 | 36 | 66 | 64 |
|  | 128 | 1678 | 34 | 34 | 20 | 34 | 34 | 62 | 55 |
| Note: The Rel-16 requirements are for fading channel. |  |  |  |  |  |  |  |  |  |

Observation 4: The performance trend for UE Rx-Tx is similar to RSTD for -13dB and -6dB Es/Iot.

Observation 5: For -3dB Es/Iot,

- it is feasible to achieve Rel-16 accuracy for -3dB and AWGN channel with 1-sample under AWGN channel,

- it is feasible to achieve Rel-16 accuracy for -3dB and fading channel with 1-sample under TDL-D channel

- it is feasible to achieve Rel-16 accuracy for -3dB and fading channel for some but not all cases with 1-sample under TDL-A/C channel

Conclusions

In this paper we provided our views on reduced number of samples for positioning measurement.

Observation 1: For RSTD accuracy, with 1-sample

- it is feasible to achieve similar accuracy as Rel-16 requirements at -6dB Es/Iot under all propagation conditions assumed in Rel-16

- it is feasible to achieve similar accuracy as Rel-16 requirements at -13dB Es/Iot for some but not all cases under AWGN channel

- it is NOT feasible to achieve similar accuracy as Rel-16 requirements for all cases at -13dB Es/Iot under TDL-A/C or TDL-D channel

Observation 2: For RSTD accuracy, with 2-sample

- it is feasible to achieve similar accuracy as Rel-16 requirements at -6dB Es/Iot under all propagation conditions assumed in Rel-16

- it is feasible to achieve similar accuracy as Rel-16 requirements at -13dB Es/Iot for all cases under AWGN and TDL-D channel

- it is NOT feasible to achieve similar accuracy as Rel-16 requirements for most cases at -13dB Es/Iot under TDL-A/C channel

Observation 3: For PRS-RSRP accuracy,

- It is NOT feasible to achieve Rel-16 accuracy for -3dB Es/Iot with 1 or 2 samples even at same -3dB Es/Iot.

- It is feasible to achieve Rel-16 accuracy for -13dB Es/Iot with 1 sample for some but not all cases at -13dB Es/Iot. It is feasible to achieve Rel-16 accuracy for -13dB Es/Iot with 2 samples for all cases at -13dB Es/Iot.

- It is feasible to achieve Rel-16 accuracy for -13dB Es/Iot with 1 sample for all cases at -6dB Es/Iot.

Observation 4: The performance trend for UE Rx-Tx is similar to RSTD.

Observation 5: For -3dB Es/Iot,

- it is feasible to achieve Rel-16 accuracy for -3dB and AWGN channel with 1-sample under AWGN channel,

- it is feasible to achieve Rel-16 accuracy for -3dB and fading channel with 1-sample under TDL-D channel

- it is feasible to achieve Rel-16 accuracy for -3dB and fading channel for some but not all cases with 1-sample under TDL-A/C channel

Reference

- R4-2120419, WF on NR Positioning Enhancements (Part 1), Ericsson

- R4-2120330, Link level simulation assumptions for NR PRS measurements with reduced latency, Huawei, HiSilicon