3GPP TSG-RAN4 Meeting #101-bis-e	R4-2201641

# Electronic Meeting, 17 – 25 January, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on positioning measurements in RRC Inactive state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN4 has agreed to introduce requirements for PRS measurement in RRC Inactive state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define requirements for PRS measurement in RRC Inactive state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance PRS measurement in RRC Inactive state cannot be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

5.5	Measurements for positioning

5.5.1	Introduction

This clause contains requirements for UE capable of performing NR positioning measurements defined in TS 38.215 [4], including RSTD, PRS-RSRP, UE Rx-Tx time difference, in RRC Inactive state.

Editor’s note: applicability conditions TBA

5.5.2	RSTD measurements

5.5.2.1	Introduction

The requirements in clause 5.5.2 shall apply provided the UE has received NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] requesting the UE to measure and report DL RSTD measurements defined in TS 38.215 [4], and UE is capable of such measurements in RRC Inactive state.

5.5.2.2	Requirements Applicability

The requirements in clause 5.5.2 apply for periodic and triggered RSTD measurements, provided:

-	PRS-RSTD related side conditions given in clause 10.1.23 for FR1 and FR2 are fulfilled, for a corresponding Band.

5.5.2.3	Measurement Capability

UE PRS RSTD measurement capability is as indicated by the UE in NR-DL-TDOA-ProvideCapabilities, according to TS 37.355[34].

Editor’s note: measurement capability may be revisted depending on RAN1/2 agreements.

5.5.2.4	Measurement Reporting Requirements

Editor’s note: TBA

5.5.2.5	Measurements Period Requirements

Editor’s note: TBA

5.5.3	PRS-RSRP measurements

5.5.3.1	Introduction

The requirements in clause 5.5.3.5 shall apply provided the UE has received a message from LMF via LPP [34] requesting the UE to measure and report PRS-RSRP measurements defined in TS 38.215 [4], and UE is capable of such measurements in RRC Inactive state.

5.5.3.2	Requirements applicability

The requirements in clause 5.5.3 apply for periodic and triggered PRS-RSRP measurements, provided:

-	PRS-RSRP related side conditions given in clause 10.1.24 are met for a corresponding Band.

5.5.3.3	Measurement Capability

UE PRS-RSRP measurement capability is as indicated by the UE in NR-DL-AoD-ProvideCapabilities according to TS 37.355 [34].

Editor’s note: measurement capability may be revisted depending on RAN1/2 agreements.

5.5.3.4	Measurement Reporting Requirements

Editor’s note: TBA

5.5.3.5	Measurement Period Requirements

Editor’s note: TBA

5.5.4	UE Rx-Tx time difference measurements

5.5.4.1 Introduction

The requirements in this clause shall apply, provided the UE has received nr-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] requesting the UE to measure and report one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4], and UE is capable of such measurements in RRC Inactive state.

5.5.4.2 Requirements Applicability

The requirements in clause 5.5.4 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25 are met for a corresponding band.

-	SRS is configured on at least one of the PCell, PSCell and SCell.

5.5.4.3 Measurement Capability

UE Rx-Tx time difference measurement capability is as indicated by the UE in NR-Multi-RTT-ProvideCapabilities, according to TS 37.355 [34].

Editor’s note: measurement capability may be revisted depending on RAN1/2 agreements.

5.5.4.4 Measurement Reporting Requirements

Editor’s note: TBA

5.5.4.5	Measurement Period Requirements

Editor’s note: TBA

<End of Change 1>