3GPP TSG RAN WG4 Meeting #101-bis-e	R4-2201672

Electronic Meeting, 17th January – 25th January 2022

Agenda Item:	6.21.2.6

Source: 	Ericsson

Title:	LS response on SRS on UL SRS-RSRPP definition

Document for:	Approval

# Introduction

In RAN1#107-e, RAN1 sent LS to RAN4 [1] on UL SRS-RSRPP definition and requested feedback on working assumption for reference point. In this paper, we provide our view on the reference point for UL SRS-RSRPP measurement.

# Discussion

SSS transmit power, UL RToA, gNB Rx-Tx time difference, UL AoA and UL SRS RSRP as defined as NG-RAN measurement capabilities [2]. Among these, UL RToA, gNB Rx-Tx time difference, UL AoA and UL SRS-RSRP measurements have an impact on achievable positioning accuracy. Moreover, for precise UL-RToA and gNB Rx-Tx (Rx part) time difference measurements the measurements must be done on first path via which SRS is received in the UL. To aid this purpose, reference point selection for UL-RToA and gNB Rx-Tx time difference measurements are based on gNB types. Since UL SRS-RSRPP is also a path-based measurement, therefore in our point of view the reference point for UL SRS-RSRPP measurement, similar to UL-RToA and gNB Rx-Tx measurements, must be based on gNB types.

Adopt reference point for UL-RToA and gNB Rx-Tx measurement also for UL SRS-RSRPP measurement.

The reference point for UL SRS-RSRPP shall be: 
- for type 1-C base station TS 38.104 [3]: the Rx antenna connector, 
- for type 1-O or 2-O base station TS 38.104 [3]: the Rx antenna (i.e., the centre location of the radiating region of the Rx antenna), 
- for type 1-H base station TS 38.104 [3]: the Rx Transceiver Array Boundary connector.

RAN4 to send LS to RAN1 regarding the RAN4 observations on UL SRS-RSRPP measurement reference point.

# Conclusion

- In this contribution we presented our view on UL SRS-RSRPP measurement reference point and made the following proposals:

- Adopt reference point for UL-RToA and gNB Rx-Tx measurement also for UL SRS-RSRPP measurement.

Proposal: The reference point for UL SRS-RSRPP shall be: 
- for type 1-C base station TS 38.104 [3]: the Rx antenna connector, 
- for type 1-O or 2-O base station TS 38.104 [3]: the Rx antenna (i.e., the centre location of the radiating region of the Rx antenna), 
- for type 1-H base station TS 38.104 [3]: the Rx Transceiver Array Boundary connector.

RAN4 to send LS to RAN1 regarding the RAN4 observations on UL SRS-RSRPP measurement reference point.

# References

- R1-2112744, “LS on UL SRS-RSRPP definition”.

- 3GPP TS 38.215: “Physical layer measurements”.

- 3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

# Appendix

3GPP TSG-RAN WG4 Meeting #101-bis-e		R4-xxxxxxx

Electronic Meeting, 17th January – 25th January  2022

Title:	Response to LS on UL SRS-RSRPP definition

Response to:	R1-2112744 “LS on UL SRS-RSRPP definition”

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	RAN WG4

To:	RAN WG1

Contact Person:

Name:	Deep Shrestha

E-mail Address:	deep.shrestha@ericsson.com

Attachments:

1. Overall Description:

RAN4 thanks RAN1 for the LS. RAN4 has discussed the working assumption for reference point for UL SRS-RSRPP measurement. Based on the discussion RAN4 would like to provide following feedback.

RAN4 feedback to RAN1:

Since UL SRS-RSRPP is a path-based measurement, therefore from RAN4 point of view working assumption for reference point for UL SRS-RSRPP measurement shall be similar to other path-based measurements. This will also ensure effective and efficient collection of path-based measurements as the reference point for measurements is same.

Working assumption: The reference point for UL SRS-RSRPP shall be:

- for type 1-C base station TS 38.104 [1]: the Rx antenna connector,

- for type 1-O or 2-O base station TS 38.104 [1]: the Rx antenna (i.e., the centre location of the radiating region of the Rx antenna),

- for type 1-H base station TS 38.104 [1]: the Rx Transceiver Array Boundary connector.

2. Actions:

To RAN WG1 group.

ACTION: 	RAN4 kindly requests RAN1 to take the above feedback into consideration when formulating the working assumption for UL SRS-RSRPP measurement reference point.

3. References:

- 3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

4. Date of Next TSG-RAN WG4 Meetings:

| TSG RAN WG4 Meeting #102-e | 21 Feb – 03 Feb, 2022 | E-Meeting |
| --- | --- | --- |
| TSG RAN WG4 Meeting #103-e | 16 May – 27 May, 2022 | E-Meeting |