3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201680

Electronic Meeting, 17th – 25th January 2022

Source:	AT&T

Title:	TP for TR 38.841 Addition of CA_n29-n77

Agenda item:			5.30.1

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.841 to add CA_n29-n77.

# 2. Text Proposal

---Start of changes---

## 5.X	CA_n29-n77

### 5.X.1	Configurations

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n29A-n77A | n778 | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n29A-n77(2A) | n778 | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.X.2	Maximum output power

The combination CA_n29-n77 does not have an uplink CA configuration since n29 is an SDL operating band. The power class would be defined by the single uplink carrier requirements with appropriate ∆TIB applied. Concerning the co-existence analysis, CA_n29-n77 is similar to Case b as shown below

Table 5.X.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration or single uplink carrier | Power class 2 cases for CA_n29-n77 | CA power class | Carrier n29 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| n77 | Case b | 26dBm | N/A | 26dBm |

The tolerance for single uplink carrier is applied.

### 5.X.3	REFSENS requirements

According to PC3 DC_29A-30A_n77A in TR 37.717-21-11, 5th order harmonic mixing is produced from band n77 UL that might fall in Rx of band 29.

Therefore, additional MSD values for 5th order harmonic mixing need to be defined for PC2 CA_n29-n77.

#### 5.X.3.1	Power class 2 Case A

Power Class 2 Case A is not applicable for CA_n29-n77.

#### 5.X.3.2	Power class 2 Case B

The additional MSD due to receiver harmonic mixing for Case B are defined in Table 5.X.3.2-1. For harmonic mixing, the n77 power has increased by 3dB which results in 3dB more power down-converted at the 5th RX LO harmonic.

Table 5.X.3.2-1: Reference sensitivity exceptions due to harmonic mixing for PC2 CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n772 | n291 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |

The additional MSD due to intermodulation for PC2 Case B CA_n29-n77 is not applicable.

### 5.X.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n29A-n77A requirements are applied for PC2 CA_n29A-n77A.

---End of changes---

# References