3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201683

# Electronic meeting, January 17th-25th, 2022

Source:		UIC

Title:			Proposed changes for clause 7.1 in 3GPP TR 38.852

Agenda item:		5.5.4

Document for:	Approval

# Introduction

Clause 7.1 establishes the necessary context for BS RF in the RMR 1900 context. The proposed text updates will improve readability and context.

# Proposed changes

***********************************************1st Change*******************************************************

## 7.1	BS specific requirements

### 7.1.1	General

According to ECC Decision (20)02 [1], the BS RF work for RMR1900 is limited to the Wide Area Base Stations, determined by the use along railway lines and their corresponding characteristics defined by 3GPP TS 38.104 [4].

In accordance to ECC Decision 20(02) [1], the related studies were done for the non-AAS BS architectures, defined in 3GPP TS 38.104 [4]. Therefore the RAN4 requirements derivation was limited to the BS type 1-C requirements, only.

As the EIRP limits were defined in the ECC Decision (20)02 [1], consideration of non-AAS BS architecture required to convert those EIRP limits into the conducted requirements assuming a fixed antenna gain value of 18 dBi, encompassing internal losses (feeder, etc.) of 0 dB.

***********************************************2nd Change******************************************************

#### 7.1.2.1	BS maximum output power

Based on ECC Decision (20)02 [1], the BS maximum output power for BS operating in band n101 applicable for [uncoordinated deployment], shall not exceed the Prated,c,AC of 47 dBm/10MHz.

***********************************************3rd Change******************************************************

##### 7.1.2.2.2	Tx spurious emissions

Additional Tx spurious emission requirements are considered for band n101 in 3GPP TS 38.104 [4], based on conversion of the EIRP limits from ECC Decision (20)02 [1], as summarized in table 7.1.2.2.2-1:

- Table 7.1.2.2.2-1: Additional Tx spurious emissions requirement derivation for n101

| - Spurious frequency range | - Basic limit | - Measurement bandwidth |
| --- | --- | --- |
| 1920 MHz – 1980 MHz | -61 dBm (NOTE) | 5 MHz |
| NOTE: Assuming a 18 dBi antenna gain. |  |  |

***********************************************4th Change*******************************************************

### 7.1.3	Receiver characteristics

RAN4 will not specify requirements applicable for BS enhanced selectivity considering adjacent band n1.

Editor´s Note: To be developed!

#### 7.1.3.2	Rx blocking

Rx blocking requirements applicable for band n101 in 3GPP TS 38.104 [4], based on the conversion of EIRP limits from ECC Decision (20)02 [1], are summarized in table 7.1.3.2-1.

Editor´s Note: Interfering signal characteristic is FFS.

- Table 7.1.3.2-1: Rx blocking requirement derivation for n101

| - BS channel bandwidth of the lowest/highest carrier received (MHz) | - Wanted signal mean power (dBm) | - Interfering signal mean power (dBm) | - Center Frequency of Interfering Signal (MHz) | - Type of interfering signal |
| --- | --- | --- | --- | --- |
| [5], 10 | PREFSENS + 3 dB | Wide Area BS: -20 | 1807.5 MHz to 1877.5 MHz | 5 MHz NR or LTE signal |

********************************************End of Changes****************************************************